3GPP TSG-SA3 Meeting #122	S3-251915

- Fukuoka, Japan, 19 – 23 May 2025

Source:	NTT DOCOMO

Title:	Pseudo-CR on update of KI#3 in 33.713

Document for:	Approval

Agenda item:	5.9

Spec:	3GPP TR 33.713

Version:	0.8.0

Work Item:	FS_AIOT_sec

Comments

So far, attack on AIoT device identifiers privacy has only considered the identity transmitted in the inventory response. Active attacks may be able to read out the identifier based on device response to group paging. Therefore KI#3 is updated to reflect this.

This pCR has been updated from S3-251639 by removing the requirement.

* * * First Change * * * *

## 5.3	Key issue #3: Privacy by protecting AIoT device identifiers

### 5.3.1	Key issue details

5G Ambient IoT service is a type of cellular IoT communication system where Ambient IoT devices utilize harvested energy to generate RF signals for bi-directional information transmission. Ambient IoT devices are characterized by limited functions, requiring only small and infrequent data transfers.

TS 22.369 [2] clause 5.2.6 defines the following privacy-related requirements:

“The 5G system shall be able to provide a mechanism to protect the privacy of information (e.g., location and identity) exchanged during communication between an Ambient IoT device and the 5G network or an Ambient IoT capable UE.”

In AIoT services, identifiers of AIoT device are used to identify the device. If the identifiers associated with a device are not privacy protected (e.g., exposed over the air), an attacker (e.g., an over-the-air attacker) can identify and track an AIoT device based on the identifiers associated with the AIoT device. Thus, this key issue is to investigate potential mechanisms to privacy protect the AIoT device identifiers.

### 5.3.2	Security Threats

An attacker can identify, monitor and track an AIoT device based on the identifiers associated with the AIoT device if the identifiers are not privacy protected.

If group ID is realized as a bitmask, it is possible for an attacker to read out the permanent ID of a device by performing a binary search on the permanent ID with multiple paging requests with an increasing number of bits in the bitmask. On each inventory request, the attacker can read out one additional bit of the permanent ID.

### 5.3.3	Potential security requirements

Mechanisms for mitigating privacy threats (described above) by identifying, linking, and tracking the identifiers of AIoT Device(s) shall be supported.

Editor’s Note: AIoT use cases that do not need the above privacy protection mechanisms are FFS.

* * * End of Changes * * * *