3GPP TSG-SA3 Meeting #122	S3-251930

# Fukuoka, Japan, 19 – 23 May 2025

Source:	OPPO

Title:	Security mechanism and procedures for inter-CU LTM in non-DC case

Document for:	Approval

Agenda item:	4.12

Spec:	3GPP TS 33.501

Version:	19.2.0

Work Item:	NR_Mob_Ph4_Sec

Comments

It is proposed to add security mechanism and procedures for inter-CU LTM in non-DC scenario. Since RAN3 has reached the following agreements for security handling:

-	The new source gNB needs to provide the new KgNB*(s) to the corresponding candidate gNB via a LTM Configuration Update procedure.

-	The new NCC value is sent from gNB-CU to the gNB-DU via the UE Context Modification procedure.

Proposed Changes

* * * Start of Change * * * *

#### 6.X.Y	Security mechanism and procedures for inter-CU LTM

Editor’s Notes: This clause contains the security procedure for inter-CU LTM in 5G.

##### 6.X.Y.1	Non-DC case

This clause describes the security mechanism and procedure for inter-CU LTM in non-DC case.

During the LTM preparation phase, the source gNB shall compute KNG-RAN* from target PCI, its frequency ARFCN-DL/EARFCN-DL, and either from currently active KgNB in case of horizontal key derivation or from the NH in case that it has an unused {NH, NCC} pair and vertical key derivation as described in Annex A.11. Then the source gNB shall forward the {KNG-RAN*, NCC} pair to the corresponding candidate gNB(s) via the LTM Configuration Update procedure. The source gNB shall send the UE's 5G security capabilities and ciphering and integrity algorithms used in the source cell to the target gNB. The target gNB shall select the algorithm with highest priority from the received 5G security capabilities of the UE according to the prioritized locally configured list of algorithms.

During the LTM cell switch execution phase, the source gNB shall include the NCC value in the Cell Switch Command MAC CE to the UE. If the target gNB selects different algorithms compared to the source gNB, the chosen algorithms of the target gNB shall be include in the Cell Switch Command MAC CE.

For the UE’s behaviors, if the NCC value the UE received in the Cell Switch Command MAC CE is equal to the NCC value associated with the currently active KgNB, the UE shall derive the KNG-RAN* from the currently active KgNB as described in Annex A.11. If the UE received an NCC value that was larger than the NCC associated with the currently active KgNB, the UE shall first synchronize the locally kept NH parameter by computing the function defined in Annex A.10 iteratively and increasing the NCC value until it matches the NCC value received. When the NCC values match, the UE shall compute the KNG-RAN* from the synchronized NH parameter as described in Annex A.11.

##### 6.X.Y.2	DC case

In the case where CU is acting as SN and MN is unchanged, the security mechanism and procedures for SCPAC as specified in clause 6.10.2.4 apply.

* * * End of Change * * * *