3GPP TSG-SA3 Meeting #122	S3-252070

Fukuoka, Japan, 19 – 23 May 2025

Title:	[draft] Reply LS on security handling for inter-CU LTM in non-DC cases

Response to:	LS S3-251824/R3-252449 on security handling for inter-CU LTM in non-DC cases from RAN3

Release:	Release 19

Work Item:	NR_Mob_Ph4-Core

Source:	Xiaomi [to be SA3]

To:	RAN3

Cc:	RAN2

Contact person:	Wei Lu

luwei10(at)xiaomi(dot)com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1	Overall description

SA3 would like to thank RAN3 for their reply LS on security handling for inter-CU LTM in non-DC cases.

For the completion of SA3 normative work on security handling for inter-CU LTM in non-DC cases, SA3 would like to ask the following questions for clarification, regarding the RAN3 agreements in R3-252449 and R3-252520:

- According to TS 33.501 clause 6.9.2.3.2, the source gNB shall forward the {KNG-RAN*, NCC} pair to the target gNB in Xn handovers. When the new source gNB provides new KgNB*(s) to the corresponding candidate gNB via LTM Configuration Update message as agreed in R3-252520, does the source gNB also provide the associated NCC to the candidate gNB via the same LTM Configuration Update message? This concerns how the candidate gNB, when becoming the next serving gNB, should determine the NCC value and corresponding KNG-RAN* to be used for the subsequent inter-CU LTM execution.

- When the new source gNB provides new KgNB*(s) to the corresponding candidate gNB via a LTM Configuration Update procedure, does the source gNB provide

a)	one KgNB* for each of the cells covered by one candidate gNB, i.e. one KgNB* per candidate cell? Or

b)	one KgNB* for each candidate gNB, i.e. one KgNB* per candidate gNB?

If a), does the source gNB have to initiate a LTM Configuration Update procedure per cell or per candidate gNB? This concerns how the {KNG-RAN*, NCC} pair(s) should be included in the LTM Configuration Update message.

- Each candidate gNB needs to select the AS security algorithms used with the UE and determine UE’s UP security activation status based on the UE AS security context received via LTM Configuration Update procedure. Does the candidate gNB need to return the selected AS security algorithms and determined UE’s UP security activation status to the source gNB via the same LTM Configuration Update procedure? This concerns how the UE security context is synchronized between the UE and each of the candidate gNBs.

2	Actions

To RAN3

ACTION: 	SA3 respectfully asks RAN3 to provide feedback to the above questions for clarification.

3	Dates of next TSG SA WG 3 meetings

SA3#123	25 – 29 August 2025		Goteborg, Sweden

SA3#124	13 – 17 October 2025		China