3GPP TSG-SA3 Meeting #122	S3-252233

Fukuoka, Japan, 17-21 May 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.122 | CR | DRAFT | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resource Owner authorization revocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3_sec |  |  |  |  |  | Date: |  |  | 2025-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The authorization information is clarified in TS 23.222 Clause Annex E on Configuration data for CAPIF as follows. > Authorization information related to the list of supported applications >> Application identifier >> Purpose >> Scope One resource owner’s resource may be used/exposed for more than one application as independent aspects, so a revocation of service exposure to one application should not impact the other services, where revocation should be correctly done specific to the applications as preferred by the resource owner e.g., a UE’s location is being currently consumed by two apps 1 and 2, but at a certain time the UE determines that its’ location info is not allowed to be exposed for app 2. Therefore, the draft CR clarifies the revocation aspects. Further ENs are resolved with clarification texts with reference to TS 23.222 as applicable. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Normative text provided to clarify resource owner authorization revocation information and corresponding procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Mis-implementation and cause incompatibility. A generic revocation of service exposure will leads to service failure for the resource owner with all ongoing active application servcies. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 6.5.3.4	Revocation

The CCF can initiate the Authorization Revocation Request message as defined in clause 8.23.4 of TS 23.222 [3] with additional information to identify the RNAA-related revoked token.

NOTE:  The CCF can receive a revocation request message from the resource owner via the UE, resource owner function, web page etc. All these mechanisms are out of the scope of the present document.

AEF, storing the information about the RNAA-related revoked token, shall check whether the token presented by an API invoker is revoked or not, before responding to the API invoker’s invocation request.

The CCF provided notification message to the API invoker shall include the information to identify the RNAA-related revoked token.

The procedure illustrated in Figure 6.5.3.4-1 and explained below can be used for revoking RNAA-related token in RNAA scenarios.

Pre-conditions:

1.	The API invoker is authenticated and authorized to use the service API.

API invoker

Resource owner function

CAPIF Core function

AEF

0. TLS session establishment.

1. Resource owner authorization revocation request

2.  Identify the RNAA-related revoked token.

3. Revoke API invoker authorization request

4. Invalidate the authorization of API invoker for service API and resource

5-7: Same steps as the steps 4-6 in clause 8.23.4 of  TS 23.222

8. Resource owner authorization revocation response

Figure 6.5.3.4-1: Procedure for revoking resource owner authorization

- CCF and ROF shall establish TLS session over CAPIF-8 reference point as specified in clause 6.11.

- Triggered by the resource owner, the resource owner function sends resource owner authorization revocation request to the CCF. The resource owner authorization revocation information in request message includes the API invoker information (where the API invoker may be either an application on a server or an application on a UE as specified in TS 23.222 [3]), information related to service API. The request can include Resource Owner ID (e.g., GPSI), service operation information and application identifier.

NOTE: According to TS 23.222 Annex E [3], the authorization information related to the list of supported applications are mainted for CAPIF which includes Application identifier, purpose and scope. One resource owner’s resource may be used/exposed for more than one application as independent aspects, so a revocation of service exposure to one application should not impact the other services, where revocation should be correctly done specific to the applications as preferred by the resource owner e.g., a UE’s location is being currently consumed by two apps 1 and 2, but at a certain time the UE determines that its’ location info is not allowed to be exposed for app 2 then the CCF should be able to correctly implement the related revocation as required by the Resource owner.

Editor’s Note: Whether purpose of data processing is needed is FFS.

- The CCF determines the details of the resource owner ID, API invoker ID and the service API in the scope of the token based on the received resource owner authorization revocation information to identify the RNAA-related token to be revoked.

- The CCF sends the Revoke resource authorization notify message to the AEF as defined in clause 8.35 of TS 23.222 [3] with additional information to identify the RNAA-related revoked token.

Editor’s Note: clarification on additional information used for AEF to identify the RNAA-related token is ffs.

- The AEF, storing the resource owner authorization revocation information about the RNAA-related revoked token, checks whether the token presented by an API invoker is revoked or not, before responding to the API invoker’s invocation request.

NOTE: The AEF does not allow any service for an API invoker when the access token is not valid (e.g., token is already revoked or expired etc.), so the related resource is not further impacted due to any invalid token based requests.

5-7. Step 5-7 are the same as the steps 4-6 in clause 8.23.4 of TS 23.222 [3].

8.    The CCF sends a resource owner authorization revocation response as an acknowledgement to the ROF that the resource owner authorization has been revoked.

Alternatively, there is an AEF initiated revocation procedure i.e., according to TS 23.222 Clause 8.23, an AEF sends Revoke API Invoker authorization request to the CCF, in such as case the CCF after authorization revocation sends Revoke API invoker authorization notify to the API Invoker.

* * * End of Changes * * * *