3GPP TSG-SA3 Meeting #122	S3-252248

Fukuoka, Japan, 19-23 May 2025

Source:	Nokia

Title:	Update to S3-251845 on Finer level of authorization

Document for:	Approval

Agenda item:	4.22

Spec:	3GPP TS/TR 33.122

Version:	19.0.0

Work Item:	CAPIF_Ph3_sec

Comments

pCR to S3-251845 Living CR on Finer level of authorization

This pCR is addressing EN of SA6 TS: Authorization related to network slice are to be detailed by SA3.

******* START OF CHANGES

## 6.Y	Authorization for finer level service API access

The authorization function shall support finer level authorization as specified in TS 23.222 [X]. In both RNAA and non-RNAA scenarios, finer level service API access authorization can be used to limit access to specific API service resources and service API operations.

To enable finer level service API access, the authorization request or access token request includes the requested service resources and service operations at the respective granularity. For RNAA, the request may also include the resource owner ID.

If the request includes desired Network Slice Info of the service API, the authorization request or access token request shall include related details.

If the API invoker is authorized, the CCF responds to the API invoker with an access token including finer level authorization information, or an authorization code.

******* END OF CHANGES