3GPP TSG-SA3 Meeting #122	S3-252284

Fukuoka, Japan,  19 - 23 May 2025                                                 (merger of S3-251938, S3-252103, S3-252188)

Source:	ZTE, Apple, LG  Electronics

Title:	Update conclusion for KI#1

Document for:	Approval

Agenda item:	4.12

Spec:

Version:

Work Item:	Security aspects of NR mobility enhancement Phase 4

Comments

This pCR proposes to update the conclusion for KI#1. According to the response from RAN3 (R3-252449), the new source gNB needs to provide the new KgNB*(s) to the corresponding candidate gNB via a LTM Configuration Update procedure. Therefore, it is proposed to conclude on KI#1 based on this principle.

Proposed Changes

* * * First Change * * * *

5	Conclusions

5.1	Conclusion for key issue #1

For the case where CU is acting as MN and DC is not configured, it is agreed that the normative work is based on the following principles:

-	During LTM preparation phase, the serving gNB sends the {KNG-RAN*, NCC} pair (per candidate cell), UE's 5G security capabilities, ciphering and integrity algorithms used in the serving cell and the UE's UP security policy along with the UE security context to the candidate gNB(s).

-	Whenever the serving gNB generates the KNG-RAN* (per candidate cell) after LTM preparation phase, the serving gNB sends the generated {KNG-RAN*, NCC} pair (per candidate cell) to the candidate gNB(s) using LTM Configuration Update message.

-	During LTM cell switch execution phase, the serving gNB includes NCC value in the LTM cell switch command  MAC CE.

-	For the parameters in AS security context, which are not changing per inter-CU LTM handover (including UE’s 5G security capabilities, selected AS security algorithms, UE’s UP security policy, UE’s UP security activation status):

-	If the gNB receives mismatched UE security context (e.g. UE’s 5G security capabilities and/or UE’s UP security policy) in the Path-Switch Acknowledge message after LTM switch,

-	it initiates an intra-cell handover procedure which includes RRC Connection Reconfiguration procedure indicating the change of selected algorithms (as defined in clause 6.7.3.1) and/or UE’s UP activation status (as defined in clause 6.6.1) to the UE.

-	it updates the UE’s 5G security capabilities and/or UE’s UP security policy for the active PDU session to all candidate gNBs.

* * * End of Changes * * * *