3GPP TSG-SA3 Meeting #122	S3-251849

Fukuoka, Japan, 19 – 23 May 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.511 | CR | 0086 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of test names and Clean up of 33.511 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BSI (DE) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G_Maint |  |  |  |  |  | Date: |  |  | 2025-04-24 |
|  | D |  |  |  |  |  |  |  |  |  |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Missing test names, some format glitches and inconsistent header wording |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of test name, format changes and correction of header wording |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Non-standard format remains and test automation may be affected |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.1.1, 4.2.2.1.2, 4.2.2.1.4 - 4.2.2.1.13, 4.2.2.1.15, 4.2.2.1.19, 4.2.2.1.22, 4.2.2.1.23 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** START OF 1st CHANGE **********

##### 4.2.2.1.1	Integrity protection of RRC-signalling

Requirement Name: Integrity protection of RRC-signalling

Requirement Reference: TS 33.501 [2], clause 5.3.3

Requirement Description: The gNB supports integrity protection and replay protection of RRC-signalling as specified in TS 33.501 [2], clause 5.3.3.

Threat References: TR 33.926 [5], clause D.2.2.2 – Control plane data integrity protection.

Test Case:

Test Name: TC_CP_DATA_INT_RRC-SIGN_gNB

Purpose:

To verify that the RRC-signalling data sent between UE and gNB over the NG RAN air interface are integrity protected.

Pre-Condition:

-	The gNB network product shall be connected in emulated/real network environments. UE may be simulated.

-	Tester shall have access to the integrity algorithm and the integrity protection keys.

-	The tester can capture the message via the NG RAN air interface, or can capture the message at the UE.

-	The NIA0 is disabled at UE and gNB.

Execution Steps:

1.	The tester triggers the gNB to send AS SMC message to the UE, and UE responses AS SMP.

2.	The tester checks any RRC message sent by gNB after sending AS SMC and before UE enters CM-Idle state is integrity protected.

Expected Results:

Any RRC-signalling over the NG RAN air interface is integrity protected after gNB sending AS SMC.

Expected format of evidence:

Evidence suitable for the interface, e.g. Screenshot containing the operational results.

********** END OF 1st CHANGE **********

********** START OF 2nd CHANGE **********

##### 4.2.2.1.2	Integrity protection of user data between the UE and the gNB

Requirement Name: Integrity protection of user data between the UE and the gNB.

Requirement Reference: TS 33.501 [2], clause 5.3.3

Requirement Description: The gNB supports integrity protection and replay protection of user data between the UE and the gNB as specified in TS 33.501 [2], clause 5.3.3.

NOTE: 	This requirement does not apply to the gNB that is used as a secondary node connecting to the EPC.

Threat References: TR 33.926 [5], clause D.2.2.4 – User plane data integrity protection.

Test Case:

Test Name: TC-UP-DATA-INT_gNB

Purpose:

To verify that the user data packets are integrity protected over the NG RAN air interface.

Pre-Condition:

-	The gNB network product shall be connected in emulated/real network environments. UE may be simulated.

-	Tester shall enable the user plane integrity protection and ensure NIA0 is not used.

-	Tester shall have knowledge of integrity algorithm and integrity protection keys.

-	The tester can capture the message via the NG RAN air interface, or can capture the message at the UE.

-	The NIA0 is disabled at the UE and gNB.

Execution Steps:

1.	The tester triggers the gNB to send RRCConnectionReconfiguration with integrity protection indication "on".

2.	The tester checks any User data sent by gNB after sending RRCConnectionReconfiguration and before UE enters CM-Idle state is Integrity protected.

Expected Results:

Any user plane packets sent between UE and gNB over the NG RAN air interface after gNB sending RRCConnectionReconfiguration is integrity protected.

Expected format of evidence:

Evidence suitable for the interface e.g. Screenshot containing the operational results.

********** END OF 2nd CHANGE **********

********** START OF 3rd CHANGE **********

##### 4.2.2.1.4	RRC integrity check failure

Requirement Name: RRC integrity check failure

Requirement Reference: TS 33.501 [2], clause 6.5.1

Requirement Description: The RRC integrity checks are performed both in the ME and the gNB. In case failed integrity check (i.e. faulty or missing MAC-I) is detected after the start of integrity protection, the concerned message is discarded. This can happen on the gNB side or on the ME side, as specified in TS 33.501 [2], clause 6.5.1.

Threat References: TR 33.926 [5], clause D.2.2.2, Control plane data integrity protection

Test Case:

Test Name: TC-CP-DATA-RRC-INT-CHECK_gNB

Purpose:

Verify that RRC integrity check failure is handled correctly by the gNB.

Pre-Conditions:

Test environment with a UE. The UE may be simulated. RRC integrity protection is activated at the gNB.

Execution Steps

The UE sends a RRC message to the gNB without MAC-I;

or

The UE sends a RRC message to the gNB with a wrong MAC-I.

NOTE:	RRC integrity protection is provided by PDCP. In a PDCP message without MAC-I, the last 4 Bytes of the PCDP Data will be interpreted as a wrong MAC-I and therefore the integrity check will fail.

Expected Results:

The RRC message is discarded by the gNB.

Expected format of evidence:

Sample copies of the log files.

********** END OF 3rd CHANGE **********

********** START OF 4th CHANGE **********

##### 4.2.2.1.5	UP integrity check failure

Requirement Name: UP integrity check failure

Requirement Reference: TS 33.501 [2], clause 6.6.4

Requirement Description: If the gNB or the UE receives a PDCP PDU which fails integrity check with faulty or missing MAC-I after the start of integrity protection, the PDU is discarded as specified in TS 33.501 [2], clause 6.6.4.2.

Threat References: TR 33.926 [5], clause D.2.2.4, User plane data integrity protection

Test Case:

Test Name: TC_GNB_UP_INTEGRITY_CHECK_FAIL

Purpose:

Verify that UP integrity check failure is handled correctly by the gNB.

Pre-Conditions:

Test environment with a UE. The UE may be simulated. UP integrity protection is activated at the gNB.

Execution Steps

The UE sends a PDCP PDU to the gNB without MAC-I;

or

The UE sends a PDCP PDU to the gNB with a wrong MAC-I.

NOTE:	In a PDCP PDU message without MAC-I, the last 4 Bytes of the PCDP PDU Data will be interpreted as a wrong MAC-I and therefore the integrity check will fail.

Expected Results:

The PDCP PDU is discarded by the gNB.

Expected format of evidence:

Evidence suitable for the interface e.g. Screenshot containing the operational results.

********** END OF 4th CHANGE **********

********** START OF 5th CHANGE **********

##### 4.2.2.1.6	Ciphering of RRC-signalling

Requirement Name: Ciphering of RRC-signalling

Requirement Reference: TS 33.501 [2], clause 5.3.2

Requirement Description: The gNB supports ciphering of RRC-signalling as specified in TS 33.501 [2], clause 5.3.2.

Threat References: TR 33.926 [5], clause D.2.2.1 – Control plane data confidentiality protection.

Test Case:

Test Name: TC-CP-DATA-CIP-RRC-SIGN_gNB

Purpose:

To verify that the RRC-signalling data sent between UE and gNB over the NG RAN air interface are confidentiality protected.

Pre-Condition:

-	The gNB network product shall be connected in emulated/real network environments. The UE may be simulated.

-	The tester shall have access to the NG RAN air interface or can capture the message at the UE.

Execution Steps:

1.	The tester triggers the UE to send a Registraton Request to the AMF.

2.	The AMF sends a KgNB and the UE security capability to the gNB.

3.	The gNB selects an algorithm and sends AS SMC to the UE.

4.	The gNB receive AS SMP from the UE.

Expected Results:

Control plane packets sent to the UE after the gNB sends AS SMC is ciphered.

Expected format of evidence:

Evidence suitable for the interface, e.g. Screenshot containing the operational results.

********** END OF 5th CHANGE **********

********** START OF 6th CHANGE **********

##### 4.2.2.1.7	Ciphering of user data between the UE and the gNB

Requirement Name: Ciphering of user data between the UE and the gNB

Requirement Reference: TS 33.501 [2], clause 5.3.2

Requirement Description:  The gNB supports ciphering of user data between the UE and the gNB. as specified in TS 33.501 [2], clause 5.3.2.

Threat References: TR 33.926 [5], clause D.2.2.3 – User plane data confidentiality protection at gNB

Test Case:

Test Name: TC-UP-DATA-CIP_gNB

Purpose:

To verify that the user data packets are confidentiality protected over the NG RAN air interface.

Pre-Condition:

-	The gNB network product shall be connected in emulated/real network environments. The UE may be simulated.

-	The tester shall have access to the NG RAN air interface or can capture the message at the UE.

Execution Steps:

1.	The tester triggers theUE to send PDU session establishment Request to the SMF.

2.	The SMF sends a UP security policy with UP ciphering required or preferred to the gNB.

3.	The gNB sends RRCConnectionReconfiguration with ciphering protection indication "on".

4.	The tester checks any user data sent by the gNB after sending RRCConnectionReconfiguration and before the UE enters into CM-Idle state.

Expected Results:

The user plane packets sent to the UE after the gNB sends RRCConnectionReconfiguration is confidentiality protected.

Expected format of evidence:

Evidence suitable for the interface e.g. Screenshot containing the operational results.

********** END OF 6th CHANGE **********

********** START OF 7th CHANGE **********

##### 4.2.2.1.8	Replay protection of user data between the UE and the gNB

Requirement Name: Replay protection of user data between the UE and the gNB.

Requirement Reference: TS 33.501 [2], clause 5.3.3

Requirement Description: The gNB supports integrity protection and replay protection of user data as specified in TS 33.501 [2], clause 5.3.3.

Threat References: TR 33.926 [5], clause D.2.2.4 – User plane data integrity protection.

Test Case:

Test Name: TC-UP-DATA-REPLAY_gNB

Purpose:

To verify that the user data packets are replay protected over the NG RAN air interface.

Pre-Condition:

-	The gNB network product shall be connected in emulated/real network environments. The UE may be simulated.

-	The tester shall have access to the NG RAN air interface.

-	The tester shall have access to the N3 interface.

-	The tester shall activate the user plane integrity protection of user data packets.

Execution Steps:

1.	The tester shall capture the user plane data sent between UE and gNB using any network analyser over the NG RAN air interface.

2.	Tester shall filter user plane data packets sent between UE and gNB.

3.	Tester shall replay the captured user plane packets or shall use any packet crafting tool to create a user plane packet similar to the captured user plane packet and replay to the gNB.

4.	Tester shall check whether the replayed user plane packets were processed by the gNB by capturing over NG RAN air interface to see if any corresponding response message is received from the gNB or by verifying the gNB log files if there are entries about UP packet discard or by capturing the N3 interface to see if any of the replayed user plane packets have been forwarded by the gNB.

5.	Tester shall confirm that gNB provides replay protection by dropping/ignoring the replayed packet if no corresponding response is received from the gNB to the replayed packet or if the gNB logs include corresponding log entries or if no replayed user plane packets have been forwarded over the N3 interface.

6.	Tester shall verify from the result that if the replayed user plane packets are not accepted by gNB, the NG RAN air interface is replay protected.

Expected Results:

The user plane packets sent between the UE and gNB over the NG air interface is replay protected.

Expected format of evidence:

-	Evidence suitable for the interface, e.g. Screenshot containing the operational results.

-	Log files, e.g., containing corresponding log events.

********** END OF 7th CHANGE **********

********** START OF 8th CHANGE **********

##### 4.2.2.1.9	Replay protection of RRC-signalling

Requirement Name: Replay protection of RRC-signalling.

Requirement Reference: TS 33.501 [2], clause 5.3.3

Requirement Description: The gNB supports integrity protection and replay protection of RRC-signalling as specified in TS 33.501 [2], clause 5.3.3.

Threat References: TR 33.926 [5], clause D.2.2.2 – Control plane data integrity protection.

Test Case:

Test Name: TC-UP-DATA-RRC-REPLAY_gNB

Purpose:

To verify the replay protection of RRC-signalling between UE and gNB over the NG RAN air interface.

Pre-Condition:

-	The gNB network product shall be connected in emulated/real network environments.

-	Tester shall have knowledge of the integrity algorithm and the corresponding protection keys.

-	The tester shall have access to the NG RANs air interface.

-	The tester shall have access to the N2 interface.

-	The tester shall activate the integrity protection of RRC-signalling.

Execution Steps:

1.	The tester shall capture the data sent between UE and the gNB using any network analyser over the NG RAN air interface.

2.	Tester shall filter RRC signalling packets.

3.	Tester shall check for the PDCP COUNT of the filtered RRC signalling packets and shall use any packet crafting tool to create RRC signalling packets similar to the captured packets or the tester shall replay the captured RRC uplink packet to the gNB to perform the replay attack over gNB.

4.	Tester shall check whether the replayed RRC signalling packets were processed by the gNB or not, by capturing over NG RAN air interface to see if any corresponding response message is received from the gNB or by verifying the gNB log files if there are entries about RRC packet discard or by capturing the N2 interface to see if any of the replayed user plane packets have been forwarded by the gNB.

5.	Tester shall confirm that gNB provides replay protection by dropping/ignoring the replayed packet if no corresponding response is sent by the gNB to the replayed packet or if the gNB logs include corresponding log entries or if no replayed user plane packets have been forwarded over the N2 interface.

Expected Results:

The RRC signalling over the NG RAN air interface is replay protected.

-

Expected format of evidence:

- Evidence suitable for the interface, e.g. Screenshot containing the operational results.

- Log files, e.g., containing corresponding log events.

********** END OF 8th CHANGE **********

********** START OF 9th CHANGE **********

##### 4.2.2.1.10	Ciphering of user data based on the security policy sent by the SMF

Requirement Name: Ciphering of user data based on the security policy sent by the SMF

Requirement Reference: TS 33.501 [2], clause 5.3.2

Requirement Description: The gNB activates ciphering of user data based on the security policy sent by the SMF as specified in TS 33.501 [2], clause 5.3.2.

Threat References: TR 33.926 [5], clause D.2.2.8 – Security Policy Enforcement.

Test Case:

Test Name: TC-UP-DATA-CIP-SMF

Purpose:

To verify that activation of confidentiality protection for user data is based on the security policy sent by the SMF via AMF

Pre-Condition:

-	The gNB network product shall be connected in emulated/real network environments. The UE and the 5GC may be simulated.

-	The tester shall have access to the NG RAN air interface.

-	The tester shall have knowledge of the RRC and UP ciphering algorithm and protection keys.

-	RRC ciphering is already activated at the gNB.

Execution Steps:

All execution steps are to be performed two times. Once with the UP security policies’ ciphering protection in step 2 set to "required" and the second time set to "not needed".

1.	The tester triggers PDU session establishment procedure by sending PDU session establishment request message.

2.	Tester shall trigger the SMF to send the UP security policy with ciphering protection "required" or "not needed" to the gNB.

3.	The tester shall capture the RRC reconfiguration procedure between gNB to UE over NG RAN air interface. And filter the RRC reconfiguration message sent by gNB to UE.

4.	The tester shall decrypt the RRC Reconfiguration message and retrieve the UP ciphering protection indication present in the captured message.

5.	The tester shall verify if the UP security policy received at gNB is same as the UP ciphering protection indication notified by the gNB to the UE in the RRC Reconfiguration message.

6.	Tester shall capture the RRC Reconfiguration complete message sent between UE and gNB.

7.	Tester shall capture the user plane data sent between UE and gNB using any network analyser.

Expected Results:

When the received UP cipher protection indication is set to “required”, the captured user plane data appear to be garbled (i.e. no longer plaintext) and the user plane packets are confidentiality protected based on the UP security policy sent by the SMF.

When the received UP cipher protection indication is set to "not needed", the captured user plane data appear to be plaintext and the user plane packets are not confidentiality protected based on the UP security policy sent by the SMF.

Expected format of evidence:

Evidence suitable for the interface, e.g. Screenshot containing the operational results.

********** END OF 9th CHANGE **********

********** START OF 10th CHANGE **********

##### 4.2.2.1.11	Integrity of user data based on the security policy sent by the SMF

Requirement Name: Integrity of user data based on the security policy sent by the SMF

Requirement Reference: TS 33.501 [2], clause 5.3.3

Requirement Description: The gNB activates integrity protection of user data based on the security policy sent by the SMF as specified in TS 33.501 [2], clause 5.3.3.

Threat References: TR 33.926 [5], clause D.2.2.8 – Security Policy Enforcement.

Test Case:

Test Name: TC-UP-DATA-INT-SMF

Purpose:

To verify that activation of integrity protection for user data packets is based on the security policy sent by the SMF.

Pre-Condition:

-	The gNB network product shall be connected in emulated/real network environments. The UE and the 5GC may be simulated.

-	The tester shall have access to the NG RAN air interface.

-	The tester shall have knowledge of the integrity algorithm and protection keys.

-	RRC integrity is activated at the gNB.

Execution Steps:

All execution steps are to be performed two times. Once with the UP security policies’ ciphering protection in step 2 set to "required" and the second time set to "not needed".

1.	The tester triggers PDU session establishment procedure by sending PDU session establishment request message.

2.	Tester shall trigger the SMF to send the UP security policy with integrity protection is "required" or "not needed" to the gNB.

3.	The tester shall capture the RRC reconfiguration message sent by gNB to UE over NG RAN air interface.

4.	The tester shall decrypt the RRC reconfiguration message and retrieve the UP integrity protection indication presenting in the decrypted message.

5.	Tester shall check whether UP integrity is enabled /disabled to verify if the UP security policy received at gNB is same as the UP integrity protection indication notified by the gNB to the UE in the RRC reconfiguration message.

6.	Tester shall capture the user plane data sent between UE and gNB using any network analyser.

Expected Results:

When the received UP integrity protection is set to "required", the user plane packets are integrity protected based on the security policy sent by the SMF.

When the received UP integrity protection is set to "not needed", the user plane packets are not integrity protected based on the security policy sent by the SMF.

Expected format of evidence:

Evidence suitable for the interface, e.g. Screenshot containing the operational results.

********** END OF 10th CHANGE **********

********** START OF 11th CHANGE **********

##### 4.2.2.1.12	AS algorithms selection

Requirement Name: AS algorithms selection

Requirement Reference: TS 33.501 [2], clause 6.7.3.0 and clause 5.11.2.

Requirement Description: The gNB selects the algorithms to use dependent on: the UE security capabilities of the UE, the configured allowed list of security capabilities of the currently gNB as specified in TS 33.501 [2], clause 5.11.2.

Each gNB is configured via network management with lists of algorithms which are allowed for usage. There is one list for integrity algorithms, and one for ciphering algorithms. These lists are ordered according to a priority decided by the operator as specified in TS 33.501 [2], clause 6.7.3.0.

Threat References: TR 33.926 [5], D.2.2.5 – AS algorithm selection and use

Test Case:

Test Name: TC-AS-alg-select_gNB

Purpose:

Verify that the gNB selects the algorithms with the highest priority in its configured list.

Pre-Conditions:

Test environment with the gNB has been pre-configured with allowed security algorithms with priority.

Execution Steps

1)	The tester triggers the UE to send registration request message to the gNB.

2)	The gNB receives UE context setup request message.

3)	The gNB sends the AS SECURITY MODE COMMAND message.

4)	The UE replies with the AS SECURITY MODE COMPLETE message.

Expected Results:

The gNB initiates the SECURITY MODE COMMAND message that includes the chosen algorithm with the highest priority according to the ordered lists and is contained in the UE NR security capabilities.

The MAC in the AS SECURITY MODE COMPLETE message is verified, and the AS protection algorithms are selected and applied correctly.

Expected format of evidence:

Sample copies of the log files.

********** END OF 11th CHANGE **********

********** START OF 12th CHANGE **********

##### 4.2.2.1.13	Key refresh at the gNB

Requirement Name: Key refresh at the gNB

Requirement Reference: TS 33.501 [2], clause 6.9.4.1; TS 38.331 [6], clause 5.3.1.2

Requirement Description: Key refresh is possible for KgNB, KRRC-enc, KRRC-int, KUP-enc, and KUP-int (if available), and is to be initiated by the gNB/ng-eNB when a PDCP COUNTs are about to be re-used with the same Radio Bearer identity and with the same KgNB. as specified in TS 33.501 [2], clause 6.9.4.1.

The network is responsible for avoiding reuse of the COUNT with the same RB identity and with the same key, e.g. due to the transfer of large volumes of data, release and establishment of new RBs, and multiple termination point changes for RLC-UM bearers and multiple termination point changes for RLC-AM bearer with SN terminated PDCP re-establishment (COUNT reset) due to SN only full configuration whilst the key stream inputs (i.e. bearer ID, security key) at MN have not been updated. In order to avoid such re-use, the network e.g. uses different RB identities for RB establishments, change the AS security key, or an RRC_CONNECTED to RRC_IDLE/RRC_INACTIVE and then to RRC_CONNECTED transition as specified in TS 38.331 [6], clause 5.3.1.2.

Threat References: TR 33.926 [5], clause D.2.2.7 Key Reuse

Test Case:

Test Name: TC_GNB_KEY_REFRESH_DRB_ID

Purpose:

Verify that the gNB performs KgNB refresh when DRB-IDs are about to be reused under the following conditions:

-	the successive Radio Bearer establishment uses the same RB identity while the PDCP COUNT is reset to 0, or

-	the PDCP COUNT is reset to 0 but the RB identity is increased after multiple calls and wraps around.

Pre-Conditions:

The UE, AMF and SMF may be simulated.

Execution Steps

1) 	The tester triggers the gNB to send the AS Security Mode Command message to the UE.

2)	The UE responds with the AS Security Mode Complete message.

3)	A DRB is set up.

4)	The tester sets up and tears down the DRB for multiple times within one active radio connection without the UE going to idle (e.g. by triggering the UE to make multiple IMS calls, or by triggering the SMF to request PDU session modification and deactivation via the AMF), until the DRB ID is reused.

Expected Results:

Before DRB ID reuse, the gNB takes a new KgNB into use by e.g. triggering an intra-cell handover or triggering a transition from RRC_CONNECTED to RRC_IDLE or RRC_INACTIVE and then back to RRC_CONNECTED.

NOTE:	Random Access Procedure defined in clause 9.2.6 of TS 38.300[8] runs in the above procedures.

Expected format of evidence:

Part of log that shows all the DRB identities and the corresponding procedure. This part can be presented, for example, as a screenshot.

********** END OF 12th CHANGE **********

********** START OF 13th CHANGE **********

##### 4.2.2.1.15	AS protection algorithm selection in gNB change

Requirement Name: AS protection algorithm selection in gNB change.

Requirement Reference: TS 33.501 [2], clauses 6.7.3.1 and 6.7.3.2

Requirement Description: The target gNB/ng-eNB selects the algorithm with highest priority from the received 5G security capabilities of the UE according to the prioritized locally configured list of algorithms (this applies for both integrity and ciphering algorithms). The chosen algorithms are indicated to the UE in the Handover Command message if the target gNB/ng-eNB selects different algorithms compared to the source gNB/ng-eNB as specified in TS 33.501 [2], clause 6.7.3.1, and clause 6.7.3.2.

Threat References: TR 33.926 [5], D.2.2.5 – AS algorithm selection and use

Test Case:

Test Name: Alg_select_change_gNB

Purpose:

Verify that AS protection algorithm is selected correctly.

Pre-Conditions:

Test environment with source gNB, target gNB and AMF. Source gNB and AMF may be simulated.

Execution Steps:

Test Case 1:

1.	The tester triggers the source gNB to transfer the ciphering and integrity algorithms used in the source cell to the target gNB in the handover request message.

2.	Target gNB verifies the algorithms and selects AS algorithms which have the highest priority according to the ordered lists. Target gNB includes the algorithm in the handover command.

Test Case 2:

1.	The tester triggers the AMF to send the UE NR security capability to the Target gNB.

2.	The target gNB selects the AS algorithms which have the highest priority according to the ordered lists in the HANDOVER COMMAND.

The above test cases assume that the algorithms selected by the target gNB are different from the ones received from the source gNB.

Expected Results:

For both test cases:

The selected AS ciphering and integrity protection algorithms in the handover complete message have the highest priority according to the ordered list.

The MAC in the handover complete message is valid and is based on the correctly selected AS ciphering and integrity protection algorithms.

Expected format of evidence:

Snapshots containing the result.

********** END OF 13th CHANGE **********

********** START OF 14th CHANGE **********

##### 4.2.2.1.19	UP security activation in Inactive scenario

Requirement Name: UP security activation in Inactive scenario

Requirement Reference: TS 33.501 [2], clause 6.8.2.1.3.

Requirement Description: If the UP security activation status can be supported in the target gNB/ng-eNB, the target gNB/ng-eNB uses the UP security activations that the UE used at the last source cell. Otherwise, the target gNB/ng-eNB responds with an RRC Setup message to establish a new RRC connection with the UE as specified in TS 33.501 [2], clause 6.8.2.1.3.

Threat Reference:  TR 33.926 [5], clause D.2.2.9 State transition from inactive state to connected state.

Test Case:

Test Name: TC_GNB_INACTIVE_TO_ACTIVE

Purpose:

Verify that the target gNB/ng-eNB uses the UP security activation status to activate the UP security.

Pre-Conditions:

-	The gNB network product shall be connected in emulated/real network environments.

-	The UE may be simulated.

Execution Steps

1.	The tester shall complete a Registration Procedure and PDU Session establishment procedure to make sure the gNB configure the UP security, and get the UP security activation status.

2.	The gNB sends RRC Release message with a suspend config to the UE.

3.	The tester deletes the UP security activation status of the UE.

4.	The tester triggers the UE to send RRC Resume message.

Expected Results:

The gNB sends RRC Setup message to the UE.

Expected format of evidence:

Screenshot containing the operational results.

********** END OF 14th CHANGE **********

********** START OF 15th CHANGE **********

##### 4.2.2.1.22	Checking expiry certificate

Requirement Name: Expired Certificate checking at base station

Requirement Reference: TS 33.310 [10], clause 7.2

Requirement Description:

Certificate Management Protocol v2 (CMPv2) [11] may be the supported protocol to provide certificate lifecycle management capabilities for TLS entities. All TLS entities and TLS CAs may support initial enrolment by the TLS entity to the TLS CA via CMPv2, i.e. receiving a certificate from the TLS CA, and updating the key of the certificate via CMPv2 before the certificate expires.

Threat References: TBD

Test case:

Test Name: TC_EXPIR_CERT_CHCK

Purpose:

Verify that the gNB can check whether its certificate issued by operator CA is about to expire and to act accordingly.

Pre-Conditions:

-	If the gNB under test does not support handling certificates as defined in clause 9 of TS 33.310[10], this test does not apply.

-	The gNB network product is connected in emulated/real network environments.

-	A TLS CA may be emulated, if needed.

-	The gNB is configured the necessary information to connect with the CA server.

-	Optionally, the vendor may provide necessary information in a document, e.g. describing how to handle the case when a gNB checks the operator certificate is about to be expired.

Execution Steps

1.	The tester configures the gNB with certificates that is assigned by operator CA.

2.	The tester configures the UTC timer of gNB to the time when its own certificate is about to be expired.

3.	The gNB initiates the CMPv2 procedure to get the new operator certificate.

Expected Results:

-	The gNB raises an alarm or requests a new certificate from the CA.

-	The gNB received a new operator certificate.

Expected format of evidence:

The logs and the communication flow in a .pcap file.

********** END OF 15th CHANGE **********

********** START OF 16th CHANGE **********

##### 4.2.2.1.23	Peer certificate checking

Requirement Name: Peer certificate checking at base station

Requirement Reference: TS 33.310 [10], clause 9.5.1

Requirement Description:

-	The base station may be pre-provisioned with the operator root CA certificate.

-	If the base station is not pre-provisioned with the operator root CA certificate, then the base station takes the operator root certificate from the certificates received in the initialization response. The selection is based on checking which root certificate can be used to validate the received base station certificate.

NOTE1:	Examples on validation factors defined in clause 6.3.1 of TS 33.310[10] can be taken into consideration, for example, whether the certificated has been revoked, expired, on the CRL distribution point is empty, etc.

Test case:

Test Name: TC_PEER_CERT_CHCK

Purpose:

Verify that the gNB has the ability to check the peer certificate is valid or not.

Pre-Conditions:

-	If the gNB under test does not support handling certificates as defined in clause 9 of TS 33.310[10], this test does not apply.

-	The gNB is configured with or has obtained an operator root CA certificate.

-	The gNB network product shall be connected in emulated/real network environments.

-	A peer, e.g., AMF, SEG, gNB may be emulated.

NOTE2:	According to 5GS, only AMF, SEG/UPF, gNB can connect to a gNB. The peer means the network function that provides the operator assigned certificate to the gNB for establishing the N2, N3, Xn interfaces.

-	The gNB is configured the necessary information to connect with the peer.

-	Optionally, the vendor may provide necessary information in a document, e.g., describing the checking factors and how to handle the case when a gNB validate the certificate is invalid.

Execution Steps:

1.	The tester configures the peer with an invalid certificate e.g., by using a wrong signature or a certificate that has expired.

2.	The tester triggers the gNB to connect to the peer.

Expected Results:

-	The gNB does not connect with the peer and may raise an alarm at the same time.

Expected format of evidence:

The logs and the communication flow in a .pcap file.

********** END OF CHANGE **********