3GPP TSG-SA3 Meeting #122	S3-252031

# Fukuoka, Japan, 19 – 23 May 2025

Source:	Nokia

Title:	Proposal regarding AIOTF requirements.

Document for:	Approval

Agenda item:	4.26

Spec:	3GPP TS 33.369

Version:	0.1.0

Work Item:	AmbientIoT-SEC

Comments

This pCR proposes requirements based on the currently drawn conclusions in TR 33.713.

Proposed Changes

* * * First Change * * * *

### 4.2.2	Requirements on the AIOTF

The AIOTF shall support ciphering of command messages between the AIoT device and the AIOTF.

The AIOTF shall implement the following ciphering algorithms:

NEA0, 128-NEA1 and 128-NEA2 as defined in Annex D of the TS 33.501 [5].

The AIOTF may implement 128-NEA3.

The AIOTF shall support integrity protection and replay protection of command messages between the AIOT device and the AIOTF.

The AIOTF shall implement the following integrity protection algorithms:

NIA0, 128-NIA1 and 128-NIA2 as defined in Annex D of the TS 33.501 [5].

The AIOTF may implement 128-NIA3.

The AIOTF shall authenticate an AIOT device.

NOTE: Whether to enforce one-way or mutual authentication is part of AIOTF configuration and left for implementation.

Editor’s Note:	Requirements concerning protection of identifiers is not reflected in the current requirements.

Editor’s Note:	Further requirements are FFS.

* * * End of Change * * * *