3GPP TSG-SA3 Meeting #122 	S3-252103

# Fukuoka, Japan, 19 - 23 May 2025

# Source:	Apple

# Title:	Conclusion for LTM

# Document for:	Approval

Agenda Item:	4.12

1	Decision/action requested

Approve the pCR to the living CR of LTM security

- References

- S3-250765 Living CR for NR mobility enhancement

- S3-251124 Reply LS on security handling for inter-CU LTM in non-DC cases

3	Rationale

This contribution proposes the conclusion for LTM security based on the agreement in S3-251124, which is the LS sent from SA3 to RAN2 in SA3#120 meeting in February.

4	Detailed proposal

*** Start of 1st Change ***

5	Conclusions

5.1	Conclusion for key issue #1

For the case where CU is acting as MN and DC is not configured, it is agreed that the normative work is based on the following principles:

-	For the parameters in AS security context, which are not changing per inter-CU LTM handover (including UE’s 5G security capabilities, selected AS security algorithms, UE’s UP security policy, UE’s UP security activation status):

-	If the gNB receives mismatched UE security context (e.g. UE’s 5G security capabilities and/or UE’s UP security policy) in the Path-Switch Acknowledge message after LTM switch,

-	it initiates an intra-cell handover procedure which includes RRC Connection Reconfiguration procedure indicating the change of selected algorithms (as defined in clause 6.7.3.1) and/or UE’s UP activation status (as defined in clause 6.6.1) to the UE.

-	it updates the UE’s 5G security capabilities and/or UE’s UP security policy for the active PDU session to all candidate gNBs.

Editor’s Note: The conclusion for the parameters in AS security context, which are changing per each inter-CU LTM handover (including NCC value, keySetChangeIndicator) is FFS.

-	For the parameters in AS security context, which are changing per inter-CU LTM handover (including UE’s 5G security capabilities, selected AS security algorithms, UE’s UP security policy, UE’s UP security activation status), it is agreed that the NCC can be transmitted to the UE in plaintext via MAC CE message, even though the unprotected MAC leads to increased attack scenarios and probability of handover failure.

NOTE 1: In addition, SA3 will not consider the protection of MAC CE messages in this release, but may consider studying MAC CE protection in the future.

NOTE 2: The details on key distribution between the source gNB and the target gNB are left to RAN3 to decide.

*** End of 1st Change ***