3GPP TSG-SA3 Meeting #122	S3-252123

# Fukuoka, Japan, 19 - 23 May 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 2140 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addressing the ENs of Key derivation, Nonce and counter values, and VCID uniqueness in the forwarding mode. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2-Sec |  |  |  |  |  | Date: |  |  | 2025-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Some editor’s notes remain, and further detailing is needed for Stage 3 to complete the work. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Resolution of editor’s notes related to key derivation, nonce and counter values, and VCID uniqueness in the case of using forwarded mode. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No protection of the MRI information. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 18.2.5, 18.2.6 and 18.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**************** START of CHANGE****************

18.2.5	Key derivation in the Forwarded mode

The keys are extracted from the QUIC connection between UPF and AS proxy using the TLS 1.3 key exporter defined in RFC 8446 [77].

The following input shall be used for the TLS 1.3 key exporter:

Label:  ‘EXPORTER_3GPP_XRM_MASTER_AESGCM (or AESCCM)’

Context_value: The VCID value || The forty (40) most significant bits of the counter

Key_length:  256 bits [77]

NOTE: The labels shall be registered in IANA.

18.2.6	Nonce and counter values in the Forwarded mode

The nonce shall consist of either the 32 least significant bits of VCID concatenated with the 64-bit counter C (for GCM, 96-bit nonce) or the 64-bit counter C alone (for CCM, 64-bit nonce), where the VCID portion shall remain constant during the session.

To maintain synchronization even for large burst losses of XRM transformed packets, the full 64-bit counter is sent every time the 16-bit Least Significant Bit counter wraps. The counter is sent reliably on the corresponding HTTP/3 request QUIC stream.

The receiver of the transformed packet, i.e. the UPF for AS to UE traffic, shall perform replay protection to ensure that each 64-bit counter is only accepted once. In case the receiver detects reuse of the counter, the e2e packet shall be discarded (i.e. not forwarded).

18.2.7	VCID uniqueness in the forwarding mode

The VCID value must be unique and shall never be reused for a particular HTTP/3 sessions’ QUIC connection to ensure secure Media related information (MRI) protection in forwarded mode.

**************** END of CHANGE****************