3GPP TSG-SA3 Meeting #122																			S3-252242

Fukuoka, Japan, 19 – 23 May 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.700-22 | CR | 0005 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Solution 11 editorial updates |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_CAPIF_PH3-sec |  |  |  |  |  | Date: |  |  | 2025-05-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Need for text corrections, typo and figure updates |  |  |  |  |  |  |  |  |
| Summary of change: |  | Implementing as provided in reason for changes |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Readibility issue in text in figure |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.11, 6.11.1, 6.11.2, 6.11.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********* START OF CHANGES

## 6.11	Solution #11: Client initiated backchannel authorization (CIBA)

### 6.11.1	Introduction

This solution addresses key issue #3 on Authorizing API invoker on one UE accessing resources related to another UE.

More precisely, this solution addresses the requirements in key issue #3 by allowing offline interaction with the resource owner, after triggering of API invocation by the API invoker.

In some cases, there can be no preconfigured authorization information and at the API invocation time the resource owner is not using the application which needs to direct the resource owner to the authorization server. UE-originated API invocation (Location tracking) captured in Annex A.2 of TR 23.700-95 [13] can be given as an example use case. Client credentials (requiring a preconfigured authorization data) and authorization code flow (including PKCE) (requiring resource owner accessible by the API invoker at the API invocation time) are not enough to address these type of use cases. Utilization of client-initiated backchannel authorization (CIBA) [11] flow is proposed for such use cases.

### 6.11.2	Solution details

High-level steps of the solution are presented in Figure 6.11.2-1 and explained below. For details the CIBA flow [11] can be checked.

Figure 6.11.2-1: High-level procedure flow of the solution

1.	The AF sends the authorization request to the CCF.

2.	The CCF/ Authorization Function checks whether the request requires resource owner permission. If the request requires resource owner permission and there is no available resource owner permission data, then the CCF/ Authorization Function decides to return ACK to the AF.

3.	The CCF/ Authorization Function return ACK to the AF about initiating the CIBA flow, so that the AF executes steps 5 and 6 regularly until the CCF/ Authorization Function obtains permission from the resource owner.

4.	The CCF/ Authorization Function obtains permission from the resource owner via ROF. Note that this is an offline step, i.e. the execution of steps 5-6 does not have any impact on this step.

5-6. The AF regularly sends the token request to the CCF/ Authorization Function until the CCF/ Authorization Function obtains the permission from the resource owner. The CCF/ Authorization Function sends the response which depends on whether the CCF/ Authorization Function has obtained the permission from the resource owner.

### 6.11.3	Evaluation

The solution addresses the use cases where the API invoker and resource owner are residing on different UEs. The CCF/ Authorization Function needs to support the CIBA mechanism, which has been designed especially for this use case. There is no impact on the AEF.

Steps 5-6 are executed multiple times between the API invoker and CCF/ Authorization Function. These steps are existing steps in the CIBA specification.

********* END OF CHANGES