# 3GPP TSG-SA3 Meeting #122																			S3-252308

# Fukuoka, Japan, 19 – 23 May 2025

Source:	Ericsson

Title:	Pseudo-CR on AIoT device security requirements related to cryptographic algorithms

Document for:	Approval

Agenda item:	4.26

Spec:	3GPP TS 33.369

Version:	0.1.0

Work Item:	AmbientIoT-SEC

Comments

As expressed in the SA3 work item description SP-250359, "[a]n Ambient IoT device has low complexity, small size and lower capabilities and lower power consumption than previously defined 3GPP IoT devices". More specifically, according to the RAN work item description RP-243326, Ambient IoT devices in scope of the Rel-19 work have ~1 µW peak power consumption.

The device requirements for cryptographic primitives have been studied in the research literature, for example in "Secure Communication with RFID tags in the Internet of Things" (Dominikus/Kraxberger, Security Comm. Networks 2014). Important metrics for classification are hardware area and power consumption. Since hardware area is among the important metrics, ideally an AIoT device should only need to support one cryptographic algorithm. This cryptographic algorithm needs to have low power consumption suitable to a low power device.

For interoperability and cost reasons, SA3 should agree on only one such algorithm to be supported on all AIoT devices according to TS 33.369. Traditionally, SA3 has required two sets of cryptographic algorithms to be supported on the UE, for crypto agility reasons. However, for AIoT devices it is important to provide cost efficient security.

This contribution suggests that AES could be a suitable candidate for this one cryptographic algorithm to be supported. AES is proven to be supportable on low power devices such as passive UHF RFID tags. In fact, for several commercial products there are public statements of support for AES. AES is a well-known, well-understood and widely supported algorithm in the mobile network ecosystem as well. Modes such as AES-CTR provide symmetric key encryption, and MAC constructions such as AES-CMAC provide a MAC. It is also possible to use AES for the derivation of session keys, if necessary, as e.g. used in MILENAGE. Hence, authentication, communication security and privacy mechanisms under discussion in TR 33.713 can be implemented using only AES as basic cryptographic tool. Therefore, this contribution suggests that AES shall be supported on the AIoT device, and no other cryptographic algorithms apart from potentially null algorithms. It is too early to specify exactly how the cryptographic primitive will be used, and most probably it will not be exactly like in the existing modes 128-NEA2 and 128-NIA2. Therefore, for the moment, it is proposed to only state that AES is to be supported, and not yet in which mode or construction.

As solutions for authentication, communication security and privacy are dependent on a freshness parameter, the AIoT device also needs to be capable of producing secure random bits. The secure random bits can be either generated in the device through a dedicated hardware random bit generator component, or pseudo-randomly by applying a deterministic random bit generator.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]		3GPP TS 23.369: "Architecture support for Ambient power-enabled Internet of Things".

[3]	3GPP TS 38.300: "NR; NR and NG-RAN Overall description; Stage-2".

[4]	3GPP TS 22.369 "Service requirements for Ambient power-enabled IoT".

[5]	3GPP TS 33.501 "Security architecture and procedures for 5G System".

* * * Next Change * * * *

### 4.2.1	Requirements on the device

Editor’s Note: This clause contains the security requirement on the device, including secure storage and processing of credentials.

#### 4.2.1.X Requirements related to authentication between device and network

The AIoT device shall support the following cryptographic primitives:

-	a secure method for random bit generation.

Editor’s Note: Security requirements on the method for random bit generation is FFS.

Editor’s Note: Further cryptographic primitives are FFS.

* * * End of Changes * * * *