3GPP TSG-SA3 Meeting #122													                                S3-252319

Fukuoka, Japan, 19 – 23 May 2025                                            Merger of S3-251846, S3-252116

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.529 | CR | 0004 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Solving issues to TS 33.529 according to review conclusion from GSMA NESASG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Indian Institute of Technology Bombay |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G_SMSF |  |  |  |  |  | Date: |  |  | 2025-05-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | GSMA NESASG pointed out issues to TS 33.529 SMSF SCAS in S3-251821, including missing threat references in Clauses 4.2.7.1, 4.2.7.2, 4.2.7.3, 4.2.7.4 and 4.2.7.5, along with an unresolved comment in Clause 4.2.7.4. This CR is to solve them. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Complement threat references and solve the comment. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not comply with the structural requirements for SCAS and cannot be adopted by GSMA NESASG. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.2.6, 4.2.7.1, 4.2.7.2, 4.2.7.3, 4.2.7.4, 4.2.7.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The clause number of the threat marked in yellow, as referred by 4.2.7.1 depends on the final clause number of S3-251844. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**************** START of CHANGES****************

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 33.117: "Catalogue of general security assurance requirements".

[3]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[4]	3GPP TR 33.926: "Security Assurance Specification (SCAS) threats and critical assets in 3GPP network product classes".

[5]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

[6]	3GPP TS 23.501: "System Architecture for the 5G System (5GS)".

[7]	3GPP TS 29.540: "5G System; SMS Services".

[8]	3GPP TS 29.338: "Diameter based protocols to support Short Message Service (SMS) capable Mobile Management Entities (MMEs)".

[9]	3GPP TS 29.002: "Mobile Application Part (MAP) specification".

[10]	3GPP TS 33.210: "Network Domain Security (NDS): IP network layer security".

[11]	3GPP TS 33.310: "Network Domain Security (NDS): Authentication Framework".

[12]	IETF RFC 6733: "Diameter Base Protocol".

[13]	3GPP TS 33.204: "3G Security; Network Domain Security (NDS): Transaction Capabilities Application Part (TCAP) user security".

[14]	Void.

**************** END of 1st CHANGE****************

**************** START of 2nd CHANGE****************

### 4.2.6	Network Devices

The clause 4.2.6 of TS 33.117 [2] applies to SMSF, except for the clauses 4.2.6.2.3 and 4.2.6.2.4, which are not applicable. There are no SMSF-specific additions to clause 4.2.6 of TS 33.117 [2].

**************** END of 2nd CHANGE****************

**************** START of 3rd CHANGE****************

4.2.7.1	Protection on SGd Diameter Interface between SMSF and the Diameter application node

Requirement Name: Protection data and information on SGd

Requirement Reference: TS 33.501 [3], clause 9.5, TS 33.210 [10], clause 6.2, TS 33.310 [11], clause 6.1.3a.

Requirement Description: TS 33.501 [3] mentions that protection of Diameter interface shall be supported according to NDS/IP as specified in TS 33.210 [10], unless security is provided by other means e.g. physical security. For authentication between SMSF and Diameter application node over diameter interface, mutual authentication based on client and server certificates is performed, if using TLS. Certificate based authentication follows the profiles given in TS 33.210 [10] clause 6.2, and TS 33.310 [11] clause 6.1.3a, with the restriction that it shall be compliant with the profile given by Diameter Base Protocol as defined in RFC 6733 [12], except the cipher suites. A SEG may be used to terminate the NDS/IP IPsec tunnels.

Threat References: TR 33.926 [4], clause x.z.2.1, SMSF control data and user data protection on SGd Diameter Interface

Test Case:

Test Name: TC_Protect_Diameter_SGd

Purpose: To verify the mechanisms implemented to protect data and information in transfer to and from the SMSF's Diameter protocol-based SGd interface.

NOTE:	This test case applies to the network function with SGd Diameter interface.

Preconditions:

Network product documentation containing information about supported NDS/IP protocols is provided by the vendor.

A Diameter application node peer implementing the security protocol configured by the vendor shall be available.

SMSF documentation, stating which security protocols for protection of data in transit are implemented and which profiles in TS 33.310 [11] and TS 33.210 [10] are applicable, is provided by the vendor. The tester shall base the tests on the profile defined by 3GPP in Clause 6.2 of TS 33.310 [11].

For TLS/DTLS, the tester shall base the tests on the profile defined by 3GPP in Clause 6.1.3a of TS 33.310 [11] and Clause 6.2 of TS 33.210 [10], with the restriction that it shall be compliant with the profile given by Diameter Base Protocol as defined in RFC 6733 [12], except the cipher suites.

For IKE and IPsec, the tester shall base the tests on the profile defined by 3GPP in TS 33.210 [10].

Procedure and execution steps, expected results, expected format of evidence:

These are the same as for the test case in TS 33.117 [2], clause 4.2.3.2.4, excluding execution step 4, and the profiles as mentioned in requirement description shall be followed in pre-conditions.

**************** END of 3rd CHANGE****************

**************** START of 4th CHANGE****************

4.2.7.2	Protection of Diameter Session on SGd Interface

Requirement Name: Diameter session on SGd interface

Requirement Reference: TS 29.338 [8], clause 4.5; RFC 6733 [12], clause 8.8

Requirement Description:

SMSF supports implicit termination of SGd Diameter application sessions. The client (server) includes in its requests (responses) the Auth-Session-State AVP set to the value NO_STATE_MAINTAINED (1), as described in IETF RFC 6733 [12]. The server sets the Auth-Session-State AVP value to NO_STATE_MAINTAINED (1), irrespective of what value the client sets. Neither the Authorization-Lifetime AVP nor the Session-Timeout AVP be present in requests or responses [8].

To protect Diameter sessions, SMSF supports the following requirements:

1.	Diameter session ID AVP be unique, i.e. uniquely identify the user session and distinguish the session from all other active sessions.

2. The session ID be generated by the Diameter application node that initiates the session.

Threat References: TR 33.926 [4], clause Y.2.2.2, Diameter session information disclosure

Test Case:

Test Name: TC_DIAMETER_SGd_SESSION

Purpose:

Verify that the above Diameter session and session ID related requirements have been met.

Procedure and execution steps:

Pre-Conditions:

-	This text case is applicable only if network product supports Diameter SGd Interface

-	The Diameter application node uses a session ID to identify a session between the node and its peer.

-	The documentation should describe the algorithm used to generate the session IDs, for details see Section 8.8 in RFC 6733 [12].

Execution Steps:

1)	The tester logs in the network product.

2)	The tester sends SGd application request message as follows:

a)	Auth-Session-State AVP is set to the value NO_STATE_MAINTAINED (1).

b)	Neither the Authorization-Lifetime AVP nor the Session-Timeout AVP is present in requests.

c)	The tester generates session ID as per the documentation and uses it as session ID AVP in the message.

3)	The tester sends SGd application request message as follows:

a)	Auth-Session-State AVP is set to the value STATE_MAINTAINED (0).

b)	Authorization-Lifetime AVP and Session-Timeout AVP are present in request.

c)	The tester generates session ID as per the documentation and uses it as session ID AVP in the message.

4)	The tester checks the response for its request message in step 2 and 3. In both cases, the tester verifies that:

a)	Auth-Session-State AVP is set to the value NO_STATE_MAINTAINED (1).

b)	Neither the Authorization-Lifetime AVP nor the Session-Timeout AVP is present in response.

c)	The session IDs in two request messages are different and unique.

5)	The tester sends SGd application request message with same session ID in a row and checks the response.

6)	The tester sends SGd application request message with different session ID in a row and checks the response.

7)	The tester logs in with different user ID and sends SGd application request message with same session ID in step 2 and checks the response.

Expected Results:

1)	A confirmation from the tester that the Auth-Session-State AVP is indeed set correctly in response messages.

2)	A confirmation from the tester that the neither the Authorization-Lifetime AVP nor the Session-Timeout AVP is present in response messages.

3)	In case of a duplicate or non-unique session ID, an error response is generated with Result-Code AVP as DIAMETER_INVALID_AVP_VALUE 5004 and Session-ID AVP is added in Failed AVP.

4)	A response message indicating success is received to the request message when session ID is unique.

Expected format of evidence:

A confirmation that the tester has confirmed that:

1.	The session ID AVP follows the requirements 1 and 2 in the requirement description.

2.	The correct Auth-Session-State, Authorization-Lifetime and Session-Timeout AVP configurations are used.

3.	The network product does not accept duplicate session IDs.

Test result (Passed or not)

**************** END of 4th CHANGE****************

**************** START of  5th CHANGE****************

4.2.7.3	Protecting availability and integrity on Diameter-based SGd interface

Requirement Name: Diameter filtering on the SGd interface

Requirement Reference: TS 29.338 [8], clause 6.3.2.2

Requirement Description:

TS 29.338 [8] defines the following commands and their command codes for the SGd application: MO-Forward-Short-Message-Request (OFR) - 8388645, MO-Forward-Short-Message-Answer (OFA) - 8388645, MT-Forward-Short-Message-Request (TFR) - 8388646, MT-Forward-Short-Message-Answer (TFA) - 8388646, Alert-Service-Centre-Request (ALR) - 8388648 and Alert-Service-Centre-Answer (ALA) - 8388648. It also mentions that the Application ID field for OFR, OFA, TFR, TFA commands allocated by IANA is 16777313 and that for ALR, ALA commands is 16777312. 

SMSF provides a mechanism, or rely on the other network function, to filter incoming Diameter messages on the SGd interface based on the Application IDs and command codes.

In particular, SMSF that supports filtering provides a mechanism:

1)	To filter incoming Diameter messages on the SGd interface at the Application layer of the stack ISO/OSI.

2)	To allow specified actions to be taken when a filter rule matches. In particular at least the following actions should be supported:

-	Discard: the matching message is discarded; no subsequent rules are applied and no answer is sent back.

-	Accept: the matching message is accepted.

-	Account: the matching message is accounted for i.e. a counter for the rule is incremented. This action can be combined with the previous ones. This feature is useful to monitor traffic before its blocking.

3)	To enable/disable the logging for each rule for troubleshooting.

4)	To filter on the basis of the value(s) of any portion of the protocol header.

5)	To reset the accounting.

6)	To disable/enable each defined rule.

Threat References: TR 33.926 [4], clause Y.2.2.1, Diameter filtering

Test case:

Test Name: TC_Diameter_SGd_FILTERING

Purpose:

To verify that the Network Product provides filtering for incoming Diameter messages on the SGd interface.

Procedure and execution steps:

Pre-Conditions:

-	This test case is applicable only if the network product supports Diameter SGd Interface and the embedded filtering capability

-	The tester has the privileges to configure Diameter filtering rules on the network product.

-	The vendor declares that Diameter filtering is enabled and provides a list of the filtering rules.

-	The vendor includes a guideline to configure the Diameter filtering in the documentation accompanying the network product.

-	A network traffic generator or a pcap file containing the Diameter messages is available.

-	A network traffic analyser on the network product (e.g. tcpdump) is available.

Execution Steps

1.	The tester logs in the network product.

2.	The tester configures the network product with the following rules:

a)	Accept messages that meet the filtering rules supported by the vendor on the SGd interface.

b)	Discard all other messages on the SGd interface.

c)	For each rule above the accounting is also enabled.

3.	The tester turns on the network traffic analyser on the SGd interface.

4.	The tester sends Diameter messages to the network product by replaying a pcap file or using a network generator, ensuring that the messages pass the supported filtering rules.

a)	Using the network analyser, the tester verifies that the messages are correctly received by the network product.

b)	Using the accounting, the tester verifies that the messages are not discarded because response messages are sent back by the network product.

5.	The tester sends Diameter messages to the network product by replaying a pcap file or using a network generator, ensuring the messages do not pass the supported filtering rules.

a)	Using the network analyser, the tester verifies that the messages are discarded by the network product.

b)	Using the accounting, the tester verifies that the messages are discarded and that no response is sent back by the network product.

Expected Results:

-	For step 4 the tester receives successful Diameter response messages from the network product.

-	For step 5 the tester receives no response from the network product.

-	For steps 4 and 5, messages that pass and do not pass the filtering rules are correctly accounted.

Expected format of evidence:

A testing report provided by the testing agency which will consist of the following information:

-	The used tool(s) name and version information

-	Settings and configurations used

-	Pcap trace

-	Screenshot

Test result (Passed or not)

**************** END of 5th CHANGE****************

**************** START of 6th CHANGE****************

4.2.7.4	Protecting from unknown peers on Diameter-based SGd interface

Requirement Name: Disabling peer discovery on the SGd interface

Requirement Reference: RFC 6733 [12], clause 5.2;

Requirement Description:

Capability Exchange Request (CER) and Capability Exchange Answer (CEA) are Diameter messages used on SGd interface to discover diameter nodes and know their capability. If peer discovery is enabled, an attacker can connect to the SMSF and learn about its capabilities by sending CER messages. Network products with the SGd interface should be configured to accept only known peers.

To protect SMSF from attacks by unknown peers (attackers), it supports the following requirements:

1)	It maintains list of all trusted nodes with static IP addresses in a peer table.

2)	It is configured only to respond to the CER messages from trusted nodes listed in the peer table, and it ignores CERs from all other nodes.

Threat References: TR 33.926 [4], Annex Y.2.2.1, Diameter filtering

Test case:

Test Name: TC_DIAMETER_SGd_PEERDISCOVERY

Purpose:

To verify that the SMSF ignores requests from any unauthorized nodes that tries to discover its capabilities.

Procedure and execution steps:

Pre-Conditions:

-	This test case is applicable only if the network product supports Diameter SGd Interface.

-	The tester has the privileges to configure network product's peer table to include only directly connected peers with static IP addresses.

-	The vendor declares that peer discovery is disabled on the network product.

Execution Steps

1)	The tester logs in the network product.

2)	The tester configures the peer tables of the network product with a list of static IP addresses.

3)	The tester configures its source IP using one from the list in the network product's peer table. The tester then sends a CER message by replaying a pcap file or using a network generator:

a)	Using the network analyser, the tester verifies that the message is correctly received by the network product.

b)	The tester verifies that the network product responds with a CEA.

4)	The tester configures its source IP to one that is not in the peer table list of the network product. The tester then sends a CER message by replaying a pcap file or using a network generator:

a)	Using the network analyser, the tester verifies that the message is discarded by the network product.

b)	The tester verifies that the network product does not respond with a CEA.

Expected Results:

-	The network product responds with a CEA for CERs received from the peers listed in its peer table and ignores CERs from all others.

Expected format of evidence:

A testing report provided by the testing agency which will consist of the following information:

-	The used tool(s) name and version information

-	Settings and configurations used

-	Pcap trace

-	Screenshot

Test result (Passed or not)

**************** END of 6th CHANGE****************

**************** START of  7th CHANGE****************

4.2.7.5	Protecting availability and integrity on Map-based SS7 interface

Requirement Name: Map filtering on the SS7 interface

Requirement Reference: TS 33.204 [13], clause 4.1; TS 29.002 [9], clause 12

Requirement Description:

TS 33.204 [13], clause 4.1 mentions "TCAPsec does not validate the TCAP user payload content. Message screening functions for particular message types may be needed on top of TCAPsec." TS 29.002 [9] defines the Short Message Service (SMS) management services in clause 12. 

Hence, message filtering mechanism shall be used on Map-based SS7 interface of SMSF to filter and allow only SMS management services related messages which are applicable for SMSF, for example, MAP-MT-FORWARD-SHORT-MESSAGE service, MAP-MO-FORWARD-SHORT-MESSAGE service, and MAP-ALERT-SERVICE-CENTRE service.

In particular, SMSF that supports filtering provides a mechanism:

1)	To filter incoming Map messages on the SS7 interface at the Application layer of the stack ISO/OSI.

2)	To allow specified actions to be taken when a filter rule matches. In particular at least the following actions should be supported:

-	Discard: the matching message is discarded; no subsequent rules are applied and no answer is sent back.

-	Accept: the matching message is accepted.

Threat References: TR 33.926 [4], clause Y.2.3.1, Map filtering

Test case:

Test Name: TC_Map_SS7_FILTERING

Purpose:

To verify that the Network Product provides filtering for incoming Map messages on the SS7 interface.

Procedure and execution steps:

Pre-Conditions:

-	This test case is applicable only if the network product supports Map/SS7 Interface and the embedded filtering capability.

-	The vendor declares that Map filtering is enabled and provides a list of the filtering rules.

-	The network product is preconfigured with the following rules:

a)	Accept messages that meet the filtering rules supported by the vendor on the SS7 interface.

b)	Discard all other messages on the SS7 interface.

- 	A network traffic generator or a pcap file containing the Map messages is available.

Execution Steps

1.	The tester turns on the network traffic analyser on the SS7 interface.

2.	The tester sends Map messages to the network product by replaying a pcap file or using a network generator, ensuring that the messages pass the supported filtering rules.

a)	Using the network analyser, the tester verifies that the messages are correctly received by the network product.

3.	The tester sends Map messages to the network product by replaying a pcap file or using a network generator, ensuring the messages do not pass the supported filtering rules.

a)	Using the network analyser, the tester verifies that the messages are discarded by the network product.

Expected Results:

-	For step 4 the tester receives successful Map response messages from the network product.

-	For step 5 the tester receives no response from the network product.

Expected format of evidence:

A testing report provided by the testing agency which will consist of the following information:

-	The used tool(s) name and version information

-	Settings and configurations used

-	Pcap trace

-	Screenshot

Test result (Passed or not)

**************** END of CHANGES****************