3GPP TSG-SA3 Meeting #123	S3-252509

Goteborg, Sweden, 25 – 29 August 2025

# Source:	GSMA

# Title:	CVD-2025-0101 – Four critical vulnerabilities in the access control mechanism of the 5G core Network

# Document for:	Information, Discussion

Agenda Item:               7

# Liaison Statement

| Liaison Statement Title: | CVD-2025-0101 – Four critical vulnerabilities in the access control mechanism of the 5G core Network |
| --- | --- |
| Security Classification: | Non-confidential |

| Source Meeting Information |  |  |
| --- | --- | --- |
| Meeting Number | Meeting Date | Meeting Location |
| GSMA CVD PoE |  | Conference Call |

| Document Details |  |  |
| --- | --- | --- |
| Document Number: | Creation Date: | Document Author: |
| CVD-2025-0101 | 20/05/25 | Vadim Paryev |
| Originating GSMA Source: | Deadline for response: | Liaison Statement Contact |
| GSMA CVD PoE | None | GSMALiaisons@gsma.com |

| Action Required by Recipient |  |
| --- | --- |
| External Recipients: | To: 3GPP CT1, 3GPP SA3 |
| Internal Recipients: |  |

Description

The GSMA Coordinated Vulnerability Disclosure Program Panel of Experts (CVD PoE) has recently reviewed a submission CVD-2025-0101 " From Control to Chaos: A Comprehensive Formal Analysis of 5G’s Access Control", from researchers at Pennsylvania State University.

One of the attacks in this submission named "A1: Coarse Scope Attack" exploits the fact that content of the producerSnssaiList attribute in the OAuth 2.0 access token granted by the NRF to the NF Consumer is not clearly enough defined in the 3GPP specifications, which can lead to incorrect implementations in the NRF. Specifically, instead of including only the authorized for the particular NF Consumer slices into this attribute, the NRF can mistakenly include all slices supported by the NF Producer. Consequently, a malicious NF Consumer can use this access token to access services of all slices served by the NF Producer, beyond those to which this NF Consumer is authorized to. This compromises the isolation between network slices, potentially leading to data leakage, service disruption, and unauthorized resource usage.

3GPP TS 29.510 Table 6.3.5.2.4-1 defines producerSnssaiList in the following way: "This IE may be included if the NRF supports providing list of S-NSSAIs of the NF service producer in the access token claims." From the above, it is not clear which S-NSSAIs should be included in this list - only the slices authorized for the NF Consumer, or all slices supported by the NF Producer. If all S-NSSAIs supported by NF Producer are included (because TS is interpreted by NRF vendor in that way), then the described attack is possible.

3GPP TS 33.501 §13.4.1.1.2 also somehow describes the handling of slicing authorization. However, the explicit statement that only authorized for the particular NF Consumer slices supported by the NF Producer should be included in the producerSnssaiList, is also not present there. Also, for case 1b (Access Token Request for specific NF Producer instance), list of S-NSSAIs (producerSnssaiList) is not even mentioned as possible claim in the access token. Whereas it is mentioned for case 1a (Access Token Request for specific NF Producer type): "The claims may include a list of S-NSSAIs or NSI IDs for the expected NF Service Producer instances."

Action  to 3GPP

CVD PoE kindly asks 3GPP to consider adding statements about producerSnssaiList conditions and content, for example:

- 3GPP TS 29.510: to add a note in the definition of producerSnssaiList in Table 6.3.5.2.4-1, indicating this attribute includes the slices requested and authorized during Access Token Request verification by NRF.

- 3GPP TS 33.501: §13.4.1.1.2 could be more explicit about the exact conditions and content of the list of S-NSSAIs (producerSnssaiList) in the access token, including case 1b.