3GPP TSG-SA3 Meeting #123	S3-252590

Goteborg, Sweden, 25 – 29 August 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | Draft-CR | rev | <Rev#> | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PRINS Refinement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CableLabs, Nokia, Charter Communications, China Telecom, Deutsche Telekom, Comcast Communications |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | DUMMY |  |  |  |  |  | Date: |  |  | 2025-07-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | GSMA NRG sent an LS (S3-252539) to 3GPP SA3 requesting further refinement of PRINS to make it easier for roaming intermediaries (RIs) to support PRINS and its end-to-end application layer security property. Those changes are to implement the refinements of PRINS requested by GSMA. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Protecting HTTP CONNECT requests and response by adding integrity protection of 3gpp-Connect-Req-info and 3gpp-Connect-Resp-Info headers. This allows an RI to cryptographically authenticate HTTP CONNECT requests and responses. 2. Allowing some parameters exchanged in N32-c procedures (e.g., protection policy) that are relevant to RIs to be sent over N32-f. Note that clause 13.2.2.3 of TS 33.501 already allows error messages to be sent over N32-f if they are relevant to RIs. 3. Adding HTTPS as an option for protecting the transport of N32-f, in addition to NDS/IP domain security and TLS VPN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some roaming intermediaries may not be willing to support PRINS due to security and operational concerns. Even if they support PRINS, some fraud (e.g., by misusing HTTP CONNECT) may happen in 5G roaming. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.9.3.2a, 13.1.2, 13.2.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * 1st Change * * * *

#### 5.9.3.2a	Support for Messages generated by Roaming Intermediaries

A PLMN SEPP that makes use of Roaming Intermediaries shall be able to handle error messages generated by Roaming Intermediaries, delivered over the N32 connection.

The following error messages relevant to Roaming Hub shall be supported,

-	'an IE is encrypted while it was expected to be available in the clear',

-	'an IE is not encrypted while its availability in the clear is not required',

-	'the N32 connection cannot be setup due to contractual reasons',

-	'the N32 connection cannot be setup due to a connectivity issue', and

-	'the message was not delivered due to contractual reasons'.

The mechanism used by the SEPP for setting up N32-c via a chain of Roaming Intermediaries shall contain sufficient information such that a) the Roaming Intermediaries can determine and authenticate the identities of the initiating PLMN and b) the initiating PLMN and the target PLMN can authenticate each other.

NOTE 1: The Roaming Intermediary can reject the N32-c connection if no roaming relation exists. In this case, the expected error is "the N32 connection cannot be setup due to contractual reasons".

Additionally, it shall be possible for the Roaming Hubs to generate application layer control plane messages in order to reject traffic. Application layer control plane messages may be generated by the Roaming Hubs in order to reject registration attempts (refer to TS 23.502 [8] clause 4.2.2.2), to terminate sessions (see TS 23.502 [8] clause 4.3.4.3) and/or deregister the UE (refer to TS 23.502 [8] clause 4.2.2.3.3) and shall be sent using the corresponding NF Service operation to the NF, when relevant decisions are enforced by the Roaming Hub.

In this case, such messages are transparent to the SEPP and the SEPP shall act on them as any other message on the N32-f interface not making use of Roaming Intermediaries. How the SEPP authorizes such messages is left to implementation.

* * * End of 1st Change * * * *

* * * 2nd Change * * * *

### 13.1.2	Protection between SEPPs

TLS shall be used for N32-c connections between the SEPPs.

If the N32-c TLS is established between the SEPPs over one or more Roaming Intermediaries using HTTP CONNECT, as specified in clause 5.5.2.1 of TS 29.573 [73], the 3gpp-Connect-Req-Info header in the HTTP CONNECT request message and the 3gpp-Connect-Res-Info header in the HTTP CONNECT response message shall support integrity protection and replay protection.

The SEPP shall maintain a set of trust anchors. For N32-c and N32-f when the selected security method is "TLS", each trust anchor consists of a list of trusted root certificates and a list of corresponding PLMN-IDs. For NDS/IP or TLS VPN used for N32-f when the selected security method is "PRINS", each trust anchor may also include a list of unique identifiers for a given Roaming Intermediary (RI). Any given PLMN-ID and any given RI identifier shall appear in at most one trust anchor.

NOTE 3: The PLMN-IDs in a given trust anchor for N32-c represent a particular PLMNThe PLMN-IDs in a given trust anchor for N32-f when the selected security method is PRINS represent the PLMNs that are reachable via a particular RI.

During N32-c connection setup, the SEPP shall map the PLMN-ID of the remote SEPP end entity certificate to the associated trust anchor for the purposes of certificate chain verification. Only the root certificates in the associated list shall be treated as trusted during certificate chain verification. If the remote SEPP certificate contains multiple PLMN-IDs that are mapped to different trust anchors, then that certificate shall be rejected.

Operator Group Roaming Hubs SEPPs are equivalent to a network operator SEPP when they are in the same security domain and are not considered Roaming Intermediaries as detailed in this clause. The communication between a group network operator's SBA network border element and the Operator Group Roaming Hub SEPP is out of scope of the present document.

If there are no Roaming Intermediaries between the SEPPs, TLS shall be used for N32-f connections between the SEPPs. Different TLS connections are used for N32-c and N32-f. If there are Roaming Intermediaries which only offer IP routing service between SEPPs, either TLS or PRINS (application layer security) shall be used for protection of N32-f connections between the SEPPs. PRINS is specified in clause 5.9.3 (requirements) and clause 13.2 (procedures).

If TLS is selected, the SEPP shall correlate the N32-f TLS connection with the N32-c connection. If the peer network is a PLMN, the SEPP compares the PLMN-IDs contained in the SEPP TLS certificates used to establish the N32-c and N32-f connections. Specifically, if the certificate used for N32-f contains one or more PLMN-IDs that are not contained in the TLS certificate used for the corresponding N32-c, the N32-f certificate shall be rejected. If the peer network is an SNPN, the SEPP compares the SNPN-ID contained in the SEPP TLS certificates used to establish the N32-c and N32-f connections.

The SEPP shall check whether the PLMN-IDs in the header and JSON fields, if any, of incoming N32-f messages, are abnormal by matching the PLMN-ID(s) in the relevant trust anchor or remote PLMN-ID(s) in the N32-f context. If TLS is used for N32-f, then the relevant trust anchor is the one selected during the setup of the correlated N32-c connection and, if PRINS is used, it is the trust anchor selected during the setup of the NDS/IP or TLS VPN. The SEPP should support a mode of logging where mismatches are logged.

If there are Roaming Intermediaries which, in addition to IP routing, offer other services that require modification or observation of the information and/or additions to the information sent between the SEPPs, PRINS shall be used for protection of N32-f connections between the SEPPs.

NOTE 1a:	The procedure specified in clause 13.5 for security mechanism selection between SEPPs allows SEPPs to negotiate which security mechanism to use for protecting NF service-related signalling over N32, and provides robustness and future-proofness, e.g. in case new algorithms are introduced in the future.

If PRINS is the selected security method for N32-f interface, one of the following additional transport protection methods shall be applied between SEPP and Roaming Intermediary for confidentiality and integrity protection:

-	NDS/IP as specified in TS 33.210 [3] and TS 33.310 [5], or

-	TLS VPN with mutual authentication following the profile given in clause 6.2 of TS 33.210 [3] and clause clause 6.1.3a of TS 33.310 [5]. The identities in the end entity certificates shall be used for authentication and policy checks, with the restriction that it shall be compliant with the profile given by HTTP/2 as defined in RFC 9113 [47].

-	HTTP over TLS (HTTPS) as defined in RFC 9110 [113] with mutual authentication.

During NDS/IP, TLS VPN, or HTTPS connection setup, the SEPP should map the RI identifier as extracted from its end entity (i.e., RI) certificate to the associated N32-f/PRINS trust anchor for the purposes of certificate chain verification. Only the root certificates in the associated list are treated as trusted during certificate chain verification. If the end entity certificate contains multiple RI identifiers that are mapped to different trust anchors, then that certificate should be rejected.

NOTE 1:	Void

NOTE 2:	Void.

NOTE 3: In HTTPS mutual authentication, how an RI authenticates PLMN SEPP or another RI is not specified in 3GPP.

NOTE 4: Whether or not a given RI identifier includes the PLMN-IDs that correspond to the roaming partners that are reachable via that RI, is not specified in 3GPP.

* * * End of 2nd Change * * * *

* * * 3rd Change * * * *

### 13.2.2	N32-c connection between SEPPs

#### 13.2.2.1	General

When the negotiated security mechanism to use over N32, according to the procedure in clause 13.5, is PRINS (described in clause 13.2), the SEPPs use the established TLS connection (henceforth referred to as N32-c connection) to negotiate the N32-f specific associated security configuration parameters required to enforce application layer security on HTTP messages exchanged between the SEPPs. A second N32-c connection is established by the receiving SEPP to enable it to not only receive but also send HTTP Requests.

The N32-c connection is used for the following purposes:

-	Key agreement: The SEPPs independently export keying material associated with the first N32-c connection between them and use it as the pre-shared key for generating the shared session key required.

-	Parameter exchange: The SEPPs exchange security related configuration parameters that they need to protect HTTP messages exchanged between the two Network Functions (NF) in their respective networks.

-	Error handling: The receiving SEPP sends an error signalling message to the peer SEPP when it detects an error on the N32-f interface.

If the exchanged parameters such as protection policy or security profile are relevant for the Roaming intermediaries, they shall be sent over N32-f to the peer SEPP.

The following security related configuration parameters may be exchanged between the two SEPPs:

a.	Modification policy. A modification policy, as specified in clause 13.2.3.4, indicates which IEs can be modified by a Roaming Intermediary (RI) of the sending SEPP.

b. 	Data-type encryption policy. A data-type encryption policy, as specified in 13.2.3.2, indicates which types of data will be encrypted by the sending SEPP.

c.	Cipher suites for confidentiality and integrity protection, when application layer security is used to protect HTTP messages between them.

d.	N32-f context ID. As specified in clause 13.2.2.4.1, N32-f context ID identifies the set of security related configuration parameters applicable to a protected message received from a SEPP in a different PLMN.

* * * End of 3rd Change * * * *