3GPP TSG-SA3 Meeting #123	S3-252606

# Goteborg, Sweden, 25 – 29 August 2025

Source:	ZTE

Title:	Resolving ENs for individual inventory option A

Document for:	Approval

Agenda item:	4.1.1

Spec:	3GPP TS 33.369

Version:	0.2.0

Work Item:	AmbientIoT-SEC

Comments

To protect privacy of AIoT device identifiers, temporary IDs are recommended during inventory and command messages. During last meeting, several options have been endorsed in S3-252326 with ENs left to be resolved.

So, this contribution is made to address the following ENs in individual inventory (clause 4.2 option A in S3-252326):

Editor’s Note: whether AIoTF or ADM computes T-ID is FFS.

Editor’s Note: in case AIoTF computes T-ID, a key KAIoTF derived from KAIoT in ADM is used. How AIOTF retrieves the KAIoTF is FFS.

Editor’s Note: in case the T-ID is computed by the ADM, whether the cryptographic key is the long-term key KAIoT or a key derived from KAIoT , and the impact of interaction between AIOTF and ADM and the analysis of load of ADM is FFS.

The above ENs are related to where the temp ID is calculated and corresponding procedures. Considering the initial ideas on AIOTF functions and the load of the ADM, we propose to derive and check the temporary ID in the AIOTF. In this way, the Kaiotf needs to be calculated in the ADM and transferred to the AIOTF.

Editor’s Note: whether and how to address attacks of an attacker broadcasting T-ID and Nonce triggering all AIoT Devices to constantly compute T’-D (e.g., energy depletion in the AIoT devices) is FFS.

Editor’s Note: how to address the attack that manipulates the RANDAIOT_N in the Paging message is FFS.

In the endorsed solution, Nouncenetwork is included in the paging message, which may cause replay attack and DoS attack. What's more, after receiving the paging message, the AIoT device shall first calculate the temporary ID and then do comparison. In this way, the temporary ID is calculated even if the AIoT device does not match. This has high requirements for power consumption, and is not applicable to the ultra-low power consumption of the AIoT device.

Therefore, we propose to modify the solution to pre-configure the initial temporary ID, so that the random number does not need to be carried in the paging message, also reducing the impact to paging message. In this way, a random number is needed from device to update the temp ID.

Editor’s Note: which input key (e.g., KAIoT or KAIoTF) to be used is FFS

Editor’s Note: which algorithm will be used is FFS

To solve the above ENs, we provide a method of using HMAC and Kaiotf.

* * * First Change * * * *

4.2	Procedure options for AIoT Device identifier protection during Indivdual inventory

The content below is from S3-252326 [1]. It includes two privacy options (option A & B) for the individual inventory.

It is FFS how to protect the AIoT Device ID when an individual AIoT Device is paged for an Individual Inventory that may or may not be followed by a command. The current situation is that there are two options (A and B) considered. These are captured below. More work is needed to reach consensus and complete the normative specification for this procedure.

Option A: Individual Inventory Procedure

This procedure details how the Device Identity is protected when the network performs an Individual inventory only and when the inventory phase is followed by a command.

The following high-level principles applies:

A temporary ID is generated and verified independently by the AIoT Device and network based on shared device credential and the RAND included in the paging message. The AIoT Device needs to store the temporary ID.

Figure:  Option A: Privacy protection of AIoT device ID in inventory with AIoT device ID

- The AIoT Device has a pre-configured initial Temp ID, the Network (the ADM) also have the initial Temp ID stored together with the device long term key, KAIOT in the AIoT Device profile.

1.	The AF initiates Inventory as specified in clause 6.2.2 of TS 23.369 [2]. The AIoT_Inventory request contains the AIoT device ID.

2. (optional) If there is no Temp ID in the AIOTF, the AIOTF sends AIoT_DeviceID Request to ADM to fetch the current Temp ID (i.e.,T-IDn) from the AIoT Device Profile in the ADM.

Base on local policy, AIOTF determines whether or not to fetch Kaiotf from ADM. If an AIOTF key is needed, an AIOT key indication is included in the message. Kaiotf is derived from Kaiot in the ADM as, Kaiotf = HMAC (TIDn), using Kaiot as the key.

- (optional) The ADM sends AIoT_DeviceID Response to the AIOTF.

- The AIoTF shall include the T-IDn in an Inventory Request and send the Inventory Request to the NG-RAN.

- 5.

The NG-RAN shall send a Paging message containing the T-IDn.

- 6. If an AIoT Device matches its identity to T-IDn. The AIoT device generates a new RANDd, constructs T-IDn+1, and upates T-IDn to T-IDn+1. T-IDn+1 is generated as specified in B.D.

Editor’s Note: how to handle if the AIoT device could not store the new Temp ID is FFS.

If the AIoT device does not match with the received T-IDn, the AIoT device shall stop further processing of the Inventory Request, i.e., the rest of the steps are not performed.

- The AIoT device shall include the T-IDn+1 and RANDd in a D2R message and send it to the NG-RAN.

- The NG-RAN shall send the Inventory Report containing the T-IDn+1 and RANDd to the AIoTF.

- Upon receiving the response, the AIOTF finds a matching T-IDn. If there is a match, the AIOTF constructs T-IDn+1, and upates T-IDn to T-IDn+1. T-IDn+1 is generated in the same way as AIoT device did in step 6.

- AIOTF sends an AIoT_T-ID Update Request to ADM. Both Device ID and T-IDn+1 are included in the message.

- After receiving the Temp ID update request, the ADM upates T-IDn to T-IDn+1in the AIoT Device profile.

- The ADM sends AIoT_T-ID Update Response to the AIOTF.

- The AIoTF shall send the AIoT_Inventory Notify to the AF as specified in clause 6.2.2 of TS 23.369 [2].

B.D	Temporary ID generation scheme

Temporary IDs are generated based on the following parameters:

-	AIoT Temp ID;

-	RANDd;

HMAC is used as the temp ID generation algorithm and Kaiotf is used as the key.

* * * End of Changes * * * *