3GPP TSG-SA3 Meeting #123	S3-252704

Goteborg, Sweden, 25 – 29 August 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.512 | CR | 0047 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of TC_SYNC_FAIL_SEAF_AMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BSI (DE), Montsecure |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5GA |  |  |  |  |  | Date: |  |  | 2025-07-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Test case C of TC_SYNC_FAIL_SEAF_AMF is ambiguous and does not indicate whether the Authentication Failure message should be sent with security protection enabled or not. Additionally, the expected results section of TC_SYNC_FAIL_SEAF_AMF is ambiguous and does not allow for automated tests as it is hard to detect whether an AMF does not respond or process a message. Furthermore, the specification states that the AUSF should initiate a new authentication procedure with the UE after receiving its response from the UDM instead of this being optional (applicable to test case A). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Split up test case C of TC_SYNC_FAIL_SEAF_AMF into two subtests of which one tests a security protected message and the other tests a plain message. Additionally, specific timeout values are added to the test case steps and expected results section of TC_SYNC_FAIL_SEAF_AMF such that this test case can be automated without having to check the behaviour of the AMF from the logs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The definition of TC_SYNC_FAIL_SEAF_AMF is loose and might cause different testers to execute the test differently (e.g. one tester execution the test with a security protected message while the other executes it with a plain message). This might lead to the test results being inconsistent depending on the implementation of the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 4.2.2.1.1	Synchronization failure handling

Requirement Name: Synchronization failure handling

Requirement Reference: TS 33.501 [2], clause 6.1.3.3.2

Requirement Description:  As specified in TS 33.501 [2] clause 6.1.3.3.2, upon receiving an authentication failure message with synchronisation failure (AUTS) from the UE, the SEAF sends an Nausf_UEAuthentication_Authenticate Request message with a synchronisation failure indication to the AUSF and the AUSF sends an Nudm_UEAuthentication_Get Request message to the UDM/ARPF, together with the following parameters:

-	RAND sent to the UE in the preceding Authentication Request, and

-	AUTS received by the SEAF in the response from the UE to that request, as described in clause 6.1.3.2.0 and 6.1.3.3.1 of TS 33.501 [2].

An SEAF will not react to unsolicited "synchronisation failure indication" messages from the UE.

The SEAF does not send new authentication requests to the UE before having received the response to its Nausf_UEAuthentication_Authenticate Request message with a "synchronisation failure indication" from the AUSF (or before it is timed out).

Threat References: TR 33.926 [6], clause K.2.2.1, Resynchronization

Test Case:

Test Name: TC_SYNC_FAIL_SEAF_AMF

Purpose:

Verify that synchronization failure is correctly handled by the SEAF/AMF.

Pre-Conditions:

-	Test environment with UE and AUSF. The UE and the AUSF may be simulated.

-	AMF network product is connected in emulated/real network environment.

Execution Steps

Test A:

1)	The tester triggers the UE to perform a registration procedure up until the point where the UE receives a NAS Authentication Request message from the network.

2)	The tester configures the UE to send an Authentication Failure message to the SEAF/AMF with synchronisation failure (AUTS), after receiving the NAS authentication request message.

3)	Immediately after sending the Authentication Failure message, the UE starts timer T3520 as specified in section 5.4.1.3.7 of TS 24.501.

4)	The SEAF/AMF sends a Nausf_UEAuthentication_Authenticate Request message with a "synchronisation failure indication" to the AUSF.

5)	The AUSF sends a Nausf_UEAuthentication_Authenticate Response message to the SEAF/AMF immediately after receiving the request from the SEAF/AMF, to make sure the SEAF/AMF will receive the response before timeout of T3520.

Test B:

1)	The tester triggers the UE to perform a registration procedure up until the point where the UE receives a NAS Authentication Request message from the network.

2)	The tester configures the UE to send an Authentication Failure message to the SEAF/AMF with synchronisation failure (AUTS), after receiving the NAS Authentication Request message.

3)	Immediately after sending the Authentication Failure message, the UE starts timer T3520 as specified in section 5.4.1.3.7 of TS 24.501.

4)	The SEAF/AMF sends a Nausf_UEAuthentication_Authenticate Request message with a "synchronisation failure indication" to the AUSF.

5) The tester configures the AUSF in a way, that it does not send a Nausf_UEAuthentication_Authenticate Response message to the SEAF/AMF before timeout of T3520.

Test C: (unsolicited synchronisation failure; not security protected)

1)	The tester triggers a UE to perform a Registration Procedure. The tester keeps track of the RAND value sent by the SEAF/AMF to the UE in the Authentication Request during the procedure.

2)	While the UE is registered, the tester sends an unsolicited Authentication Failure message with _synchronisation failure indication_ to the SEAF/AMF. The AUTS value in the synchronisation failure indication message is computed using the RAND value from step 1. The Authentication Failure message must not be security protected, e.g. it is sent in a plain 5GS NAS message.

3)	Immediately after sending the Authentication Failure message, the UE starts timer T3520 as specified in section 5.4.1.3.7 of TS 24.501.

Test D: (unsolicited synchronisation failure; security protected)

1)	The tester triggers a UE to perform a registration procedure. The tester keeps track of the RAND value sent by the SEAF/AMF to the UE in the Authentication Request during the procedure.

2)	While the UE is registered, the tester sends an unsolicited Authentication Failure message with _synchronisation failure indication_ to the SEAF/AMF. The AUTS value in the synchronisation failure indication message is computed using the RAND value from step 1. The Authentication Failure message must be security protected, e.g. it is sent in a security protected 5GS NAS message using the security context set up in step 1.

3)	Immediately after sending the Authentication Failure message, the UE starts timer T3520 as specified in section 5.4.1.3.7 of TS 24.501.

Expected Results:

Test A and Test B: Before receiving Nausf_UEAuthentication_Authenticate Response message from the AUSF and before the timer for receiving Nausf_UEAuthentication_Authenticate Response message runs out,

-	For Test A, the SEAF/AMF may initiate new authentication towards the UE before the timer T3520 runs out.

-	For Test B, the SEAF/AMF does not send any new authentication request to the UE while timer T3520 is running.

Test C and Test D: The UE does not receive an Authentication Request from the SEAF/AMF while timer T3520 is running. Additionally, the SEAF/AMF does not send a Nausf\_UEAuthentication\_Authenticate Request message with a "_synchronisation failure indication_" to the AUSF.

Expected format of evidence:

Evidence suitable for the interface, e.g., Screenshot, packet capture or application logs containing the operational results.