3GPP TSG-SA3 Meeting #123	S3-252814

Goteborg, Sweden, 25 – 29 August 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.210 | CR | 0088 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Mandating AEAD ciphersuites for TLS 1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | CryptoSP |  |  |  |  |  | Date: |  |  | 2025-08-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1. TS 33.210 clause 6.2.3 says “The rules on allowed cipher suites given in TLS 1.2 (RFC 5246 [50]) shall be followed.” 2. RFC 5246 clause 9 says “In the absence of an application profile standard specifying otherwise, a TLS-compliant application MUST implement the cipher suite TLS_RSA_WITH_AES_128_CBC_SHA (see Appendix A.5 for the definition)” 3. Furthermore, TS 33.210 v16.3.0 (and onwards) clause 6.2.3 says “Only cipher suites with AEAD (e.g. GCM) and PFS (i.e., ECDHE) shall be supported.” 4. Since AES_128_CBC is not an AEAD, the cipher suite TLS_RSA_WITH_AES_128_CBC_SHA is not allowed as per the bullet 3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For version v19.0.0 onwards, delete the line quoted in bullet 1 above, i.e., newer versions shall not implement the cipher suite TLS_RSA_WITH_AES_128_CBC_SHA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Note that the resolution for rel-19 is different from that for rel-16 (and rel-17 and rel-18), that's why this is not a mirror CR. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

## 6.2.3	Profiling for TLS 1.2

TLS 1.2 (RFC 5246 [50]) shall support the following restrictions and extensions:

TLS cipher suites

-	T

he following cipher suites are mandatory to support and recommended to use:

-	TLS_ECDHE_ECDSA_WITH_AES_128_GCM_SHA256 as defined in RFC 5289 [55]

-	TLS_ECDHE_RSA_WITH_AES_128_GCM_SHA256 as defined in RFC 5289 [55]

-	Support of the following cipher suites is recommended:

-	TLS_ECDHE_ECDSA_WITH_AES_256_GCM_SHA384 as defined in RFC 5289 [55]

-	TLS_ECDHE_RSA_WITH_AES_256_GCM_SHA384 as defined in RFC 5289 [55]

-	Only cipher suites with AEAD (e.g. GCM) and PFS (i.e., ECDHE) shall be supported.

Diffie-Hellman groups

-	For ECDHE, the curve secp256r1 (P-256) as defined in RFC 8422 [71] shall be supported, secp384r1 (P-384) as defined in RFC 8422 [71] should be supported.  Except x25519, elliptic curve groups of less than 256 bits shall not be supported.

-	Finite field Diffie-Hellman (i.e. DHE) shall not be supported.

TLS hash algorithms and signature algorithms

-	Hash algorithms: SHA-256 shall be supported. SHA-384 should be supported. MD5 and SHA-1 shall not be supported.

-	Signature algorithms: ecdsa, rsa_pss_rsae, and rsa_pkcs1 shall be supported. Usage of rsa_pkcs1 is not recommended.

-	ecdsa_secp384r1_sha384 should be supported.

TLS compression

-	The “null” compression method as specified in TLS 1.2 RFC 5246 [50] is mandatory to support. All other compression methods shall not be supported.

TLS extensions

-	If TLS Extensions are used in conjunction with TLS, then for RFC 6066 [57] shall apply.

-	The Server Name Indication (SNI) extension defined in RFC 6066 [57] shall be supported.

-	The Truncated HMAC extension, defined in RFC 6066 [57] shall not be supported.

-	TLS Session Resumption based on RFC 5246 [50] or RFC 5077 [59] should be supported.

-	TLS servers and TLS clients shall support RFC 5746 [60]. The server shall accept client-initiated renegotiation only if secured according to RFC 5746 [60].

- The Extended Master Secret extension, defined in RFC 7627 [61] shall be supported.

- Signature Algorithms, defined in RFC 5246 [50] shall be supported.

-	The Supported Groups extension, defined in RFC 8422 [71] and RFC 7919 [62] shall be supported.

-	The OCSP Status (a.k.a. certificate status request) extension, defined in RFC 6066 [57] should be supported.

PSK cipher suites

-	If pre-shared key (psk) cipher suites are implemented in TLS, then RFC 5489 [64] shall apply and the following cipher suites are mandatory to support and recommended to use:

-	TLS_ECDHE_PSK_WITH_AES_128_GCM_SHA256 as defined in RFC 8442 [51].

-	Support of the following cipher suite is recommended:

-	TLS_ECDHE_PSK_WITH_AES_256_GCM_SHA384 as defined in RFC 8442 [51].

* * * End of Changes * * *