3GPP TSG-SA3 Meeting #123	S3-252827

# Goteborg, Sweden, 25 – 29 August 2025

Source:	Ericsson

Title:	Pseudo-CR on Addressing EN in AIoT Authentication Procedure

Document for:	Approval

Agenda item:	4.1.1

Spec:	3GPP TS 33.369

Version:	0.2.0

Work Item:	AmbientIoT-SEC

Comments

This document proposes changes to the AIoT authentication procedure by addressing the existing Editor’s Notes.

Since the long-term key of a device is stored in the ADM, and there could be multiple AIOTFs, it makes more sense to generate the RANDAIoT_n in the ADM instead of AIOTF. Therefore, we propose to remove the following Editor’s Notes and make the related necessary changes in the procedure.

Editor’s Note: Whether ADM or AIOTF generates RANDAIOT_n is FFS.

For the sake of not involving ADM multiple times in the authentication procedure, we propose to use a session key Kint to derive RESAIOT. The session key can be derived on the fly, it does not need to be stored even. The device only needs to store the nonces (RANDAIOT_n, RANDAIOT_d). Therefore, we propose to remove the following Editor’s Note.

Editor’s Note: How RESAIOT is derived and whether it is derived from KAIoT or intermediate key is FFS.

Since we are using an intermediate key to compute RESAIOT, the ADM is involved only once in the authentication procedure. Therefore, we remove the following Editor’s Note:

Editor’s note: The impact of interaction between AIOTF and ADM is FFS. If the authentication is expected to be run more often than normal UE, (e.g., during each inventory procedure), the analysis of load of ADM is FFS.

This contribution also proposes to derive a session key Kenc for encryption of the command. If only inventory is performed without command, Kenc will not be used. However, the overhead for derivation of a second session key is small and deriving Kenc together with Kint simplifies the procedure considerably.

According to the modified procedure, adding steps 8, 9 and 10 is a good justification for removing following EN. This is because steps 8, 9 and 10 enable the device to authenticate the network and achieve mutual authentication.

Editor’s note: How to perform the mutual authentication for command procedure will be specified.

* * * First Change * * * *

## 5.2	Authentication procedure

Editor’s Note: This clause contains the security procedures on the authentication.

### 5.2.1	General

This clause describes the authentication procedure for Ambient IoT devices for both Inventory procedure and Command procedure when authentication is triggered by the network.

Editor’s Note: The alignment with the ID privacy procedure is FFS.

NOTE: KAIOT is the long-term key.

### 5.2.2	Authentication procedure

The authentication procedure is aligned with inventory procedure and command procedure in 6.2.2 and 6.2.3 of TS 23.369[2].

Figure 5.2.1-1: Authentication procedure

0. Step 1-6 of clause 6.2.2 Procedure for Inventory or clause 6.2.3 Procedure for command in TS 23.369 [2] is performed.

0.1 AIOTF sends Device ID to the ADM.

1. The ADM shall compute a key Kenc by a key derivation function using the long-term key KAIOT of the device and the freshly generated random RANDAIOT_n and a constant 0x01. The ADM shall also compute Kint by using a key derivation function using the long-term key KAIOT of the device and the freshly generated random RANDAIOT_n and a constant 0x02.

Editor’s note: How to derive key Kenc and Kint is FFS. For example, AES-CMAC / 128-NIA2 can be used as KDF as explained in Lily Chen, “Recommendation for Key Derivation Using Pseudorandom Functions”, NIST Special Publication 800 NIST SP 800-108r1-upd1.

1.1 The ADM shall send Kenc, Kint and RANDAIOT_n to the AIOTF.

2. AIOTF shall send inventory request message including RANDAIOT_n to NG-RAN.

Editor’s Note: The inclusion of RANDAIOT_n in Paging Request and the size of RANDAIOT_n needs RAN confirmation.

3. NG-RAN shall send the paging request message including RANDAIOT_n to the AIoT device.

Editor’s Note: Whether replay attack is possible is FFS.

4. Upon receiving the paging request message, AIoT device shall derive Kint in the same way the ADM generated Kint, and shall randomly generate a fresh RANDAIOT_d. The AIoT device shall also derive RESAIOT using Kint, RANDAIOT_n, and RANDAIOT_d for network authenticating AIoT Device. RESAIOT is derived by applying 128-NIA2 with KEY = Kint, COUNT = 0, MESSAGE = (inventory response || RANDAIOT_d ), DIRECTION = UPLINK, BEARER = 0.

Editor’s Note: Where the authentication credentials are processed in AIOT device is FFS.

5. AIoT device sends D2R message to the NG-RAN, including RESAIOT and RANDAIOT_d from device.

Editor’s Note: The security requirements of generating RANDAIOT_d are FFS.

Editor’s Note: Whether RANDAIOT_d is required for inventory procedure is FFS.

6. NG-RAN sends Inventory report message to AIOTF, including the RESAIOT and RANDAIOT_d.

7. AIOTF verifies RESAIOT using Kint, by computing XRESAIOT in the same way as the device computed RESAIOT, and comparing with the RESAIOT sent by the AIoT device.

Steps 8-10 apply if the inventory procedure is followed by a command:

8.  AIOTF shall send the command message encrypted using 128-NEA2, Kenc and RANDAIOT_d to NG-RAN. The AIOTF shall also send the MAC over the encrypted command message computed by using 128-NIA2, Kint and RANDAIOT_d.

Editor's Note: Detailed computation of encrypted and integrity protected command message using Kenc/Kint,  RANDAIOT_d and 128-NEA2/128-NIA2 are FFS.

Editor’s Note: Where the authentication credentials are processed in AIOT device is FFS.

9. NG-RAN shall send the parameters received from the AIOTF to AIoT device.

10. AIoT device verifies the command.

* * * End of Changes * * * *