3GPP TSG-SA3 Meeting #123	S3-252857

# Goteborg, Sweden, 25 – 29 August 2025

Source:	OPPO

Title:	Update requirements on AIOT privacy

Document for:	Approval

Agenda item:	4.1.1

Spec:	3GPP TS 33.369

Version:	0.2.0

Work Item:	AmbientIoT-SEC

Comments

It is proposed to update the requirement on AIOT privacy in clause 4.2.1.4 Requirements for identifier privacy, 4.2.2.3 Requirements on Privacy, and add a new subclause 4.2.3.x under the Requirements on the ADM.

For the group inventory, SA3 has identified the potential security threat using the filtering information if too many bits of the permanent device identifier are exposed. As specified in TS 23.369, the Offset is included when the indicated component is Identification Information which distinguishes different AIoT Devices. Therefore, it is concluded that the AIoT device should discard the inventory request message with filtering information containing its complete Identification Information. Additionally, to ensure sufficient anonymity, the number of devices identified by the filtering information should be sufficiently large. The requirements on the device and AIOTF are updated correspondingly.

Since the S3-252326 Living Document (Procedure for Protection of AIoT Device Identifier Privacy) presents two options for individual inventory ID privacy protection, the privacy-related requirements for the AIOTF and ADM remain unclear. Therefore, it is proposed to include a placeholder to capture the plenary's decision on ID privacy protection.

* * * First Change * * * *

#### 4.2.1.4	Requirements for identifier privacy

-	The device shall support a mechanism for the use of temporary IDs.

-	The device shall support resynchronization of desynchronized temporary IDs.

-	The device shall discard the inventory request message with the filtering information containing its complete Identification Information.

* * * Second Change * * * *

#### 4.2.2.3	Requirements on Privacy

- The AIOTF shall support the temporary ID based mechanism to protect the privacy of AIOT device identifier.

- The AIOTF shall ensure sufficient anonymity for the group inventory by guaranteeing that the number of devices identified by the filtering information should be sufficiently large.

* * * Third Change * * * *

#### 4.2.3.x	Privacy related requirement to ADM

- The ADM shall support the temporary ID based mechanism to protect the privacy of AIOT device identifier.

* * * End of Change * * * *