3GPP TSG-SA3 Meeting #123	S3-252865

# Goteborg, Sweden, 25 – 29 August 2025

Source:	Xiaomi

Title:	Clarification on the security for group inventory

Document for:	Approval

Agenda item:	4.1.1

Spec:	3GPP TS 33.369

Version:	0.2.0

Work Item:	AmbientIoT-SEC

Comments

As specified in TS 23.369, the inventory procedure includes the individual inventory procedure and group inventory procedure. For group inventory, the AIoTF can send the inventory request for paging a group of AIoT devices or all AIoT devices in the target area. Once receiving the inventory request, each device sends the AIOT NAS message (i.e. inventory response) to the AIOTF.

10.	NG-RAN sends one or more Inventory Report messages to the AIOTF including the Correlation ID, Reader ID and the AIOT NAS message(s) from the AIoT Device(s) as specified in TS 38.413 [10]. The NG-RAN may aggregate multiple Inventory Report messages based on the assistance information before reporting the response to the AIOTF as described in clause 5.9. The AIOTF stores the mapping between the Reader ID and AIoT Device ID(s).

If multiple AIOT NAS messages (i.e. inventory response) are received, how to perform the authentication by the network side is not clarified in TS 33.369. Therefore, it is proposed to add further clarification regarding the authentication in the group inventory case.

* * * First Change * * * *

5.2.2	Authentication procedure

The authentication procedure is aligned with inventory procedure and command procedure in 6.2.2 and 6.2.3 of TS 23.369[2].

Figure 5.2.1-1: Authentication procedure

0. Step 1-6 of clause 6.2.2 Procedure for Inventory or clause 6.2.3 Procedure for command in TS 23.369 [2] is performed.

1. ADM shall generate RANDAIOT_n.

Editor’s Note: Whether ADM or AIOTF generates RANDAIOT_n is FFS.

2. AIOTF shall send inventory request message including RANDAIOT_n to NG-RAN.

Editor’s Note: The inclusion of RANDAIOT_n in Paging Request and the size of RANDAIOT_n needs RAN confirmation.

3. NG-RAN shall send the paging request message including RANDAIOT_n to the AIoT device.

Editor’s Note: Whether replay attack is possible is FFS.

4. Upon receiving the paging request message, AIoT device shall derive RESAIOT using KAIoT and RANDAIOT_n for network authenticating AIoT Device.

Editor’s Note: How RESAIOT is derived and whether it is derived from KAIoT or intermediate key is FFS.

Editor’s Note: Where the authentication credentials are processed in AIOT device is FFS.

5. AIoT device sends D2R message to the NG-RAN, including RESAIOT and RANDAIOT_d from device.

Editor’s Note: The security requirements of generating RANDAIOT_d are FFS.

Editor’s Note: Whether RANDAIOT_d is required for inventory procedure is FFS.

6.  NG-RAN sends Inventory report message to AIOTF, including the aggregated AIoT Device NAS message(s). Each AIoT Device NAS message includes AIoT device identifier, the RESAIOT and RANDAIOT_d.

7. AIOTF sends one of multiple sets of AIoT device identifier and RANDAIOT_d to ADM.

Editor’s note: The impact of interaction between AIOTF and ADM is FFS. If the authentication is expected to be run more often than normal UE, (e.g., during each inventory procedure), the analysis of load of ADM is FFS.

8. For each received AIoT device identifier, ADM derives XRESAIOT using the same method as in AIoT device.

Editor’s Note: Where the authentication credential is processed in AIOT device is FFS.

9. ADM sends one or multiple sets of AIoT device identifier and XRESAIOT to AIOTF.

10. For each discovered AIoT device, AIOTF verifies RESAIOT. If the verification is successful, the steps 12-14 in clause 6.2.2 for inventory procedure or the step 8-11of clause 6.2.3 for command procedure in TS 23.369 [2] continues.

Editor’s note: How and where to derive keys is FFS.

Editor’s note: How to perform the mutual authentication for command procedure will be specified.

* * * End of Changes * * * *