3GPP TSG SA WG3 (Security) Meeting #79bis	S3-151625

Sophia Antipolis, France, 28 – 30 June 2015

Source:	China Mobile, Alcatel-Lucent

Title:	Deleting Editor’s note in B.4.2

Document for:	Discussion and Approval

Agenda Item:	4

Work Item / Release:	SCAS/Rel-13

This pCR proposes to resolve and remove the Editors note in section B.4.2.

- 1 Discussion:

Annex B.4.2 currently contains the following Editor’s Note:

Editor’s note: relates to clause 6.7/RX-2, 7.3, 7.4, 7.5 and Req.s 3.01-1 through 3.01-5 from DT’s catalogue.

From Annex C, we can find that Req.s 3.01-1 through 3.01-5 have been already addressed by B.4.2. The locations are as following:

the mark DTAG3186 for 3.01-1 is located in B4.2.1

the mark DTAG3191 for 3.01-2 is located in B4.2.2

the mark DTAG3197 for 3.01-3 is located in B4.2.3

the mark DTAG3200 for 3.01-4 is located in B4.2.4

the mark DTAG3202 for 3.01-5 is located in B4.2.5

After having studied the requirements in clause 6.7/RX-2, 7.3, 7.4, 7.5, it has been noted that these requirementscan be also addressed by clause B.4.2 and B.4.3.1. The detailed analyses are below:

- “6.7/RX-2: Test/debug ports should be disabled and physically removed from vendor, and they should not be left in place for servicing and there should be no local programming interface on the MME.”: It is addressed by B.4.2.1, because the test dubug ports and local programming interface are explicit examples about insecure protocols. This contribution proposes to revise the mark on the related section in the main body and the mark “151146 4” in the main body should be deleted if this proposal has been accepted.

- “7.3	No unnecessary or insecure ports/ services”: The first requirement in clause 7.3 has been coverd in clause B.4.2.1. The second requirement is to explain how to check open ports and running services, it can be addressed by test case. This contribution proposes to mark the second requirement to be  addressed by test case related B.4.2.1.

- “7.4 Services-Interfaces Binding”: It gives method about how to check running services and their binding to interfaces, it can be addressed by test case. This contribution proposes to mark the second requirement will be addressed by test case related B.4.2.2.

- “7.5	IP-Source address spoofing mitigation” : This has been addressed by B.4.3.1 and a mark in the main body of the TR 33.806 is proposed.

- 2 pCR

***	BEGIN FIRST CHANGES	***

6.7 		Security requirements on MME Management and Maintenance interfaces

NOTE: RX-1 of this requirement is covered by B.3.3.2

-	Requirement Name: MME Management and Maintenance interfaces

-	Requirement reference: to be done later

-	Requirement Description:

1) RX-1: The MME shall support mechanisms to provide confidentiality and integrity protection of the communication between itself and the OAM network.

2)  RX-2: Test/debug ports should be disabled and physically removed from vendor, and they should not be left in place for servicing and there should be no local programming interface on the MME.

Editor’s Note: Physically removing test/debug ports is not practical. RX-2 should be reworked.

3)	RX-3: The MME shall support mutual authentication between MME and specific management entities

Editor's Note: These management entities need to be further defined.

4)	RX-4: The secure communication mechanisms between the MME and it management entities shall use industry standard protocols and industry accepted algorithms and key lengths for encryption and data integrity.

-	Security Objective references: SECURE MME ADMINISTRATION

-	Threat References:  Threat of eavesdropping on MME management and maintenance interface data, Threat of man-in-the-middle attack on MME management and maintenance interface, Threat of modification of information in transit on MME management and maintenance interface

-	Test Case:

1) Test Case 1 for RX-1:

- Pre-conditions: The MME is powered on and the tester is able to capture traffic on MME management and maintenance interface and the tester can trigger communication between MME and OAM network. The tester has the ability to read the language of the traffic data.

- Steps taken to perform the test:

- The tester starts capturing traffic on MME management and maintenance interface.

- The tester triggers communication between MME and OAM network.

- The tester injects the traffic data.

- The tester checks the captured traffic data.

- Expected results:

The tester cannot get information through the raw traffic data or modify the raw traffic data.

3) Test case for RX-3:

- Pre-conditions: The MME is powered on, and has legal credential. The tester can trigger authentication between MME and specific management entities. Entity A has legal credential and entity B has illegal credential.

- Steps taken to perform the test:

- The tester check authentication mechanism configuration on MME, set one of options which shall support mutual authentication.

- The tester triggers communication between MME and entity A.

- Then, the tester triggers communication between MME and entity B.

- Expected results:

- Communication between MME and entity A established successfully.

- Communication between MME and entity B cannot be established.

- If illegal credential on MME, Communication cannot established either.

NOTE: This is a detailed requirement.

***	END OF THE FIRST CHANGES	***

***	BEGIN THE SECOND CHANGES	***

7.3	No unnecessary or insecure ports/ services

-	Requirement Name: removing unnecessary or insecure ports/ services

-	Requirement reference: to be done later

-	Requirement Description:

-	R1: MME shall only have ports opened and run services that are needed for MME operation

-	The open ports and running services shall be checked using BVT tools as described in clause 9 on BVT.

-	Threat References:

Editor’s Note: reference to a threat from clause 5.4 tba here.

-	Test Case:

-	Run appropriate BVT tools, as described in clause 9.

NOTE: This requirement can be mapped to the security objectives: SECURE MME ADMINISTRATION, HARDENING

7.4	Services-Interfaces Binding

-	Requirement Name: Service-Interface Binding.

-	Requirement reference: to be done later

-	Requirement Description:

The running services and their binding to interfaces shall be checked using BVT tools as described in clause 9 on BVT.

-	Threat References:

Editor’s Note: reference to a threat from clause 5.4 tba here.

-	Test Case:

Run appropriate BVT tools, as described in clause 9.

NOTE: This requirement can be mapped to the security objectives: SECURE MME ADMINISTRATION,HARDENING

## 7.5	IP-Source address spoofing mitigation

-	Requirement Name: IP-source address spoofing mitigation

-	Requirement Reference: TBD

-	Requirement Description:

Rx-1: protection mechanism against IP-source address spoofing attacks shall be enabled. For instance if LINUX is the running O.S. then enable the spoofing prevention built-in function by setting rp_filter to 1.

Editor's Note: it shall be clarified whether all the interfaces should comply with this requirement or whether only particular interface should be impacted.

-	Threat References: the unique identifiers assigned to the threats the requirement intends to meet.

-	Test Case:

Test case for Rx-1:

If LINUX is not the running O.S. then an alternative test case shall be written by the tester.

Test overview:

This test verifies if the spoofing prevention function is enabled, and performs its enabling if disabled

Precondition(s):

- User has sufficient administrative rights to activate spoofing prevention function

Postcondition(s):

- The spoofing prevention mechanism is enabled

Procedure and expected result

| Step | Direction | Action | Expected result |
| --- | --- | --- | --- |
| 1 | User → MME | open command line terminal (clt) | clt open |
| 2 | User → clt | cat /proc/sys/net/ipv4/conf/call/rp_filter | 1 |

***	END OF  THE SECOND CHANGES	***

***	BEGIN THE FINAL CHANGES	***

B.4.2 	Technical Baseline

B.4.2.1	No unnecessary or insecure services / protocols

Requirement Name: No unnecessary or insecure services / protocols

Requirement Reference: TBA

Requirement Description:

MME shall only run protocol handlers and services that are needed for MME operation, and that have no known security vulnerabilities.

Note: Full documentation of required protocols and services of the TOE and their purpose needs to be provided by the vendor as prerequisite for the test case.

Threat References: TBA

Security Objective References: TBA

Test Case: TBA

Editor's Note: It needs to be tested in addition that a correct configuration of protocols and services survives a system reboot.

***	END OF  THE CHANGES	***