3GPP TSG SA WG3 (Security) Meeting #79bis	S3-151679

Sophia Antipolis, France, 28 – 30 June 2015	revision of S3-151643

Source:	China Mobile, TNO

Title:	Mapping TR 33.806, B.5 to TS 33.116 and TS 33.117

Document for:	Discussion/approval

Agenda Item:	4

Work Item / Release:	SCAS / Rel-13

Abstract of the contribution:

“The technical content in S3-151679 differs from that in S3-151643 in that it takes into account S3-151641 and the revision thereof and S3-151612.”

This contribution maps TR 33.806 B.5 to TS 33.116 and TS 33.117. In the present contribution, only the requirements themselves are touched, not the test cases.

# Annotated text from TR 33.806

## B.5	Basic Vulnerability Testing (BVT) requirements

### B.5.1	Introduction

Basic Vulnerability Testing activities consist of requirements for running automated Free and Open Source Software (FOSS) and Commercial off-the-shelf (COTS) security testing tools against the external interfaces of a Network Product. These activities cover at least four aspects: Port Scanning, Vulnerability Scanner by the use of Vulnerability scanners and robustness/fuzz testing, and endpoint scanning. For each of these aspects, test requirements and test results are described in the present clause.

Example of commonly available COTS security testing tools includes but not limited to the following:

Nmap, Nessus, NSFOCUS RSAS, NGSSquirrel, secureCAT, Appsentry, Codenomicon, AppScan, AWVS, Symantec Endpoint, Officescan, Mcafee Endpoint Protection, Avira AntiVir, Coverity, Fortify, and Klocwork.

NOTE: 	The individual tools used for Basic Vulnerability Testing are selected by the evaluator. The SECAM accreditation body will ensure during accreditation of the evaluator’s laboratory that the testers are able to utilize adequate tools.

### B.5.2	Port Scanning

Requirement Reference: to be done later

Requirement Description: It shall be ensured that on all network interfaces, only documented ports on the transport layer respond to requests from outside the system

Threat References: Denial of Service, Information Disclosure

Security Objective references: PROTECTED COMMUNICATIONS, HARDENING.

Test case: TC_BVT_PORT_SCANNING

### B.5.3	Vulnerability Scanning

# Requirement Name: Vulnerability Scanning

Requirement Reference: to be done later

Requirement Description: The purpose of vulnerability scanning is to ensure that there no known vulnerabilities (or that  relevant vulnerabilities are identified and remediation plans in place to  mitigate them)  on the Network Product that can be detected by means of automatic testing tools via the Internet Protocol enabled network interfaces.

Vulnerability scanning tools may also report false positives and they shall be investigated and documented in the test report.

Editor’s note; the scope for Vulnerability Scanning (i.e. application only or both application and Operating System) shall be clarified.

Threat References: Denial of Service, Information Disclosure, Tampering

Security Objective references: PROTECTED COMMUNICATIONS, HARDENING.

Test case: TC_BVT_VULNERABILITY_SCANNING

### B.5.4	Robustness and fuzz testing

Requirement Name: Robustness and Fuzz testing

Requirement Reference: to be done later

Requirement Description: It shall be ensured that externally reachable services are reasonably robust when receiving unexpected input

Threat References: Denial of Service, Information Disclosure, Tampering

Security Objective references: PROTECTED COMMUNICATIONS, HARDENING.

Test case: TC_BVT_ROBUSTNESS AND FUZZ TESTING

# pCR to TS 33.117 (generic requirements)

***	START OF THE FIRST CHANGE	***

## 5.4	Basic vulnerability testing requirements

Editor’s Note: test cases and security objective references tba.

### 5.4.1	Introduction

Basic Vulnerability Testing activities consist of requirements for running automated Free and Open Source Software (FOSS) and Commercial off-the-shelf (COTS) security testing tools against the external interfaces of a Network Product. These activities cover at least four aspects: Port Scanning, Vulnerability Scanner by the use of Vulnerability scanners and robustness/fuzz testing, and endpoint scanning. For each of these aspects, test requirements and test results are described in the present clause.

Example of commonly available COTS security testing tools includes but not limited to the following:

Nmap, Nessus, NSFOCUS RSAS, NGSSquirrel, secureCAT, Appsentry, Codenomicon, AppScan, AWVS, Symantec Endpoint, Officescan, Mcafee Endpoint Protection, Avira AntiVir, Coverity, Fortify, and Klocwork.

NOTE: 	The individual tools used for Basic Vulnerability Testing are selected by the evaluator. The SECAM accreditation body will ensure during accreditation of the evaluator’s laboratory that the testers are able to utilize adequate tools.

### 5.4.2	Port Scanning

Requirement Name: Port scaning

Requirement Reference: TBA

Requirement Description:

It shall be ensured that on all network interfaces, only documented ports on the transport layer respond to requests from outside the system

Threat References: Denial of Service, Information Disclosure

Security Objective references: TBA

Test Case: TBA

### 5.4.3	Vulnerability Scanning

# Requirement Name: Vulnerability Scanning

Requirement Reference: TBA

Requirement Description:

The purpose of vulnerability scanning is to ensure that there no known vulnerabilities (or that  relevant vulnerabilities are identified and remediation plans in place to  mitigate them)  on the Network Product that can be detected by means of automatic testing tools via the Internet Protocol enabled network interfaces.

Vulnerability scanning tools may also report false positives and they shall be investigated and documented in the test report.

Editor’s note; the scope for Vulnerability Scanning (i.e. application only or both application and Operating System) shall be clarified.

Threat References: Denial of Service, Information Disclosure, Tampering

Security Objective references: TBA

Test case: TBA

### 5.4.4	Robustness and fuzz testing

Requirement Name: Robustness and fuzz testing

Requirement Reference: TBA

Requirement Description:

It shall be ensured that externally reachable services are reasonably robust when receiving unexpected input

Threat References: Denial of Service, Information Disclosure, Tampering

Security Objective references: TBA.

Test case: TBA

***	END OF THE FIRST CHANGE	***

***	START OF THE SECOND CHANGE	***

# pCR to TS 33.116 (MME-specific adaptations of generic requirements from TS 33.117)

## 5.4	MME-specific adaptations of basic vulnerability testing requirements and related test cases

All text from TS 33.117, 5.4 also applies to MMEs. There are no MME-specific adaptations or additions to this text.

***	END OF THE SECOND CHANGE	***