3GPP TSG SA WG3 (Security) Meeting #86	S3-170240

6-10 February 2017 Sophia Antipolis (France)	revision of S3-17abcd

Source:	Telecom Italia

Title:	Comments to “Questions and Interim Agreements on Key Issue #1.15: User plane security termination point, clause 5.1.3.15”

Document for:	Agreement

Agenda Item:	8.4.1

Work Item / Release:	Rel-14

Abstract of the contribution: This contribution comments the two questions and interim agreements for KI#1.15 proposed by S3-170089.

# 1. Introduction

In this document we treat the clause 5.1.3.15 and propose the related questions.

# 2. Analysis of question

The key issue addresses the need for:

-	One or more flexible UP security termination points

-	That may coincide with the UP-GW;

-	And is positioned ‘deeper’ in the network, such as beyond the gNB;

-	And can mitigate the threats resulting from NFV.

-	Different UP security termination points on a per slice basis;

-	Different UP security termination points, selectable according to traffic type (e.g. similar to APN concept);

-	Different UP security termination points for heterogeneous access to networks

This boils down to two different questions:

-	Where should the UP security terminate?

-	And, can the positions of one or more UP security termination points be flexibly selected by a suitable mechanism?

We propose to add the two questions and since there are no solutions addressing this problem we also propose an interim agreement.

# 3. Proposal for Agreement

## 3.1 Proposal for agreement for UP security termination

The first question addresses where  the UP security should terminate. One option would be to follow the LTE model and terminate the UP security in the eNB. For NextGen networks this is not desirable because:

-	Small cell deployments in less secure locations may be more prevalent

-	It is expected that many more parties control parts of the heterogeneous access network

-	Back-to-back termination and re-establishment of security for backhaul and UE in the base station is inefficient

-	It does not address use cases with different security requirements for different traffic flows per UE in an efficient way. For example: traffic with ultra-low latency requirements must be terminated in a location close to the UE, while other traffic from the same UE for other services needs protection through the whole backhaul.

So, the chance of having a compromised access node is increased compared to LTE. As such, it is desirable to terminate the UP security beyond the(R)AN. Also, we propose to introduce a so-called UP security termination fuction (UP-STF) that is tasked with terminating the UP security.

## 3.2 Proposal for agreement for flexibility of UP security termination

Based on the same comments raised within S3-170233, we propose the following changes to the text hereafter.

The second question addresses whether one or more different UP security termination points could be flexibly selected. Effectively, it asks whether one or more UP security termination points could be in different network locations depending on the slice or the service offered by the network. As such, we propose the following:

-	The UP-STF shall be co-located with the first UPF handling the UP traffic, since, according to TS 23.501:

- There may be multiple UPFs in the 5G System, depending on the service/business scenario offered.

- The UPF shall support the Uplink (UL) Classifier (CL) functionality to  filter UP traffic and that can work only on clear-text traffic.

This keeps the architecture simple and aligned with the System Architecture for the 5G System defined in TS 23.501.

# 4. Proposal

**** Beginning of change ****

## X.1 Questions and Agreements for security area #1

[…]

### X.1.#KI15# Questions and Agreements for Key Issue #1.15

#### X.1.#KI15#.0 Questions in other clauses affecting this key issue

The questions with respect to the selection of a UP confidentiality / integrity protection are affected by this clause as well, the questions on KI1.9 and KI1.10 .

#### X.1.#KI15#.x Location of UP security termination point

##### X.1.#KI15#.x.1 Description of Question

This question addresses where the UP security should be terminated, i.e. whether it should be in the (R)AN, in the CN, on the border between the two or anywhere else.

##### X.1.#KI15#.x.2 Interim Agreement

The following potential security requirements are agreed:

-	The UP security should be terminated beyond the  (R)AN ;

The following is agreed for normative work:

-	The UP security terminates in the UP security termination function (UP-STF)

#### X.1.#KI15#.y Flexibility of UP security termination points

##### X.1.#KI15#.y.1 Description of Question

This question addresses whether it should be possible to flexibly select one or more UP security termination points, depending on e.g. service requirements.

##### X.1.#KI15#.y.2 Interim Agreement

The following is agreed for normative work:

-	The UP security termination function (UP-STF) is co- located with the UPF defined in the TS 23.501.

**** End of Change ****