3GPP TSG SA WG3 (Security) Meeting #86                                                   S3-170350

6-10 Feb 2017, Sophia Antipolis, France

Source:	China Mobile/Nokia

Title:	pCR The evaluation of a new version of a 3GPP network product

Document for:	Discussion/approval

Agenda Item:	8.1

Work Item / Release:	SCAS-SA3TR/ Rel-14

Abstract of the contribution: This pCR identifies two ways to get third-party evaluation of the new version of the network product, when the vendor upgrade the network product that previous version has gain the evaluation.

Introduction

This contribution identifies two ways to get third-party evaluation of the new version of the network product, when the vendor upgrade the network product that previous version has gain the evaluation.

Proposed pCR

***	BEGIN OF THE FIRST CHANGE	***

7.6 The evaluation of a new version

After a network product completes a SECAM evaluation by a test laboratory, the vendor may upgrade the network product e.g. as a result of modified features, remediation of vulnerabilities etc. A re- evaluation of the new version of the network product can be carried out in two ways as described below:

- evaluation as per a new product by using all the test cases as defined in the SCAS’s;

- the vendor provides a detailed list of the updates to the product and provides a list of related test cases that will verify the new version of the product. Once the vendor and the test laboratory agree on the updated test plan, the evaluation takes place.

- On completion of the evaluation the test laboratory will include in the evaluation report:,

- the list of updates to the network product.;

- Test cases that were executed in the evaluation.

- The test results from the evaluation.

Operators can then decide on accepting a compliance statement for the updated version of a network product.

***	END OF THE FIRST CHANGES	***