# 3GPP TSG SA WG3 (Security) Meeting #91-Bis	S3-181801

# 21 – 25 May, 2018, La Jolla, US

# Source:	China Mobile

# Title:	Evaluation of Integrity protection of NSST

# Document for:	Approval

# Agenda Item:	8.3 - FS_ NETSLICE-MGT_Sec

1	Decision/action requested

SA3 is kindly requested to accept the proposed evaluation as in section 6 into TR33.811 v0.5.0.

2	References

[1]			3GPP TR 33.811 v 0.5.0: “Study on security aspect of 5G network management slicing”.

3	Rationale

Add evaluation for the TR 33.811 [1] solution #3.1 that was proposed in SA3#91.

4	Detailed proposal

*************** Start of Change 1 ****************

6.3			Solution #3.1: Integrity protection of NSST

This solution assumes that an NSST will be created using a structured format (e.g XML). The integrity protection is provided using digital signatures during transmission, on-boarding and storage of the NSSTs.

6.3.1	Introduction

This solution addresses the integrity protection requirements in KI#3

6.3.2	Solution details

This solution describes the procedures used to integrity protect an NSST during the life-cycle of an NSST.

The steps of the procedure are described as follows:

- During the setup of the network design environment a key-pair will be on-boarded. The private key of the key-pair will be used to create a digital signature on the NSST. The private key should be stored in tamper-resistant storage.

- The public key of the key-pair referred to in step 1 will be on-boarded into the NSSMF. This will be used to verify the data digital signatures on the NSSTs created by the network design app. The public key should be stored in tamper-resistant storage.

- When an NSST is created during the network slice subnet preparation phase it will be digitally signed by the network design app using the private key on-boarded in step 1.

- The digitally signed NSST is on-boarded to the NSSMF.

- The NSSMF verifies the digital signature on the NSST using the public key on-boarded in step 2. If the verification of the digital signature fails, the NSSMF will reject the NSST and it will not be stored in the NSST store.

- The digitally signed NSST is stored in the NSST store.

- The NSSMF receives an order for an NSSI in the network slice subnet creation phase.

- The NSSMF fetches the NSST from the NSST store.

- The NSSMF verifies the digital signature on the NSST using the public key on-boarded in step 2. If the digital signature cannot be verified, the NSSMF must reject using the NSST as a basis for the creation of an NSSI.

6.3.3	Evaluation

This solution addresses key issue of #3. Protecting Network Slice Subnet Template.

The solution has the following advantages:

-	It can protect the integrity of he network slice subnet template during transmission and in storage in the stage of preparation and creation.

-	The signature algorithm using asymmetric key pairs for integrity protection is mature and widely used.

-	It allows the management systems to verify the source of the network slice subnet template which is resistant to forgery and helps for responsibility tracking.

*************** END of Change 1 ****************