3GPP TSG SA WG3 (Security) Meeting #91	S3-181958

21 - 25 May 2018; LaJolla CA  		revision of S3-181918

Source:	TIM, InterDigital

Title:	New solution for Security of Service Based Architecture

Document for:	Approval

Agenda Item:	7.2.13.2 - Other

Work Item / Release:	5GS_Ph1-SEC

This contribution provides a new solution for the Living Document, Security of Service Based Architecture of 5G phase 1.

# 1	Decision/action requested

Approval to make the text changes indicated below

# 2	Rationale

This contribution provides a new solution for the KI #1 of the Living Document, Security of Service Based Architecture of 5G phase 1. It deals with the same problem as Solution #2 from the Living Document, but provides a more efficient method for end-to-end integrity protection with traceability and more effective methods for handling the write access authorizations for modifying the information elements and read access authorizations for reading the information elements in a signaling message.

# 3	Detailed proposal

Make the following changes to [1] (note: all text is new. Track changes highlight the concrete changes performed against the original proposal S3-181918):

***** Start of change 1 *****

### 4.3.x Solution #x: End-to-end data protection in hop-by-hop network communication links

Editor’s Note:

•	Entities generating and distributing the public/private keys mentioned in this solution need to be clarified;

### •	For the proposed solution Call Flows need to be added .

#### 4.3.x.1	General

Consider a hop-by-hop network communication link, e.g., in a 5G or 4G IPX network, whose nodes correspond to IPX entities. In particular, such a link corresponds to N32 interface in 5G. Assume that data is arranged in a signaling message as a sequence of information elements (IEs), e.g., as a sequence of AVPs in the 4G Diameter protocol. For example, IEs can be implemented as JSON elements.

Signaling messages go from source to destination via specified intermediate peers which can be authorized to read or modify (change or delete) the IEs or can add the new ones. The communication links thus have an intrinsic hop-by-hop nature and, as such, can be protected in the hop-by-hop manner (e.g., by TLS tunnels over http in 5G or by IPSec tunnels in 4G). However, such hop-by-hop protection does not ensure end-to-end integrity protection with non-repudiation and traceability of changes. Neither does it ensure that only authorized nodes should perform changes in a signaling message. Neither does it ensure that only authorized nodes should have read access to sensitive IEs.

#### 4.3.x.2	Integrity protection with non-repudiation and traceability of changes

The solution described in this clause ensures end-to-end integrity protection with non-repudiation by using hash functions and digital signatures. Each node receives only the last signaling message meant to be received by that node, after all the changes performed by previous nodes along the link, along with some auxiliary information.

The signaling message received by any node along the link is verified as authentic if and only if all included digital signatures are verified as valid. In that case, the receiving node also learns and verifies as authentic all the change operations performed by previous nodes in the respective received signaling messages. It also verifies as authentic any information about the nodes (e.g., their identity attributes) associated with the respective digital signatures. Non-repudiation is ensured by digital signatures, with respect to this associated information. Digital signatures are performed only by the nodes adding or modifying the IEs in a signaling message.

The solution is defined as follows:

- If a node adds a new IE to the sequence, then it associates to it an index that is different from the indexes of other IEs in the sequence, before sending the new IE to the next node. In particular, this relates to the source node.

- If a node modifies a received IE, by changing or deleting its value, then it associates to the modifed IE a hash0 value of its original value, without modifying its index, before sending it to the next node. Here, the hash0 function must be collision-resistant, e.g., a cryptographic hash function, or, if IE is very short, an identity function, which is not one-way.

- If a node does not modify a received IE, then it forwards it to the next node in the same form.

- If a node neither adds new IEs nor modifes the existing IEs in a received signaling message, then it forwards the received signaling message as a whole to the next node.

- If a node adds or modifies at least one IE in a signaling message, then it computes a hash value of the concatenation of the hash0 values of all added or modified IEs including their indexes. Then, it computes a digital signature on the resulting hash value, by applying the respective private key, and adds a new IE containing the digital signature together with the indexes of the added or modified IEs. Here, the hash function must be a cryptographic hash function, which is both collision-resistant and one-way.

- Each computed digital signature should include anti-replay protection mechanisms (e.g., based on nonces).

- Upon receiving a signaling message, each receiving node verifies all the digital signatures included in the signaling message, by iteratively exploiting the associated hash0 values of the original values of modified IEs and by applying the respective public keys for verification.

- The method can be applied to all or to only selected IEs in a signaling message, where the selection should be performed by the nodes adding new IEs.

#### 4.3.x.3	Integrity protection with non-repudiation, traceability of changes, and authorization

In the solution described in clause 4.3.x.2, each receiving node can locally store the authorizations of all previous nodes for performing the changes in a signaling message and can then verify the consistency of the traced operations by comparing them with the stored authorizations. However, the local storage and update of authorizations can be impractical, especially if nodes belong to different domains. If classical digital signatures are used, then an inter-operator public-key infrastructure (PKI) is required, which may be impractical.

A more effective and efficient method, using attribute-based cryptosystems is described in the following:

- Authorization rights of a node for performing the changes in a signaling message are expressed by an access policy in terms of the node attributes (e.g., their identity or domain attributes).

- Such an access policy is (dynamically) embedded in a digital signature of a node by using attribute-based signatures (ABS) or indentity-based signatures (IBS) [ref3]. The node attributes are embedded in the node private key for signing.

- In ABS, there is a common public key for signature verification and a multiplicity of private signing keys.

- An ABS signature can be verified as valid if and only if the embedded node attributes satisfy the embedded access policy and the signed information is authentic.

- Such write authorization rights are then verified by verifying a digital signature and by checking if the access policy embedded in the digital signature is compliant with the write access policy associated with an IE (e.g., as an integral part of IE value).

#### 4.3.x.4	Confidentiality protection with authorization

Confidentiality of sensitive IEs can be protected by using encryption. The objective is that the source node, or any intermediate node adding new sensitive IEs to a signaling message, should encrypt these IEs in such a way that only the further nodes along the link that are authorized to read these IEs (including the destination node) are in possession of the respective private decryption key. Classical solutions are not satisfactory due to impractical key management.

A more effective and efficient method, using attribute-based cryptosystems is described in the following:

- Confidentiality of selected IEs with authorized access to decryption keys is achieved by applying attribute-based encryption (ABE) or identity-based encryption (IBE), where the relevant read access policy is (dynamically) embedded in ciphertext and the node attributes in its decryption key (ciphertext-policy ABE –  CP-ABE) [ref2]. Preferably, both CP-ABE and ABS should use the same public and private keys (ABES) [ref3]. Alternatively, the relevant node attributes is embedded in ciphertext and the access policy in the node decryption key (key-policy ABE – KP-ABE) [ref1].

- In ABE, there is a common public key used for encryption and a multiplicity of private decryption keys.

- In ABE, the decryption can work if and only if the embedded node attributes satisfy the embedded access policy. This means that the read authorization rights are thus ensured automatically.

- ABE should be used for establishing a common shared key for a symmetric-key encryption/decryption. The same key is automatically shared by the encryption node all authorized decryption nodes. Moreover, this key can be used as static, together with a key-derivation function in order to generate dynamic session keys for encryption.

- The integrity protection of modified and re-encrypted IEs should be performed on ciphertexts, in order to enable for the nodes that are not authorized to decrypt/read encrypted IEs to verify the integrity of these IEs.

***** End of change 1 *****

***** Start of change 2 *****

# 2	References

[1]	3GPP TS 33.501 Security Architecture and Procedures for 5G System

[2]	S3-172533, Security Considerations for Service Based Architecture in 5G

[3]	S3-172539, Authorization of NF service access

[4]	C4-174343 LS on Conclusion on Service Based Architecture Protocol Selection

[5]	3GPP TS 23.502 v1.1.0 Procedures for the 5G System

[6]	S3-173153 Adding KI of Service authorization to living doc

[7]	S3-173486 Security for Service Based Interconnect interfaces

[8]	S3-173490 Application layer security based on JOSE framework

[9]	3GPP TS 23.501 v1.5.0 Procedures for the 5G System

[10]	JSON Object Signing and Encryption - https://datatracker.ietf.org/wg/jose/charter/

[11]	JSON Web Signature - https://tools.ietf.org/html/rfc7515

[12]	JSON Web Encryption - https://tools.ietf.org/html/rfc7516

[13]	JSON Web Algorithms - https://tools.ietf.org/html/rfc7518

[14]	S3-180353 SBA: Prioritization for Phase 1

[15]	S3-180354 SBA: Resolve EN on Information Elements that require protection

[16]	S3-180356 SBA: Resolve EN on use of JOSE with multiple HTTP Sessions between two NFs.

[17]	S3-180358 NF Service Register Request Procedure

[18]	S3-180155 Authorization of NF service access

[19]	S3-180177 Merge two procedures of SBA authorization

[20]	S3-180363 NF Service Request Procedure

[21]		CVE-2017-12119 - https://cve.mitre.org/cgi-bin/cvename.cgi?name=CVE-2017-12119

[22]		CVE-2016-4303 - https://cve.mitre.org/cgi-bin/cvename.cgi?name=CVE-2016-4303

[23]		CVE-2016-4425 - https://cve.mitre.org/cgi-bin/cvename.cgi?name=CVE-2016-4425

[24]		CVE-2017-12635 - https://cve.mitre.org/cgi-bin/cvename.cgi?name=CVE-2017-12635

[ref1]	V. Goyal, O. Pandey, Amit Sahai, and B. Waters, "Attribute-based encryption for fine-grained access control of encrypted data," in Proc. CCS '06, New York, 2006, pp. 89-98.

[ref2]	J. Bethencourt, A. Sahai, and B. Waters, "Ciphertext-Policy Attribute-Based Encryption," in Proc. SP '07, 2007, pp. 321-334.

[ref3]	C. Chen, J. Chen, H.-W. Lim, Z. Zhang, D. Feng, “Combined Public-Key Schemes: The Case of ABE and ABS,” in Proc. Provable Security '12, Lecture Notes in Computer Science, vol. 7496, 2012, pp. 53-69.

[ref4]		ETSI TS 103 532 v1.1.1, Attribute Based Encryption for Attribute Based Access Control (2018-03).

***** End of change 2 *****

# 4	References

[1]	S3-181474, Living Document: Security of Service Based Architecture of 5G phase 1