# 3GPP TSG SA WG3 (Security) Meeting #91-Bis	S3-182029

# 21 – 25 May, 2018, La Jolla, US	merger of S3-181671 and S3-181801

# Source:	China Mobile, Huawei, Hisilicon

# Title:	Evaluation of Integrity protection of NSST

# Document for:	Approval

# Agenda Item:	8.3 - FS_ NETSLICE-MGT_Sec

1	Decision/action requested

SA3 is kindly requested to accept the proposed evaluation as in section 6 into TR33.811 v0.5.0.

2	References

[1]			3GPP TR 33.811 v 0.5.0: “Study on security aspect of 5G network management slicing”.

[2]			TS28.530: "Management of network slicing in mobile networks; Concepts, use cases and requirements".

3	Rationale

Add evaluation for the TR 33.811 [1] solution #3.1 that was proposed in SA3#91. It also makes a few editorial corrections.

4	Detailed proposal

*************** Start of Change 1 ****************

6.3			Solution #3.1: Integrity protection of NSST

This solution assumes that an NSST will be created using a structured format (e.g XML). The integrity protection is provided using digital signatures during transmission, on-boarding and storage of the NSSTs.

6.3.1	Introduction

This solution addresses the integrity protection requirements in KI#3

6.3.2	Solution details

This solution describes the procedures used to integrity protect an NSST during the life-cycle of an NSST.

Figure 6.3.2-1: Procedure for integrity protection of NSST

The steps of the procedure are described as follows:

- During the setup of the network design environment a key-pair is on-boarded. The private key of the key-pair is used to create a digital signature on the NSST. The private key should be stored in tamper-resistant storage.

- The public key of the key-pair referred to in step 1 is on-boarded into the NSSMF. This is used to verify the data digital signatures on the NSSTs created by the network design app. The public key should be stored in tamper-resistant storage.

- When an NSST is created during the network slice subnet preparation phase it will be digitally signed by the network design app using the private key on-boarded in step 1.

- The digitally signed NSST is on-boarded to the NSSMF.

- The NSSMF verifies the digital signature on the NSST using the public key on-boarded in step 2. If the verification of the digital signature fails, the NSSMF shall reject the NSST and it is not  stored in the NSST store.

- The digitally signed NSST is stored in the NSST store.

- The NSSMF receives an order for an NSSI in the network slice subnet creation phase.

- The NSSMF fetches the NSST from the NSST store.

- The NSSMF verifies the digital signature on the NSST using the public key on-boarded in step 2. If the digital signature cannot be verified, the NSSMF shall reject using the NSST as a basis for the creation of an NSSI.

6.3.3	Evaluation

This solution describes a procedure that integrity protects an NSST through its life cycle that addresses the security requirements (for integrity) in key issue #3. Protecting Network Slice Subnet Template.

The solution has the following advantages:

-	It can protect the integrity of the network slice subnet template during transmission, in storage, in the stage of preparation and creation.

-	The solution does not specify which data signature algorithm to use, but a recommendation is to use ECDSA with at least a 256 bit key size. The signature algorithm using asymmetric key pairs for integrity protection is mature and widely used.

-	It allows the management systems to verify the source of the network slice subnet template which is resistant to forgery and helps for responsibility tracking.

*************** END of Change 1 ****************