3GPP TSG-SA3-LI Meeting #51 	SA3LI13_117r2

# Burlington, VT., USA, 22-24, Oct 2013

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.107 | CR | CRNum | rev | - | Current version: | 12.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 107 WLAN IRI Packet Header Information Reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (FBI, Ministère du Redressement Productif - France) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI12 |  |  |  |  |  |  | Date: |  | 2013-10-22 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | To provide WLAN IRI Packet Header Information reporting to support national requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | References added, new events defined to report per packet and summary reports. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Regulatory implications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 9.3, New 9.4.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** FIRST MODIFICATION ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	Void

[2]	ETSI ES 201 158 (V1.2.1 April 2002): "Lawful Interception; Requirements for network functions".

[3]	ETSI ES 201 671 (V3.1.1 May 2007): "Handover Interface for the lawful interception of telecommunications traffic".

[4]	Void

[5]	Void

[6]	Void

[7]	3GPP TS 33.106: "3G Security; Lawful Interception Requirements".

[8]	ANSI J-STD-025-A (April 2003): "Lawfully Authorised Electronic Surveillance".

[9]	 VOID

[10]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description".

[11]	3GPP TS 33.108: "3G Security; Handover interface for Lawful Interception".

[12]	 Void

[13]	3GPP TS 21.905: "Vocabulary for 3GPP Specifications".

[14]	3GPP TS 23.234: "3GPP system to Wireless Local Area Network (WLAN) Interworking; System Description".

[15]	3GPP TS 23.008: "Organization of subscriber data".

[16]	3GPP TS 29.234: "3GPP system to Wireless Local Area Network (WLAN) interworking; Stage 3".

[17]	3GPP TS 24.234: "3GPP system to Wireless Local Area Network (WLAN) interworking; User Equipment (UE) to network protocols; Stage 3".

[18]	IETF RFC 1122 (October 1989): "Requirements for Internet Hosts -- Communication Layers".

[19]	IETF RFC 1123 (October 1989): "Requirements for Internet Hosts -- Application and Support".

[20]	3GPP TS 23.246: "Multimedia Broadcast/Multicast Service (MBMS); Architecture and Functional Description".

[21]	3GPP TS 24.147: "Conferencing Using the IP Multimedia (IM) Core Network (CN) subsystem 3GPP Stage 3".

[22]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[23]	3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses".

[24]	3GPP TS 29.273: "Evolved Packet System (EPS); 3GPP EPS AAA interfaces".

[25]	3GPP TS 33.328: "IP Multimedia Subsystem (IMS) media plane security".

[26]	3GPP TS 29.212: "Policy and Charging Control (PCC); Reference points".

[27]	Void

[28]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[29]	3GPP TS 33.210: "Network Domain Security (NDS); IP network layer security".

[30]	3GPP TS 23.272: " Circuit Switched (CS) fallback in Evolved Packet System (EPS); Stage 2".

[31]	3GPP TS 22.220: " Service Requirements for Home NodeBs and Home eNodeBs".

[32]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network  (E-UTRAN); Overall description; Stage 2".

[33]	3GPP TS 25.467: "UTRAN architecture for 3G Home Node B (HNB); Stage 2"

[34]	3GPP TS 33.320: "Security of Home Node B (HNB) / Home evolved Node B (HeNB) ".

[35]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)".

[36]	IETF RFC 3966 (December 2004): "The Tel URILs for Telephone Numbers ".

[a]	IETF RFC 791:  “Internet Protocol”.

[b]	IETF RFC 2460:  “Internet Protocol, Version 6 (IPv6) Specification”.

[c]	IEFT RFC 3697:  “IPv6 Flow Label Specification”.

*** NEXT MODIFICATION ***

## 9.3	Provision of Intercept Related Information

Figure 25 shows the transfer of intercept related information to the DF2. If an event for / from a mobile subscriber occurs, the PDG, WAG, or the AAA Server sends the relevant data to the DF2.  Packet Header Information Reporting is a national option. For Packet Header Information Reporting, a PDG/WAG either isolates the relevant data and sends it to the DF2 or sends the packet stream to another entity in the network (e.g., DF3) for isolation which then provides the relevant data to the DF2.

intercepted

subscriber

AAA Server

other

party

Delivery

Function 2

LEMF

PDG/WAG

Figure 25: Provision of Intercept Related Information

### 9.3.1	X2-interface

The following information needs to be transferred from the 3G GSN or the HLR to the DF2 in order to allow a DF2 to perform its functionality:

-	target identity (MSISDN, IMSI, IMEI);

-	events and associated parameters as defined in clauses 7.3.2 and 7.4 may be provided;

-	the target location (if available) or the IAs in case of location dependent interception;

-	Correlation number;

-	Quality of Service (QoS) identifier;

-	Encryption parameters (keys and associated parameters for decrypting CC), if available and necessary.

The IRI should be sent to DF2 using a reliable transport mechanism.

The PDG/WAG detects packets containing packet header information in the communications path but the information needed for Packet Header Information Reporting may need to be transferred from the PDG/WAG either directly to the DF2 or via another network entity in order to allow the DF2 to perform its functionality.

### 9.3.2	Structure of the events

The following events are applicable to AAA Server:

-	I-WLAN Access Initiation;

-	I-WLAN re-authentication,

-	I-WLAN Access Termination;

-	I-WLAN Tunnel Establishment;

-	I-WLAN Tunnel Disconnect;

-	Start of Intercept with I-WLAN Communication Active;

The following events are applicable to the PDG and WAG:

-	I-WLAN Tunnel Establishment;

-	I-WLAN Tunnel Disconnect;

-	Start of Intercept with I-WLAN Communication Active

-	Packet Header Information Reporting.

A set of possible elements as shown below is used to generate the events. Information associated with the events are transmitted from the PDG, WAG or AAA server to DF2.

NOTE:	Some of these parameters apply to the PDG or WAG and some apply to the AAA server.  Parameters sent from the PDG, WAG or AAA server is dependent on what is available at the network element. If interception is performed at the PDG, then Packet Header Information Reporting shall also be performed at the PDG and not at the WAG.

Table 3: Information Events for WLAN Interworking Event Records

| Element | PDG | AAA Server |
| --- | --- | --- |
| Observed MSISDN MSISDN of the target subscriber (monitored subscriber). | Available, see TS 29.234 | Available, see TS 29.234 |
| Observed NAI NAI of the target subscriber (monitored subscriber). | Not available | Available, see TS 29.234 |
| Observed IMSI IMSI of the target subscriber (monitored subscriber). | Available, see TS 24.234 | Available, see TS 29.234 |
| Event type Description which type of event is delivered: I-WLAN Access Initiation, I-WLAN Access Termination, I-WLAN Tunnel Establishment, I-WLAN Tunnel Disconnect, Start of Intercept with I-WLAN Communication Active, Packet Data Header Information. | Available from ICE | Available from ICE |
| Event date Date of the event generation in the PDG or the AAA server. | Available from ICE | Available from ICE |
| Event time Time of the event generation in the PDG or the AAA server. Timestamp shall be generated relative to the PDG or AAA server internal clock. | Available from ICE | Available from ICE |
| WLAN UE Local IP address The WLAN UE Local IP address of observed party. The WLAN UE Local IP address field specified in TS 24.234 and IETF RFC 2409, represents the IPv4/IPv6 address of the WLAN UE in the WLAN AN. It is an address used to deliver the packet to a WLAN UE in a WLAN AN. Note that this address might be dynamic. | Available, see TS 24.234 and IETF RFC 2409 | Not available |
| WLAN UE MAC address The WLAN MAC address of the target subscriber. Note that this address might be dynamic and the validity of the MAC Address is outside of the scope of 3GPP. | Not available | Available, see TS 29.234 |
| WLAN UE Remote IP address The WLAN UE Remote IP address of observed party. The WLAN UE Remote IP address field specified in TS 24.234, represents the IPv4/IPv6 address of the WLAN UE in the network being accessed by the WLAN AN. It is an address used in the data packet encapsulated by the WLAN UE-initiated tunnel and is the source address used by applications in the WLAN UE. Note that this address might be dynamic. | Available, see TS 24.234 | Not available |
| WLAN Access Point Name The W-APN of the access point. | Available, see TS 24.234 | Available, see TS 29.234 |
| WLAN Operator Name The name of the WLAN operator name serving the target subscriber. | Not available | Available, see TS 29.234 |
| WLAN Location Name The name of the location of the WLAN serving the target subscriber (e.g., string like "coffee shop" or "airport", etc.). | Not available | Available, see TS 29.234 |
| WLAN Location Information Location Information regarding the WLAN as provided in RADIUS or DIAMETER signalling exchanged with the AAA server. | Not available | Available, see TS 29.234 |
| Correlation Number The correlation number is used to correlate CC and IRI. The correlation number is also used to allow the correlation of IRI records. In case of the AAA server, the Correlation Number is only used to correlate IRI records. | Generated for LI by PDG | Generated for LI by AAA server |
| Network Element Identifier Unique identifier for the element reporting the ICE. | Generated for LI by PDG | Generated for LI by AAA server |
| Initiator The initiator of the request either the network or the WLAN UE. | Generated for LI by PDG | Generated for LI by AAA server |
| NAS IP/IPv6 address The IP or IPv6 address of the NAS in the WLAN. | Not available | Available, see TS 29.234 |
| Visited PLMN ID Identity of the visited PLMN to which the user is terminating their WLAN tunnels or through which the user is establishing their WLAN tunnels. | Not available | Available, see TS 29.234 |
| Session Alive Time The amount of time in seconds during which the target subscriber can be registered for WLAN access. | Not available | Available, see TS 29.234 |
| Failed access reason Provides the reason for why a WLAN access attempt failed ("Authentication Failed"). | Not available | Available from ICE |
| Session termination reason Provides a reason for why a WLAN access session is terminated. | Not available | Available, see TS 29.234 |
| Failed tunnel establishment reason Provides a reason for why a WLAN tunnel establishment failed ("Authentication failed" or "Authorization failed"). | Available from ICE | Available from ICE |
| NSAPI Network layer Service Access Point Identifier The NSAPI information element contains an NSAPI identifying a PDP Context in a mobility management context specified by the Tunnel Endpoint Identifier Control Plane. This is an optional parameter to help DF/MF and LEA's to distinguish between the sending mobile access networks | Optional available according 23.234 Annex F; defined 29.060 7.7.17 | Not available |
| Destination IP Address The IP address, including type IPv4 or IPv6, of the destination of the IP packet. | Available from ICE | Available from ICE |
| Destination Port Number The port number of the destination of the IP packet. | Available from ICE | Available from ICE |
| Flow Label (IPv6 only) The field in the IPv6 header that is used by a source to label packets of a flow (see RFC 3697 [c]). | Available from ICE | Available from ICE |
| Packet Count The number of packets detected and reported (for a particular summary period). | Available from ICE | Available from ICE |
| Packet Data Summary Reason The reason for a Packet Data Summary message being sent to the LEMF (e.g., timed out, counter expiration, end of session) | Available from ICE | Available from ICE |
| Packet Size The size of the packet. (i.e., Total Length Field in IPv4 or Payload Length field in IPv6) | Available from ICE | Available from ICE |
| Source IP Address The IP address, including type IPv4 or IPv6, of the source of the IP packet. | Available from ICE | Available from ICE |
| Source Port Number The port number of the source of the IP packet. | Available from ICE | Available from ICE |
| Sum of Packet Sizes (for a particular summary period) The sum of values contained in the Total Length fields of the IPv4 packets or the sum of the values contained in the Payload Length fields of the IPv6 packets. | Available from ICE | Available from ICE |
| Summary Period Includes the dates and times of the first and last packets in a particular packet data interval. | Available from ICE | Available from ICE |
| Transport Protocol (e.g., TCP) The identification of the transport protocol of the packet or packet flow being reported. | Available from ICE | Available from ICE |

Table 3a: Information Events for WLAN Interworking Event Records - WAG

| Element | WAG |
| --- | --- |
| Observed MSISDN MSISDN of the target subscriber (monitored subscriber). | Available, see 3GPP TS 29.234 |
| Observed IMSI IMSI of the target subscriber (monitored subscriber). | Available, see 3GPP TS 29.234 |
| Event type Description which type of event is delivered: I-WLAN Tunnel Establishment, I-WLAN Tunnel Disconnect, Start of Intercept with I-WLAN Communication Active, Packet Data Header Information. | Available from ICE |
| Event date Date of the event generation in the PDG/WAG or the AAA server. | Available from ICE |
| Event time Time of the event generation in the PDG/WAG or the AAA server. Timestamp shall be generated relative to the PDG/WAG or AAA server internal clock. | Available from ICE |
| WLAN UE IP address The WLAN UE IP addess of observed party. The WLAN UE IP address field contains the IPv4/IPv6 address (specified by 3GPP TS 29.234) of the WLAN UE tunnel endpoint as seen by the WAG. Note that this address might be dynamic. | Available, see 3GPP TS 29.234 |
| WLAN PDG Tunnel Endpoint IP address The WLAN PDG Tunnel Endpoint IP address field contains the IPv4/IPv6 address of the PDG (as specified in 3GPP TS 29.234) as seen by the WAG. Note that this address might be dynamic. | Available, see 3GPP TS 29.234 |
| WLAN Access Point Name The W-APN of the access point. | Available, see 3GPP TS 29.234 |
| Correlation Number The correlation number is used to correlate CC and IRI. The correlation number is also used to allow the correlation of IRI records. | Generated for LI by WAG |
| Network Element Identifier Unique identifier for the element reporting the ICE. | Generated for LI by WAG |
| NAS IP/IPv6 address The IP or IPv6 address of the NAS in the WLAN. | Available, see 3GPP TS 29.234 |
| Tunnel Protocol The Tunnel Protocol as defined in the Routing-Policy AVP in 3GPP TS 29.234. | Available, see 3GPP TS 29.234 |
| Source Ports The list or range of source ports as specified in the Routing-Policy AVP provided by the AAA server in 3GPP TS 29.234. | Available, see 3GPP TS 29.234 |
| Destination Ports The list or range of destination ports as specified in the Routing-Policy AVP provided by the AAA server in 3GPP TS 29.234. | Available, see 3GPP TS 29.234 |
| Session Alive Time The amount of time in seconds during which the target subscriber can be registered for WLAN access. | Available, see 3GPP TS 29.234 |
| Destination IP Address The IP address, including type IPv4 or IPv6, of the destination of the IP packet. | Available from ICE |
| Destination Port Number The port number of the destination of the IP packet. | Available from ICE |
| Flow Label (IPv6 only) The field in the IPv6 header that is used by a source to label packets of a flow (see RFC 3697 [c]). | Available from ICE |
| Packet Count The number of packets detected and reported (for a particular summary period). | Available from ICE |
| Packet Data Summary Reason The reason for a Packet Data Summary message being sent to the LEMF (e.g., timed out, counter expiration, end of session) | Available from ICE |
| Packet Size The size of the packet. (i.e., Total Length Field in IPv4 or Payload Length field in IPv6) | Available from ICE |
| Source IP Address The IP address, including type IPv4 or IPv6, of the source of the IP packet. | Available from ICE |
| Source Port Number The port number of the source of the IP packet. | Available from ICE |
| Sum of Packet Sizes (for a particular summary period) The sum of values contained in the Total Length fields of the IPv4 packets or the sum of the values contained in the Payload Length fields of the IPv6 packets. | Available from ICE |
| Summary Period Includes the dates and times of the first and last packets in a particular packet data interval. | Available from ICE |
| Transport Protocol (e.g., TCP) The identification of the transport protocol of the packet or packet flow being reported. | Available from ICE |

*** THIRD MODIFICATION ***

### 9.4.X	Packet Header Information Reporting

Packet Header Information Reporting can be done either on a per-packet (i.e., non-summarized) basis or in a summary report.

### 9.4.X.1	Packet Data Header Report

This event is used to provide packet header reports on a per packet basis (non-summarized reporting) and is triggered by each packet sent or received by the target.  These elements will be delivered either directly to DF2 or via another network entity if available:

Table A: I-WLAN Packet Header Information Reporting - PDG

Observed MSISDN

Observed IMSI

Observed NAI

Event Type

Event Time

Event Date

Correlation number

WLAN UE Local IP Address

WLAN UE Remote IP address

WLAN Access Point Name

Network Element Identifier

Initiator (optional)

Source IP Address

Source Port Number

Destination IP Address

Destination Port Number

Transport Protocol (e.g., TCP)

Packet Size

Flow Label (IPv6 only)

Table B: I-WLAN Packet Header Information Reporting – WAG

Observed MSISDN

Observed IMSI

Event Type

Event Time

Event Date

Correlation number

WLAN UE IP address

WLAN PDG Tunnel Endpoint IP address

WLAN Access Point Name

NAS IP/IPv6 address

Tunnel Protocol

Source Ports

Destination Ports

Network Element Identifier

Source IP Address

Source Port Number

Destination IP Address

Destination Port Number

Transport Protocol (e.g., TCP)

Packet Size

Flow Label (IPv6 only)

### 9.4.X.2	Packet Data Summary Report

This event is used to report:

- the source and destination information derived from the packet headers, including:

- source and destination IP Addresses,

- IP next-layer protocol,

- Layer-4 ports, and

- Flow label, if the packet is IPv6

- summary information for the number of packets and bytes transmitted or received by the subject for each unique packet flow within a WLAN tunnel,  and

- the date and the time of the first and last packets associated with that packet flow.  A packet flow is defined as the 6-tuple of source/destination IP address/port number and the layer 4 protocol and WLAN tunnel.

IP addresses and the IP next-layer protocol are always reported, the flow label is reported if the packet is IPv6, and the layer-4 ports are reported.

The event provides packet summary reports for each unique packet data session (PDP context) and packet flow, and is triggered by one of the following:

- start of a packet flow associated with a WLAN Tunnel

- an interim report for a packet flow associated with a WLAN Tunnel is to be reported

- end of a packet flow associated with a WLAN Tunnel (including end of the WLAN Tunnel itself).

An interim report can be triggered by

- The expiration of a configurable timer per intercept (called a Summary Timer). The Summary Timer is configurable in units of seconds;

- A per-intercept configurable count threshold is reached.

These elements will be delivered either directly to DF2 or via an MF for each packet flow if available:

Table C: I-WLAN Packet Header Summary Reporting - PDG

Observed MSISDN

Observed IMSI

Observed NAI

Event Type

Event Time

Event Date

Correlation number

WLAN UE Local IP Address

WLAN UE Remote IP address

WLAN Access Point Name

Network Element Identifier

Initiator (optional)

Source IP Address

Source Port Number

Destination IP Address

Destination Port Number

Transport Protocol (e.g., TCP)

Flow Label (IPv6 only)

Summary Period

Packet Count (for this summary period)

Sum of Packet Sizes (for this summary period)

Table D: I-WLAN Packet Header Summary Reporting – WAG

Observed MSISDN

Observed IMSI

Event Type

Event Time

Event Date

Correlation number

WLAN UE IP address

WLAN PDG Tunnel Endpoint IP address

WLAN Access Point Name

NAS IP/IPv6 address

Tunnel Protocol

Source Ports

Destination Ports

Network Element Identifier

Source IP Address

Source Port Number

Destination IP Address

Destination Port Number

Transport Protocol (e.g., TCP)

Flow Label (IPv6 only)

Summary Period

Packet Count (for this summary period)

Sum of Packet Sizes (for this summary period)

If the packets are IPv4, the sum of all observed packet sizes is the sum of the values contained in the Total Length field of each packet as specified in IETF RFC 791[a].

If the packet is IPv6, the sum of all observed packet sizes is the sum of the values contained in the Payload Length field for each packet as specified in IETF RFC 2460 [b].

If no packets were detected for the duration of the Summary Timer, then the Packet Data Summary Report shall not be sent.

*** END OF MODIFICATIONS ***