3GPP SA3LI#79e-b	S3i200730

# eMeeting, 10-12 November 2020

# 3GPP TSG-RAN WG3 #110-e						R3-207062

# 2 – 12 November 2020

# Title:	Reply LS on SA WG2 assumptions from conclusion of study on architecture aspects for using satellite access in 5G

# Response to:	Reply LS on SA WG2 assumptions from conclusion of study on architecture aspects for using satellite access in 5G (R3-206842/S2-2008307)

# Release:	Release 17

# Work Item:	NR_NTN_solutions, 5GSAT_ARCH

Source:	RAN3

To:	SA2, RAN2

Cc:	SA3-LI, SA5

Contact Person:

Name:	Luis Lopes

Tel. Number:

E-mail Address:	llopes@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

RAN3 thanks SA2 for their reply LS in R3-206842/S2-2008307.

RAN3 would like to inform SA2 and RAN2 that RAN3 agreed that a Cell ID as used in the User Location Information on the NG/N2 interface corresponds to a fixed geographical area, and the Tracking Area is coupled with geographical area.

Although it is up to RAN2 to decide how cell IDs and TAIs are broadcast in SIB1 for NTN, RAN3 would like to share the approaches so far considered in RAN3:

a)	On Uu, SIB content corresponds to momentary coverage area of a satellite beam related to the geographically fixed areas of TAs/Cells - irrespective of whether the beam is fixed or moving.

b) 	The cell ID used on Uu SIB content (and probably on Xn) are decoupled from cell ID used on NG(N2). The respective mapping is performed in RAN. This requires gNB to acquire the UE’s location information.

To progress further, RAN3 would also like to ask RAN2 to provide any feedback on the above approaches including, e.g. SIB aspects, and how the RAN could acquire information on the UE’s location if this is needed e.g. for above approach, and for registration, etc.

Note: NTN WID includes “identification of potential issues associated to the use of the existing Location Services (LCS) application protocols to locate UE in the context of NTN and specify adaptations if any [RAN2/3]”. This could be used to determine the UE location with sufficient level of accuracy if needed, as requested in R3-206842.

2. Actions:

To SA WG2.

ACTION: 	RAN3 kindly asks SA2 to take the above information into account.

To RAN WG2.

ACTION: 	RAN3 kindly asks RAN2 to provide feedback on the above.

3. Date of Next RAN3 Meetings:

RAN3#111-e	January 2020	Electronic meeting