3GPP SA3LI#98	S3i250322

# 15-18 July 2025, Florence (Italy)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.127 | CR | 0296 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PTC architecture diagram |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (Nokia) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI18 |  |  |  |  |  | Date: |  |  | 2025-07-03 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Most of the clauses in the specification have an architecture diagram specially when the LI is introduced to a new NF. The diagram helps to understand the scope of LI functions provided within the NF. Such an architecture diagram is missing for the PTC clause. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Diagram is added. Statement is added to say that target non-local ID does not apply to PTC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The spec will not be consistent in providing the stage 2 details. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6.1, 7.6.1.1 (new), 7.6.1.2 (new), 7.6.8. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### ** First Change **

### 7.6.1	General

#### 7.6.1.1	Background

In the present clause, "PTC" will be used to reference events or services that occur in either of two different architectures unless specified otherwise, e.g., Mission Critical Push To Talk (MCPTT) or Push to talk over Cellular (PoC).

The following servers support PTC architecture:

-	MCPTT servers (Including Common services core as defined in TS 23.280 [24]).

-	PoC servers (Including Shared XDMS as defined in OMA-AD-PoC-V2_1-20110802-A [25]).

The PTC server will be used to represent the MCPTT server or PoC server for group communication services.

If two or more different parties involved in a PTC communication are each a target of interception, each interception shall operate independently of the others and the results of each intercept shall be delivered to the respective LEMF in accordance with the applicable warrant.

### ** Next Change **

#### 7.6.1.2	LI architecture for PTC

The figure 7.6.1.2-1 gives a reference point representation of the LI architecture for PTC with PTC Server providing the IRI-POI and CC-POI functions. Any interactions needed between the IRI-POI and CC-POI (e.g. to use the same Correlation Identifier) is implementation specific.

Figure 4.6.1.2-1: LI architecture for PTC

The PTC Server is an IMS-based application server and therefore all the SIP signalling messages and media pass to and from the target UE pass through the IMS domain.

The PTC Server shown in figure 4.6.1.2-1 includes the functionalities of a Presence Server as well.

The IRI-POI present in the PTC Server detects the PTC related events, generates and delivers the related xIRI to the MDF2 over LI_X2. The MDF2 delivers the IRI messages to the LEMF over LI_HI2.

When interception of communication contents is required, the CC-POI present in the PTC Server generates the xCC from PTC media and delivers the xCC (that includes the correlation number and the target identity) to the MDF3. The MDF3 delivers the CC to the LEMF over LI_HI3.

### ** Next Change **

### 7.6.8	Network topologies

The PTC server resides in the home network. The PTC Server shall provide IRI-POI and CC-POI functionality as illustrated in figure 4.6.1.2-1. The concept of target non-local ID does not apply to PTC.

### ** End of all Changes **