SA3LI#98	 S3i250337

July 15-18, 2025, Florence (Italy)

Source:	Nokia

Title:	pCR #8 to TR 33.929-6: RCS topology Part VI – RCS servers in different third party domains, CSP RCS users in the different CSP domains

Document for:	Agreement

Agenda Item:	10

Work Item / Release:	LI19-GUIDE

Abstract of the contribution: This pCR provides input text to TR 33.929-6 LI for RCS. Provides some RCS topologies when the RCS servers are in different third party RCS domains.  Here, the CSP RCS users are in the different CSP domains.

BACKGROUND

The TR 33.929-6 provides the implementation guidance for the LI aspects related to the RCS. This pCR gives provides the RCS network topologies for RCS when the RCS servers are in different third party domains. The CSP RCS users are in different CSP domains.

*** Related text from the previous pCR ****

# 1	Scope

# 2	References

# 3	Definitions of terms, symbols and abbreviations

# 4	Illustration of LI for IMS-based RCS

## 4.1	Overview

## 4.2	RCS topology scenarios

### 4.2.1	General

### 4.2.2	Few conventions used

In all the illustrations, when the RCS user is a subscriber of a CSP domain, 5G radio and packet core is assumed. Within the drawings, (A) represents the ingress side and (B) represents the egress side. When ingress and egress have to be shown in multiple network domains, (A1), (A11), (B1), B (11), etc., are used.

The figure 4.2.2-1 shows some of the conventions used within the topology drawings (in a general way):

Figure 4.2.2-1: Few conventions used within the drawings

### 4.2.3	RCS servers in CSP domain

### 4.2.4	Third party RCS service providers

#### 4.2.4.1	General

#### 4.2.4.2	One third party RCS service provider

#### 4.2.4.3	Different third party RCS service providers

##### 4.2.4.3.1	General

This clause illustrates the case when RCS Servers are in different third party RCS service provider domains with various cases depending on where the RCS users are.

Scenario 1: RCS users are not in CSP domain.

Scenario 2: RCS users are in the same CSP domain that does not have IMS.

Scenario 3: RCS users are in the same CSP domain that has IMS.

Scenario 4: RCS users are in the different CSP domains that do not have IMS.

Scenario 5: RCS users are in the different CSP domains that have IMS.

Not all the scenarios listed above may be deployed in practice, The present document only provides the illustrations in case those scenarios are deployed.

The illustrations do not show the roaming cases as they are similar to the illustrations shown in clause 4.2.4.2. The illustrations do not show the LI for other IMS-based services as they are also similar to the illustrations shown in clause 4.2.4.2.

##### 4.2.4.3.2	Scenario 1: RCS users are not in CSP domain

##### 4.2.4.3.3	Scenario 2: RCS users are in the same CSP domain that does not have IMS

##### 4.2.4.3.4	Scenario 3: RCS users are in the same CSP domain that has IMS

PROPOSAL

Incorporate the following to the TR 33.929-6.

***** New text ****

##### 4.2.4.3.5	Scenario 4 : RCS users are different CSP domains that do not have IMS

In the illustrations shown in this clause, RCS service is provided by different third party RCS service providers and the RCS users are in two CSP domains that do not have IMS.

RCS User (A) is a subscriber in CSP domain A1) and RCS User (B) is a subscriber in the CSP domain B1. The CSP domains provide just the access when those users establish RCS sessions and as such both signalling and media pass through the CSP domains over the packet data connections.

The figure 4.2.4.3.5-1 illustrates the case where RCS User (A) is a target for RCS in the third party RCS service provider domain A and RCS User (B) is a target for RCS in the third party RCS service provider domain B.

Figure 4.2.4.3.5-1: RCS users in the different CSP domains that do not have IMS

The RCS Server (A) present in the third party RCS service provider domain A provides the LI functions when RCS User (A) is a target for RCS in the third party RCS service provider domain A and RCS Server (B) present in the third party RCS service provider domain B provides the LI functions when the RCS User (B) is a target for RCS in the third party RCS service provider domain B.

The RCS Servers provide both IRI-POI and CC-POI functions.

Target non-local ID

The figure 4.2.4.3.5-2 illustrates the case where RCS User (A) is a target non-local ID for RCS in the third party RCS service provider domain B and RCS User (B) is a target non-local ID for RCS in the third party RCS service provider domain A.

Figure 4.2.4.3.5-2: RCS users in the different CSP domains that do not have IMS (target non-local ID)

The RCS Server (A) present in the third party RCS service provider domain A provides the LI functions when RCS User (B) is a target non-local ID for RCS in the third party RCS service provider domain A and RCS Server (B) present in the third party RCS service provider domain B provides the LI functions when the RCS User (A) is a target non-local ID for RCS in the third party RCS service provider domain B.

The RCS Servers provide both IRI-POI and CC-POI functions.

##### 4.2.4.3.6	Scenario 5: RCS users are in the different CSP domains that have IMS

This clause illustrates the case when RCS Servers are in different third party RCS service provider domains and the RCS users are different CSP domains that have IMS.

The illustrations focus RCS LI and non-roaming scenarios.

In the illustrations show in the clause, User (A) is a subscriber of CSP domain A1 and User (B) is a subscriber of CSP domain B1. Since the RCS Servers used for the RCS sessions are in third party RCS service provider domains, in this deployment option, both CSP domains provide just the access when those users establish RCS sessions and as such both signalling and media pass through the CSP domains over the packet data connection.

The signalling and media for non-RCS related IMS sessions do not pass through the third party RCS service provider domains.

The figure 4.2.4.3.6-1 illustrates the CSP deployment option 1 where RCS User (A) is a target for RCS in the third party RCS service provider domain A and RCS User (B) is a target for RCS in the third party RCS service provider domain B:

Figure 4.2.4.3.6-1: RCS users in the different CSP domains that have IMS

The RCS Server (A) present in the third party RCS service provider domain A provides the LI functions when User (A) is a target for RCS in the third party RCS service provider domain A and RCS Server (B) present in the third party RCS service provider domain B provides the LI functions when the User (B) is a target for RCS in the third party RCS service provider domain B.

The RCS Servers provide both IRI-POI and CC-POI functions.

Target non-local ID

The figure 4.2.4.3.6.2-2 illustrates the case where RCS User (A) is a target non-local ID for RCS in the third party RCS service provider domain B and RCS User (B) is a target non-local ID for RCS in the third party RCS service provider domain A.

Figure 4.2.4.3.6-2: RCS users in the different CSP domains that have IMS (target non-local ID)

The RCS Server (A) present in the third party RCS service provider domain A provides the LI functions when User (B) is a target non-local ID for RCS in the third party RCS service provider domain A and RCS Server (B) present in the third party RCS service provider domain B provides the LI functions when the User (A) is a target non-local ID for RCS in the third party RCS service provider domain B.

The RCS Servers provide both IRI-POI and CC-POI functions.

***** End of the new text ****