Source:	Nokia

Title:	On DaCAS recordings and recording scenarios

Document for:	Discussion & Agreement

Agenda Item:	7.6 – DaCAS

# 1. Introduction

At the Geneva meeting, the DaCAS work item [1] produced initial draft templates [2] for defining and collecting data for:

- potential target devices,

- setup of recording scenarios, and

- databases.

For the interim period, two telcos are planned [3]. The target for the first one is to progress the templates and continue to collect potential target devices.

This input proposes improvements for the recordings scenarios template and recordings database template. In addition, foreseen recording scenario types and guidelines on defining recording scenarios are discussed.

# 2. Discussion

For successful development of any example solution to any proposed target device, a certain set of recordings for different purposes are anticipated. In this section, a view on different recording scenario types as well as recommendations on defining recording scenarios are presented and discussed.

## 2.1 Recording scenario types

To achieve representative database for developing and evaluating example solutions as well as the proposed target devices, multiple different recording scenarios are expected. To further understand the needs for different recording scenarios, the source has identified at least following recording scenario types to be important for the successful progress of the DaCAS work.

Device characterization

- Recordings targeted to identify electro-acoustical characteristics of proposed target devices. For example, to identify device frequency response, etc.

Development recordings

- Recordings targeted to facilitate spatial capture algorithm development and potential tuning.

Evaluation recordings

- Recordings targeted to evaluate example solutions, including subjective and objective evaluation. For objective evaluation, alignment with TS 26.260 is expected.

Above categories should not be understood as strictly orthogonal scenarios, as it is expected that certain scenarios/recordings may be useful for multiple purposes. Furthermore, while other recording scenario types may be anticipated than what is presented here, it is seen useful to consider whether and how the proposed recording scenarios would fit into the above recording scenario types.

## 2.2 Recommendations on defining recording scenarios

Along the above recording scenario types, different recording scenarios for different purposes are anticipated. To ensure definition of meaningful and useful recording scenarios, at least following aspects should be considered when defining new recording scenarios:

- Varying recording scenarios should be represented, from simple single source scenarios to complex scenarios, and from free-field conditions to reverberant conditions.

- For the sake of reproducibility, high quality loudspeakers with well-defined source signals should be favored when possible.

- For device characterization, algorithm development, and objective evaluation purposes, controlled rooms in terms of reverberation and room frequency response should be favored. Referencing existing specifications is recommended, if applicable.

- For subjective evaluation purposes, vast number of varying environments should be represented. Number of proposed recording scenarios in indoor and outdoor environments should be in balance.

- Additional care should be taken when describing and defining the positioning of sound sources and UE.

# 3. Proposed updates on recording scenarios and database

The three initial templates/databases were agreed to gather necessary information about the proposed target devices, recording scenarios, and the recordings made using the proposed target devices in different recording scenarios. Furthermore, to allow development and evaluation on proposed target devices with varying example solutions by different proponents, the utilized recording scenarios for target device recordings should be as accurately defined as possible. To achieve this, further improvements and clarifications are proposed for the recording scenarios and recordings database templates.

## 3.1 Proposed updates to recording database template

The source’s understanding is that the agreed initial database for recordings in Initial template for database.xlsx will be used to collect recordings of (potential) target devices which are collected in Initial template for potential target devices.xlsx according to the specified recording setups, which are then collected in Initial template for setup of recording scenarios.xlsx. However, currently the relations between the aforesaid databases as well as all the necessary entry items are not sufficiently defined. For example, while the database entry numbers can be used to identify unique “target device - recording scenario” combinations, the collected entry should include actual information about the applied device and recording scenario, to clearly document collected content. The relations are illustrated in Figure 1 below. Furthermore, cross-referencing between the databases should be favored in general, in order to reduce redundancy.

Figure 1 – Templates/databases relation

In addition to the new identifier fields for target devices and recording scenarios, further improvements are made to the recordings database template, including removal of unnecessary redundancies between recording scenarios and recordings database, as well as further improving definitions of certain entry fields. In addition, examples are included to the proposal, to further clarify intention of different fields.

Thus, following fields and definitions are proposed to be used in the recordings database template:

No:

Recordings database entry identifier.

Target device No:

Target device identifier. This points to a certain entry in Target device database.

Recording scenario No:

Recording scenario identifier. This points to a certain entry in Recording scenarios database.

File name:

Name of the recording.

UE orientation / placement:

Describes the applied orientation of the UE during the recordings.

Should include as accurate description as possible. Information about usage of any additional positioning platforms, e.g., HATS, should be included. Referencing to the target device database is preferred, if applicable.

Example1: Azi 0, ele 0 reflects to the orientation where UE’s wide edge is towards the floor/ceiling (landscape), and the screen is towards the sound source.

Example2: Orientation B as described in target device database.

Sample rate:

Samples per second.

Bit depth:

Number of bits per sample.

Audio format:

File format of the recording, .wav or .pcm.

Rec duration:

Length in seconds. In addition, timestamps regarding the time instances of interest could be included.

Example: Overall length 45 s, first and last 5 s is silence.

Number of channels and microphone mapping:

Number of channels, and microphone mapping to channels (if applicable). Referencing to the target device database is preferred, if applicable.

Example: 3 channels, ch1=mic1, ch2=mic3, ch3=mic4.

Description/additional information:

Additional information regarding the recordings in free form.

License:

Information regarding the copyright and allowed usage.

Note: Regarding License, DaCAS work plan [3] indicates for common database (for first target devices) that “Legal framework must be finished if necessary”, with planned milestone at SA4#131-e-bis (April 11-17).

## 3.2 Proposed updates to recording scenario database template

On the recording scenarios, important aspects are the applied sound source, applied source signal and its characteristics, acoustic environment, as well as the physical relation between UE, sound source, and room (dimensions). To facilitate replication of proposed recording scenarios, these aspects should be described as accurately as possible. Thus, source’s view regarding the recording scenarios template items and their descriptions are presented below. In addition, examples are included to illustrate the intention of each field. Presented example should not be understood as a proposal of recording scenario.

No:

Recording scenario identifier

Sound source:

Applied sound source.

Examples: loudspeaker, human, car, etc.

Source signal:

Applied source signal.

Example: speech, white noise, 440 Hz sine, etc.

Signal characteristics:

Recommended to include following information if applicable: Language, Frequency range, Length, Level, Sample rate, Bit depth. In case of a real source, accurate description of the sound scene.

Example:

Language: English,

Range: 150 Hz – 12 kHz

Length: 45 s

Level: -16 dB RMS

Sample Rate: 48 kHz

Bit depth: 16 bit

Sound source calibration:

Describes the sound source calibration procedure. Recommended to include at least information regarding the playback level and calibration procedure. Additionally, frequency response calibration procedures/targets could be described, if applicable.

Example:

Level: Loudspeaker calibrated to 80 dBSPL(A) at the UE position, by utilizing 0 dBFS pink noise.

Frequency response: Not applicable.

Acoustic environment:

Acoustic properties of the recording scenario, for example information about RT60 value. Referencing to an existing specification preferred, if applicable.

Example: Anechoic chamber, free-field conditions expected down to 200 Hz.

Detailed Positioning:

Detailed description on the sound source positioning relative to the UE. Should include at least Distance, Source direction, Source angles (azimuth, elevation). Alternatively, room dimensions and source/UE positions may be defined with suitable coordinates. In addition, figures of the layout could be attached here.

Example:

Distance: 0.5 m < dUE < 2 m

Source direction: Main diaphragm towards the UE

Source angles (azi, ele): azi = -90, 0, 90, ele = 0

Note that space between the walls and sound source/UE should be at least 0.5 m

Reference recording:

Description of the reference recording equipment and positioning, if applicable. All necessary information (e.g., microphone model) should be documented to the recording database.

Example: Free-field measurement microphone at the UE position.

Description/additional info:

Description of the recording process. Any additional information should be included.

While all the proposed fields are seen to be relevant for accurately describe applied recording scenario, in certain scenarios all fields may not be required or even possible to populate. Certain freedom may be allowed regarding the proposals, but the target should always be to describe the proposed scenario and setup as accurately as possible.

# 4. Summary

In this document, improvements and clarifications on recording scenarios database template and recordings database template are presented and proposed to be taken into use. Along removing of unnecessary redundancies and introducing new entry fields, the usage and relation between databases are clarified. Proposed templates for databases are attached to this submission (Template for recording scenarios.xlsx, Template for recordings database.xlsx). Source proposes to agree on using these templates for collecting the recording scenarios and recordings. Furthermore, existing proposals are proposed to be converted and extended to comply with the new templates. Additionally, if the initial proposal on databases (Initial template for database.xlsx) was targeted to collect also other relevant data in addition to the recordings, the proposed database template (Template for recordings database.xlsx) can be extended accordingly in the future.

In addition, general discussion on different types of expected recording scenarios, as well as general guidelines are presented in this document. Other proponents are kindly asked to consider presented aspects, when proposing new recording scenarios.

# References

[1]	SP-241962: WID on Diverse audio Capturing System for UEs (DaCAS)

[2]	S4-250289: “Templates for target devices and common database”, Xiaomi Technology Spain S.L

[3]	S4-250285: Work Plan for DaCAS v0.1