3GPP TSG-SA WG4 Meeting SA4-e (AH) Audio SWG post 131	S4aA250014

Online, 31 March 2025

Source:	HEAD acoustics GmbH

Title:	Ambient noise transparency in send direction

Agenda item:	1.7

Document for:	DISCUSSION

# Introduction

For traditional mono telephony, the performance evaluation of UE's noise reduction capabilities in send direction is an integral part of almost any available test specification like e.g., 3GPP TS 26.131/132. Such test methods typically specify an ambient noise simulation that is used in addition to the usual electro-acoustic test equipment for simultaneous speech and noise playback.

For immersive UEs, a first proposal for testing noise reduction in send direction of was presented (S4-250205) at SA4#131. Here it was also discussed if and up to which degree the existing test methods for mono telephony are applicable for UEs tested according to TS 26.260/261, since the capturing functionality of devices might be different for certain scenarios:

- UEs might be used to capture an immersive scene that could include spatial cues, background noise, talkers/conversations, etc. (e.g., a live concert). This case is referred to as "spatial capture" in TS 26.260, where the captured signal should be transmitted as naturally as possible. It can be anticipated that aggressive noise suppressors will most likely not be part of the signal processing chain in send direction.

- For UEs that are intended to be used mainly in conversational scenarios (e.g., in group audio terminal or handheld hands-free mode), the most relevant spatial cues to be preserved are the talkers and their direction/distance (referred to as "user capture" in TS 26.260). In such cases, it is expected to have an additional noise suppression included in the device.

- In addition, there can also be immersive scenarios that lie between these two cases.

Apart from other important open questions (like e.g., type ambient sound/noise field reproduction), it seems in general at least possible to find/agree on test methods that analyze the two opposite applications (i.e., "evaluate naturalness of scene" vs "evaluate speech distortion/noise intrusiveness"). Immersive UEs will most likely also either indicate their operational mode and/or might be even configurable towards the one or the other behaviour - but with regard to TS 26.260, it should not be anticipated that such an information is sufficiently precise/reliable or even always available.

# Proposal for transparency

In order to define clear test methods for TS 26.260, it is proposed to introduce a "transparency test" in advance to any further evaluation under ambient noise conditions. The intention is to quantify the presence and/or degree of a noise suppressor:

- The transparency test is based on clause 5.6.2 of TS 26.260 (Loudness, applicable to user & spatial capture mode and all UE types), which uses a speech signal to measure the Send Loudness Rating in dB (SLRSpeech).

- The measurement is repeated, but with a noise(-like) signal, that is "easy" to be reduced/cancelled for any type of noise suppressor (e.g., pink/speech-shaped/babble/car/…?). This signal is played back via mouth simulator or loudspeaker at a "suitable" level (which might be a "comparable", but not necessarily "the same" level), leading to SLRNoise.

- For UEs without (noticable) noise suppression, both measurements should provide about the same values (SLRNoise ≈ SLRSpeech).

- Vice versa for UEs with strong noise suppression: SLRNoise >> SLRSpeech

- The degree of noise suppression can be quantified as DNS = SLRNoise – SLRSpeech

Depending on the resulting value of DNS, UEs can then be classified into different capture type characteristics, which was (provisionally) suggested in S4-250205. Based on this classification, different performance requirements, test methods and/or different "test paths" (certain tests are skipped for certain classes) could apply.

# Conclusion

The present document introduced a first concept for handling (at least two) anticipated UE behaviours in ambient noise conditions. Since several details are still missing and need to be clarified, this document was submitted for discussion and to collect immediate feedback from delegates on this new test approach. In addition, the source intends to provide a more detailed description of the test method as well as some preliminary measurement results for SA4#131-bis-e (test series anticipated to be continued until SA4#132).