Source:	Nokia

Title:	On objective performance evaluation

Document for:	Discussion

Agenda Item:	1.7 – DaCAS

# 1. Introduction

In the DaCAS workplan [1], agreement on the definition of performance requirements and performance evaluation methodologies for example solutions is anticipated for the next SA4 meeting in Fukuoka. This document aims to initiate the discussion on the applicability of available objective test methodologies for performance evaluation of DaCAS example solutions.

In addition, subjective performance evaluation is of interest. This input does not cover these aspects.

# 2. Objective performance evaluation

One of the agreed DaCAS objectives according to the DaCAS WID [2] is the evaluation of the available immersive audio capture example solutions based on the relevant sending side terminal audio quality test methods defined in TS 26.260 [3]. Currently, four different test methods for assessing the send quality of IVAS based UEs are defined in section 5 of 3GPP TS 26.260.

While the tests may not be directly applicable due to the lack of suitable test system(s), all the defined tests can be modified to objectively assess the performance of available DaCAS example solutions. Furthermore, despite the lack of available test systems, it is anticipated that the test setups and test methodologies can be accurately followed by proponents, further providing comparable metrics for performance evaluation.

Thus, following test methodologies based on section 5 of 3GPP TS 26.260 with suitable modifications are seen to be applicable for evaluating DaCAS example solutions objectively.

Delay

Test method for assessing the UE delay in send direction is defined in clause 5.6.1 of TS 26.260. To assess accurately delay as defined, suitable test system including system simulator and precise data acquisition would be required. In addition, obtaining accurate time synchronization between the sound source playback and target device recording may be impossible task within the DaCAS timeline.

On the other hand, in DaCAS context, the delay of interest is the algorithmic delay of the example solution, i.e., delay from inserting the input signal to example solution and obtaining the output signal (IVAS input format signal). By assessing only the delay caused by the example solution, impact of testing inaccuracies and acoustic path can be mitigated. However, it is recognized, that there are many dependencies on the obtained delay of the example solution, e.g., platform where the example solution is run.

Figure 1 Delay assessment overview

Loudness

Assessment of the send loudness is defined in section 5.6.2 in TS 26.260. The applied analysis is based on the Send Loudness Rating calculation according to ITU-T P.79. A reason for assessing the loudness, would be to ensure sufficient level of the generated IVAS input format signal in a predefined environment.

In contrast to the specified test in TS 26.260, the assessment would be done directly based on the example solution output, i.e., without any encoding/decoding and transmission. Thus, the example solution output would be directly rendered to the analysis format (Mono) via IVAS reference renderer, and the analysis would be conducted according to the specified step for the rendered example solution output signal.

In addition, other loudness metrics and/or different test conditions could be considered for characterization. For example, loudness according to ITU-T BS.1770 could be of interest with different rendered formats.

Figure 2 Simplified test flow for loudness assessment

Frequency response

For assessing the frequency response, similar method as defined in clause 5.6.3 of TS 26.260 can be applied. Depending on the example solution output, i.e., the obtained IVAS input format, format specific calculation method can be applied according to clause 5.6.3.2. Defined test method is seen suitable as it is, as long as any assumptions regarding the reference client are excluded.

Figure 3 Simplified test flow for frequency response assessment

Directional information

Spatial capture accuracy is naturally important feature to assess. For the assessment, test methodology based on clause 5.6.4 of TS 26.260 can be employed for Stereo, SBA and MASA formats. Again, the assessment would be done directly to the example solution output, excluding any assumptions about the transmission and reference client. If other formats, e.g., ISM is tested, intermediate step for rendering the example solution output to suitable format could be considered.

Figure 4 Simplified test flow for Directional information assessment

# 3. Summary

Overview on the available objective test methods for IVAS based UEs in section 5 of the 3GPP TS 26.260 specification was presented. With sufficient modifications, the presented test methodologies are seen applicable for assessing the objective performance of the DaCAS example solutions.

# References

[1]	S4-250668: “Work Plan for DaCAS v0.3”

[2]	SP-241962: “WID on Diverse audio Capturing System for UEs”

[3]	3GPP TS 26.260: “Objective test methodologies for the evaluation of immersive audio systems” version 18.1.0 Release 18