Source:	Orange

Title:	On FS_ULBC application scenarios

## Document for:	Agreement

## Agenda Item:	1.9

- Introduction

At SA#107, the “Study on Ultra Low Bitrate Speech Codec” (FS_ULBC) has been approved. In the present Tdoc we address the following study objective from the WID description [1]

- Document the application scenarios for ultra-low bit rate communication services taking into account the use cases and potential requirements documented in TR 22.887 related to IMS Voice Call Using GEO Access

We propose revisions to an initial pCR to 26.940 [2] based on the initial scope discussion in [3].

- Proposal

See Annex, including editorial updates and a discussion on the impact of including a ULBC codec in IMS voice services.

## References

- 3GPP SP-250378, SID on Ultra Low Bitrate Speech Codec, Source: China Mobile Com. Corporation, vivo, Fraunhofer IIS, Qualcomm Incorporated, Spreadtrum, Dolby Laboratories Inc., Xiaomi, Huawei

- 3GPP S4-250591, Application scenario for IMS voice call using GEO satellite, Source: vivo, Fraunhofer IIS, Qualcomm

- 3GPP S4-250496, Clarifications on scope for FS_ULBC, Source: Orange

## Annex (pCR to 26.940)

* * * First Change * * * *

# 4	Application scenarios

## 4.1	Introduction

This clause introduces application scenarios that may be relevant for an Ultra-Low Bitrate speech Codec (ULBC). For each scenario, technical assumptions and resulting potential open questions are extracted. These assumptions and open questions are addressed in the remainder of this document to derive design constraints for a ULBC when used in this application scenario.

The primary application scenario is in the context of voice communication via Geostationary Earth Orbit (GEO) satellites as introduced in clause 4.2.

## 4.2	Scenarios with IMS Voice Call over GEO

### 4.2.1 	Background

Satellite communication plays an important role in extending terrestrial network coverage, ensuring seamless connectivity for users anytime, anywhere. This technology unlocks new opportunities across various sectors, including smartphones and IoT. Geostationary Earth Orbit (GEO) satellites can deliver low-data-rate services on a global scale. By leveraging existing GEO satellites, messaging and voice services can be deployed quickly and efficiently.

Conversational two-party communication is foreseen as the most common and essential use case for IMS voice over GEO satellite access. This use case is documented in clause 5.1 of TR 22.887 [2]. Due to the limitations of low transmission data rates (in the range of [1-3] kbps) as estimated in Table 7.4.2-1 in TS 22.261 [3], an ultra-low bit rate codec is required. Users may rely on this service when they are beyond terrestrial network coverage but within satellite range, ensuring essential connectivity.

Editor’s Note: 	The bit rate range of [1-3] kbps is a working assumption according to TS 22.261 and might be adjusted after coordination with RAN groups.

### 4.2.2 	Scenario Description

#### 4.2.2.1 	General

The typical UE is a handheld device supporting GEO satellite access with built-in microphones and loudspeakers or a monaural hands-free set, as outlined in Table 7.4.2-1 in TS 22.261 [3]. This means there is no need for the user to extend the antenna or carry any extra devices to access the IMS voice call service over GEO satellite access. It is expected that the feature is available on commercially widespread mobile devices.

The typical IMS voice call service includes both regular call and emergency call as outlined in clause 6.46.11 in TS 22.261 [3].  Such service can be provided as:

-	Supplementary regular IMS voice service provided by the terrestrial operators, especially when the user suffers from no terrestrial coverage.

-	Main regular IMS voice service provided by the satellite operators.

-	An IMS voice call service using GEO satellite access in case of emergency situations.

The UEs under consideration may typically be commercial smartphones, but other form factors such as IoT devices or automotive vehicles are not excluded.

In the following, different scenarios establishing an end-to-end voice service are considered.

#### 4.2.2.2 	Scenario 1: One UE connects via GEO-satellite access only

In a common scenario, one party in the conversation is assumed to be using a handheld mobile terminal over a GEO satellite network, while the other may be on a terrestrial mobile network (e.g., VoLTE, VoNR), a fixed-line connection, or another IMS-supported platform, as outlined in Figure 4.2.2.1, where a sketch of the bi-directional voice data flow for this main scenario is depicted.

Figure 4.2.2.2-1: Bi-directional voice data flow main scenario

NOTE: 	eNodeB is co-located with the ground station in Figure 4.2.2.2-1 and does not appear as a separate function, core network stands for 3GPP core network and IMS core network.

In this scenario, UE1 is a phone supporting IMS voice call service over GEO satellite access. One may distinguish two cases depending on UE2:

- UE2 is a “regular” phone supporting IMS voice call service but not supporting ULBC nor GEO satellite access: in this case UE2. In this case, UE2 may not be aware that UE1 is using a GEO satellite link during the communication and transcoding is performed in the core network.

- UE2 is an “upgraded” phone supporting IMS voice call service with ULBC, but using another access than satellite access (e.g., LTE, NR or WLAN). In this case UE2 may be able to communicate with UE1 using  ULBC in a transcoder-free operation.

#### 4.2.2.3 		Scenario 2: Both UEs connect via GEO-satellite access

In another scenario, both parties in the conversation are connected to a GEO satellite as outlined in Figure 4.2.2.3-1 using IMS-based communication services.

Figure 4.2.2.3-1: Bi-directional voice data flow sub-scenario

NOTE: 	eNodeB is typically collocated with the ground station and not explicitly mentioned here, core network typically stands for 3GPP core network and IMS core network.

When the GEO satellite operates in a transparent payload, the voice packets are transmitted to the ground before transmitted to the other UE, even if both UEs are connected to the same GEO satellite. In this case, the communication between UE1 and UE2 may use ULBC in a transcoder-free operation.

### 4.3.2	Extracted technical assumptions and open questions

#### 4.3.2.1	General

The following general assumptions apply.

-	As the UE may be a “regular”  smartphone, the ULBC needs to be implementable in real-time (encoding and decoding) on such devices and meet common complexity and power-consumption requirements.

-	As the IMS voice call service using GEO satellite access can be considered as coverage enhancement, the voice quality preferably meets the user expectation for deployed terrestrial voice services.

Editor’s Note: More details may be added. Considerations on whether existing audio quality tests (TS 26.131 and TS 26.132) may be considered.

#### 4.3.2.2	Scenario 1

The following assumptions apply for the scenario described in clause 4.2.2.2.

-	For the connection “UE1 – GEO satellite – Ground station” (UE1 uplink), the transmission data rate is significantly limited ([1-3] kbit/s), requiring an ultra-low bit rate codec fitting the transmission data rate for this link.

-	For the connection “Ground station – GEO satellite – UE1” (UE1 downlink), the transmission data rate is expected to be limited similarly to UE1 uplink.

-	For both uplink and downlink of UE1 it is expected that the link is subject to transmission errors reflecting GEO satellite access.

Editor’s Note: “expected” should be replaced by more technical evidence when available (e.g., after coordination with RAN groups)

-	The delay in uplink and downlink of UE1 is expected to be greater than the one of typical terrestrial networks.

Editor’s Note: “expected” should be replaced by more technical evidence when available (e.g., after coordination with RAN groups)

-	For the connection “Core Network – UE2” (UE2 downlink), the transmission data rate of a regular TN network is available. This link could be covered either by an existing IMS codec (transcoding necessary) or by the same ultra-low bit rate codec as used for the satellite link (transcoding-free).

-	To ensure seamless communication across different network types, roaming, etc. transcoding functionality in core network is likely needed.

Editor’s Note: The following open issues remain to be addressed:

-	Details on bitrate limitations in uplink and downlink are needed.

-	Details on typical error conditions (including the frame loss rate and frame loss pattern) and worst-case  conditions for both links are needed.

-	Details on transmission delays conditions for both links are needed.

-	What are the impacts when an ULBC codec is used in transcoding scenarios?

Editor’s Note: More details may be added.

#### 4.3.2.3	Scenario 2

The following assumptions apply for sub-scenario 1 described in clause 4.2.2.3.

-	For both connections “UE1 – GEO satellite – Ground station” and “Ground station – GEO satellite – UE2” the transmission data rate is significantly limited ([1-3] kbit/s), requiring an ultra-low bit rate codec fitting this transmission data rate for these links.

-	This scenario may allow both transcoded (ULBC  existing IMS speech codecs ULBC) and transcoding-free operation (ULBC end-to-end)

-	This scenario may operate in transcoding-free mode as ULBC is used end-to-end.

Editor’s Note: On top of some of the open questions in clause 4.3.2.2, the following open question apply:

- What is the end-to-end transmission latency in this scenario?

Editor’s Note: More details may be added.

## 4.3	Other scenario with IMS Voice Call with ULBC

Editor’s note: clarify the handling of ULBC codec in an IMS call between two UEs, UE1 and UE2, supporting ULBC but connected with no GEO access.

* * * End of Changes * * * *