3GPPSA4-(AH) Audio SWG post 132	S4aA250031

Online, 04 June 2025

Source:	Fraunhofer IIS

Title:	On the TBS for NB-IoT NTN GEO simulation

Spec:	3GPP TS 26.940

Agenda item:	1.9

Document for:	Agreement

# 1. Introduction

At the SA4 meeting #132, a first version of the channel simulation parameter for NB-IoT NTN GEO have been partially agreed in S4-251154. The document explicitly contains the “Editor’s NOTE: Company can report candidate values of TBS.” The current TBS set is listed in the following

[

Table 1 TBS and PHY bitrate for 80ms bundling

| TBS (bits) | 144 | 256 | 328 | 424 |
| --- | --- | --- | --- | --- |
| PHY bitrate (kbps) | 1.8 | 3.2 | 4.1 | 5.3 |
| Codec bitrate (kbps) | 1.0 | 2.4 | 3.3 | 4.5 |

Table 2 TBS and PHY bitrate for 160ms bundling

| TBS (bits) | [208] | 424 | 600 | 808 |
| --- | --- | --- | --- | --- |
| PHY bitrate (kbps) | [1.30] | 2.65 | 3.85 | 5.05 |
| Codec bitrate (kbps) | [0.9] | 2.25 | 3.45 | 4.65 |

Table 3 TBS and PHY bitrate for 320ms bundling

| TBS (bits) | [328] | 776 | 1000 | 1544 |
| --- | --- | --- | --- | --- |
| PHY bitrate (kbps) | [1.025] | 2.425 | 3.125 | 4.825 |
| Codec bitrate (kbps) | [0.8025] | 2.225 | 2.925 | 4.625 |

]

The source currently sees an artificial limit on the boundaries, i.e. the highest rates as well as on the lowest rates:

## Higher date rates

The higher rates are currently set in way that the resulting codec rate is lower than 4.75kbps. While noting that this corresponds to the lowest bitrate of AMR, the source wants to clarify that AMR at 4.75kbps is far below the service quality commonly available in 3GPP IMS-based services. Note the following test results available as part of EVS characterization (TR 26.952) or in S4-250857.

Service quality

AMR at 4.75 estimate

Figure 1 EVS characterization

For IMS service continuation, ULBC needs to provide a speech quality comparable to the IMS codecs and modes which are most widely deployed today to avoid a regression of service quality. As most MNOs support wide-band or super-wideband, EVS at 13.2 kbps and EVS at 24.4 kbps should be considered. To increase the chance to preserve the speech quality, the upper boundary TBS should be chosen independently of any AMR bit rate, but to ideally meet the current service quality using the maximum TBS available. Possible TBS sizes should therefore extend beyond 4.75kbps, where the maximum should be the result of the simulations and not the input, i.e. it is well possible that some higher TBS are identified as not being applicable.

## Lower date rates

The lower boundary seems arbitrary and should at least cover the range considered by SA1 and reflected in the WID, i.e. “[1-3] kbps” for the PHY bit rate. In general, lower data rates should be considered for the channel simulation as this may enhance capacity and robustness. Also, a balanced number of codec rates should be considered.

# Summary

The source examined the currently proposed TBS tables considered for the channel simulation. It was observed that the TBS are limited and do not cover the range required to fulfill the SA1 requirements (for the lower ends) and are also limited towards the higher rates, where it would be desirable to understand the highest available rates, depending on ptime, as the result of the simulations .While the source understands that not all TBS available in table 16.5.1.2-2 of 36.213 can be evaluated, it is proposed to span at for those simulations at least the [1-3] kbps PHY bit rate range with the lowest and highest TBS inside that range and then extend towards the higher rates within reasonable limits.

* * * First Change * * * *

[

Table 1 TBS and PHY bitrate for 80ms bundling

| TBS (bits) | 120 | 144 | 176 | 256 | 328 | 424 | 568 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PHY bitrate (kbps) | 1.5 | 1.8 | 2.2 | 3.2 | 4.1 | 5.3 | 7.1 |
| Codec bitrate (kbps) | 0.7 | 1.0 | 1.4 | 2.4 | 3.3 | 4.5 | 6.3 |

Table 2 TBS and PHY bitrate for 160ms bundling

| TBS (bits) | 176 | 208 | 328 | 424 | 600 | 808 | 1000 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PHY bitrate (kbps) | 1.1 | 1.30 | 2.05 | 2.65 | 3.85 | 5.05 | 6.25 |
| Codec bitrate (kbps) | 0.7 | 0.9 | 1.65 | 2.25 | 3.45 | 4.65 | 5.85 |

Table 3 TBS and PHY bitrate for 320ms bundling

| TBS (bits) | 328 | 504 | 776 | 1000 | 1544 | 2024 |
| --- | --- | --- | --- | --- | --- | --- |
| PHY bitrate (kbps) | 1.025 | 1.575 | 2.425 | 3.125 | 4.825 | 6.325 |
| Codec bitrate (kbps) | 0.8025 | 1.375 | 2.225 | 2.925 | 4.625 | 6.125 |

]

* * * End of Changes * * * *