Source:	Beijing Xiaomi Mobile Software

Title:	On TBS values for simulation model

## Document for:	Discussion & Agreement

## Agenda Item:	1.9

# Introduction

During the recent SA4 #132meeting, there were discussions on making simulation model to derive error traces. This document makes effort to select appropriate TBS values. The goal is to make agreements on the TBS values.

# Discussions

On the potential bitrates for ULBC codec, there are several aspects should be considered as follows.

- According to the conclusion from TR 22.287 [1], the potential bitrate of ULBC is [1-3] kbit/s.

- The lowest supported bitrate of any 3GPP codec is 4.75 kbit/s as provided by the narrow band AMR codec from TS 26.071[2]

- The bitrate range from S4-251123 [3] is [0.825-4.625]kbps.

- There are overlapping bitrates among existing 3GPP codecs, e.g. the bitrate range of AMR codec is [4.75,12.2] kbps, the bitrate range of AMRWB codec is [6.60,23.85] kbps.

Based on upper aspects, this document proposes to update the TBS values for the simulation model. Several TBS values are analysed as follows:

Table 2-1 Combinations of TBS 104bit w/ 80ms bundling time

| TBS | IRU | Number of RU | ITBS (MCS) | Duration Time w/o repetition |
| --- | --- | --- | --- | --- |
| 104 | 1 | 2 | 3 | 3.75scs: 2*32 =64ms 15scs： 2 * 8 = 16ms |
|  | 0 | 1 | 7 | 3.75scs: 1*32 =32ms 15scs： 1 * 8 = 8ms |

Table 2-2 Combinations of TBS 504bit w/ 80ms bundling time

| TBS | IRU | Number of RU | ITBS (MCS) | Duration Time w/o repetition |
| --- | --- | --- | --- | --- |
| 504 | 5 | 6 | 5 | 3.75scs: 6*32 =192ms 15scs： 6 * 8 = 48ms |
|  | 4 | 5 | 6 | 3.75scs: 5*32 =160ms 15scs： 5 * 8 = 40ms |
|  | 2 | 3 | 10 | 3.75scs: 3*32 =96ms 15scs： 3 * 8 = 24ms |

Table 2-3 Combinations of TBS 136bit w/ 160ms bundling time

| TBS | IRU | Number of RU | ITBS (MCS) | Duration Time w/o repetition |
| --- | --- | --- | --- | --- |
| 136 | 0 | 1 | 9 | 3.75scs: 1*32 =32ms 15scs： 1 * 8 = 8ms |

Table 2-4 Combinations of TBS 1000bit w/ 160ms bundling time

| TBS | IRU | Number of RU | ITBS (MCS) | Duration Time w/o repetition |
| --- | --- | --- | --- | --- |
| 1000 | 7 | 10 | 6 | 3.75scs: 10*32 =320ms 15scs： 10 * 8 = 48ms |
|  | 6 | 8 | 7 | 3.75scs: 8*32 =256ms 15scs： 8 * 8 = 64ms |
|  | 5 | 6 | 10 | 3.75scs: 6*32 =192ms 15scs： 6 * 8 = 48ms |

Table 2-5 Combinations of TBS 1736bit w/ 320ms bundling time

| TBS | IRU | Number of RU | ITBS (MCS) | Duration Time w/o repetition |
| --- | --- | --- | --- | --- |
| 1736 | 7 | 10 | 10 | 3.75scs: 10*32 = 320ms 15scs： 10* 8 = 80ms |

There are some errors on TBS values of 600 and 328, the corresponding PHY bitrate and Codec bitrate should be corrected.

# Conclusion

It is proposed to update the TBS values tables as follows:

Table 1 TBS and PHY bitrate for 80ms bundling

| TBS (bits) | 104 | 144 | 256 | 328 | 424 | 504 |
| --- | --- | --- | --- | --- | --- | --- |
| PHY bitrate (kbps) | 1.3 | 1.8 | 3.2 | 4.1 | 5.3 | 6.3 |
| Codec bitrate (kbps) | 0.5 | 1.0 | 2.4 | 3.3 | 4.5 | 5.5 |

Table 2 TBS and PHY bitrate for 160ms bundling

| TBS (bits) | 136 | [208] | 424 | 600 | 808 | 1000 |
| --- | --- | --- | --- | --- | --- | --- |
| PHY bitrate (kbps) | 0.85 | [1.30] | 2.65 | 3.75 | 5.05 | 6.25 |
| Codec bitrate (kbps) | 0.45 | [0.9] | 2.25 | 3.35 | 4.65 | 5.85 |

Table 3 TBS and PHY bitrate for 320ms bundling

| TBS (bits) | [328] | 776 | 1000 | 1544 | 1736 |
| --- | --- | --- | --- | --- | --- |
| PHY bitrate (kbps) | [1.025] | 2.425 | 3.125 | 4.825 | 5.425 |
| Codec bitrate (kbps) | [0.825] | 2.225 | 2.925 | 4.625 | 5.225 |

# References

[1] TR 22.287, "Study on satellite access - Phase 4".

[2] TS 26.071,” AMR speech Codec”.

[3] S4-251123, End to end simulation and channel characteristics