Source:	Samsung Electronics Co., Ltd.

Title:	[FS_ULBC] GEO satellite channel parameters for FS_ULBC

## Document for:	Discussion and agreement

## Agenda Item:	1.9

# 1		Introduction

In the last Audio SWG telco on June 4, tdoc S4aA250032 was presented to introduce the RAN1 link budget analysis on NB-IoT for Non-Terrestrial Networks (NTN) for GEO satellite, as described in TR 36.763 [2]. This input is the revision to address the agreement in the telco;

Disposition:

- The group agreed to include references to the tables 1 and 2 from TR 36.763 in the permanent document, rather than including the entire tables. This was suggested to ensure clarity and avoid redundancy.

- A revision is expected to address the agreements.

2		Proposal

It is proposed to add the following changes into Ptoc

* * * First Change * * *

# 2	References

[1]	SP-250378, "SID on Ultra Low Bitrate Speech Codec", China Mobile Com. Corporation, vivo, Fraunhofer IIS, Qualcomm Incorporated, Spreadtrum, Dolby Laboratories Inc., Xiaomi, Huawei, 2025.

[26132]	3GPP TS 26.132: “Speech and video telephony terminal acoustic test specification”.

[38811]	3GPP TR 38.811, “Study on New Radio (NR) to support non-terrestrial networks”.

[38821]	3GPP TR 38.821, “Solutions for NR to Non-Terrestrial Networks (NTN)”

[36321] 	TR 36.321, "Evolved Universal Terrestrial Radio Access (E-UTRA); Medium Access Control (MAC) protocol specification"

[36763] 	3GPP TR 36.763, “Study on Narrow-Band Internet of Things (NB-IoT) / enhanced Machine Type Communication (eMTC) support for Non-Terrestrial Networks (NTN)”.

* * * Second Change (new clause) * * *

#### 5.2.1.4	Link budget results for C/N range from RAN1 work

TR 36.763 [36763] and TR 38.821 [38821] specify the satellite parameters for various scenarios, as named as Set-1, Set-2, Set-3, Set-4, and Set-5. Among the all Sets, only Set-1/2/3 are relevant to GEO satellite case, as addressed in Table 5.2.1.4-1.

Table 5.2.1.4-1: GEO satellite parameters for simulation calibration (based on clause 6.1.1.1 of TR 38.821, clause 6.2.1 of TR 36.763)

| GEO Satellite orbit |  | Set-1 | Set-2 | Set-3 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 35786 km | 35786 km |
| Satellite antenna pattern |  | Clause 6.4.1 in TR 38.811 | Clause 6.4.1 in TR 38.811 | Clause 6.4.1 in TR 38.811 |
| Central beam edge elevation |  | 2.3 degrees | 11.0 degrees | 12.5 degree |
| Central beam centre elevation |  | 12.5 degrees | 20 degrees | 20.9 degree |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture | S-band (i.e. 2 GHz) | 22 m | 12 m | 12 m |
| Satellite EIRP density |  | 59 dBW/MHz | 53.5 dBW/MHz | 59.8 dBW/MHz |
| Satellite Tx max Gain |  | 51 dBi | 45.5 dBi | 45.7 dBi |
| 3dB beamwidth |  | 0.4011 deg | 0.7353 degrees | 0.7353 degree |
| Satellite beam diameter |  | 250 km | 450 km | 459km |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture | S-band (i.e. 2 GHz) | 22 m | 12 m | 12 m |
| G/T |  | 19 dB/K | 14 dB/K | 16.7 dB/K |
| Satellite Rx max Gain |  | 51 dBi | 45.5 dBi | 45.7 dBi |

Based on the parameters above, RAN1 had conducted the calibration works of link budget results from 15 contributing companies and agreed the link budget results as shown in Table 5.2.1.4-2.

Table 5.2.1.4-2: Link budget results for GEO satellite case (excerpted from clause 6.2.2 of TR 36.763)

| PC3 (23 dBm), NF=7 dB |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Cases | EIRP Density | EIRP per spot | DL C/N | G/T | UL C/N 180 kHz / 90 kHz / 45 kHz / 30 kHz / 15 kHz / 3.75 kHz |
| Set-1 | 59 dBW/MHz | 81.6 dBm | -3.3 dB | 19 dB/K | -14.2 dB / -11.2 dB / -8.2 dB / -6.5 dB / -3.5 dB / 2.6 dB |
| Set-2 | 53.5 dBW/MHz | 76.1 dBm | -8.5 dB | 14 dB/K | -19.0 dB / -16.0 dB / -13.0 dB / -11.2 dB / -8.2 dB / -2.2 dB |
| Set-3 | 59.8 dBW/MHz | 84.4 dBm | -2.2 dB | 16.7 dB/K | -16.3 dB / -13.3 dB / -10.3 dB / -8.5 dB / -5.5 dB / 0.6 dB |

| PC5 (20 dBm), NF=9 dB |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Cases | EIRP Density | EIRP per spot | DL C/N | G/T | UL C/N 180 kHz / 90 kHz / 45 kHz / 30 kHz / 15 kHz / 3.75 kHz |
| Set-1 | 59 dBW/MHz | 81.6 dBm | -5.3 dB | 19 dB/K | -17.2 dB / -14.2 dB / -11.2 dB / -9.5 dB / -6.5 dB / -0.4 dB |
| Set-2 | 53.5 dBW/MHz | 76.1 dBm | -10.5 dB | 14 dB/K | -22.0 dB / -19.0 dB / -16.0 dB / -14.2 dB / -11.2 dB / -5.2 dB |
| Set-3 | 59.8 dBW/MHz | 84.4 dBm | -4.2 dB | 16.7 dB/K | -19.3 dB / -16.3 dB / -13.3 dB / -11.5 dB / -8.5 dB / -2.4 dB |

The table shows the C/N values for device type with PC3 or PC5 and the noise figure of 7 dB or 9 dB. For channel bandwidth, NB-IoT 180kHz for DL, and up to 180kHz with all permissible smaller resource allocations for UL are assumed.

In summary, the following ranges of C/N are specified in TR 36.763 [36763].

-	For DL, C/N range : -10.5 dB (min) ~ -2.2 dB (max)

-	For UL, C/N range : -22.0 dB (min) ~ +2.6 dB (max)