Source:	Xiaomi Technology Spain S.L

Title:	Database of target device 1 for scenario 1 and 2

## Document for:	Discussion &Agreement

## Agenda Item:	1.7

# Introduction

Recording scenarios have been updated based on the contributions and agreements from the SA4#132 meeting[1]. This contribution aiming to update the database for the recording scenario 1 and 2. The corresponding recording signal for target device 1 in [2] can be access though the link at the end of the document.

# Content

To get the high-quality signal both scenarios are recorded in the anechoic, the recording setup as follows table:

| No | 1 | 2 |
| --- | --- | --- |
| Target device No | 1 | 1 |
| Recording scenario No | 1 | 2 |
| File name | e.g. SINE_Mic1.wav | SWEEP_[Orientation]_[ele]deg_[azi]deg.wav e.g. SWEEP_Horizontal_+30deg_30deg.wav |
| UE orientation / placement | Placement: centre of anechoic chamber Orientation: up(mic1,mi2,mic3), horizontal(mic4) | Placement: centre of anechoic chamber Orientation: horizontal, vertical and up |
| Sample rate | 48 kHz | 48 kHz |
| Bit depth | 16bit | 16 bit |
| Audio format | .wav | .wav |
| Rec Duration | 70s for each file, first and last 30s is silence | 12s for each file, first and last 1s is silence |
| Number of channels and microphone mapping: | 4 channels, ch1=mic1, ch2=mic2, ch3=mic3, ch4=mic4 | 4 channels, ch1=mic1, ch2=mic2, ch3=mic3, ch4=mic4 |
| Description/additional info | Sound pressure level 94 dB SPL at the measured mic positions. Distance between UE and sound source was 0.6m. GENELEC 8351BP-5 was used for the measurement | Source distance: d_UE = 3m Speaker: GENELEC 8351BP-5 Height of UE and sound source: H=1.2m Recording angles (azi, ele): - Azimuth Φ step of 10°, Φ = [0°, 10°, …,360°] for all 3 orientations - For every 3rd azimuth step (Φ = [0°, 30°, …, 330°]), elevations θ with step of 30°: θ = [-60°, -30°, 0°, 30°, 60°], for horizontal. Source signal: -filename: exponential sweep.wav -Frequency range: 20Hz-20kHz -Length: 10s -Level: -18 dBFS RMS -Sample rate: 96 kHz -Bit depth: 24bit -Fade-in and fade-out be implemented. |

## UE position for scenario 1

The main diaphragm of the speaker faces the measured microphone inlet, with the speaker and UE positioned as shown in the figure. Although microphones 1 and 2 are positioned differently during the recording, they are too close to identify any differences from the image. Therefore, only the picture for mic1 is provided.

Figure 1: Picture of the recording scenario 1 for mic 1

Figure 2: Picture of the recording scenario 1 for mic 3

Figure 3: Picture of the recording scenario 1 for mic 4

## Placement of UE for scenario 2

The UE is mounted on the turntable to rotate and capture all azimuth angles. To reduce reflection, the surface of the turntable and mount has been acoustically treated with sound-absorbing materials.

Figure 4: Picture of the recording scenario 2 at azimuth 0° elevation 0°

The placement of the 3 orientations are as follows:

Figure 5: Picture of the horizontal orientation at azimuth 0° elevation 0°

Figure 6: Picture of the vertical orientation at azimuth 0° elevation 0°

Figure 7: Picture of the up orientation at azimuth 0° elevation 0°

The turntable's elevation angle is adjusted via a bracket preset to 30° or 60°as the following:

Figure 8: Picture of the  elevation 60°

Five reference recordings (-60°, -30°, 0°, 30°, 60°) are made at the UE position as the turntable is adjusted to different elevation angles.

# Conclusion

This document proposed recording database comprises scenario 1 and 2 for target device 1 in[2]. The database has been uploaded to the previous path[4] in the folder ‘scenario_1’ and ‘scenario_2’:

Link:

https://kpan.mioffice.cn/webfolder/ext/zh%24DE%24XejVH%24uVm31GQvyw%40%40?n=0.1314974258223881

password: 0x7o

## References

- S4-251057: DaCAS-1 Target devices and databases v0.1

- S4aA250006: Proposal of a target device

- S4-250823: Exponential sweep database for target device 1

- S4-250643:Adding recording scenarios and database