Source:	Nokia Corporation

Title:	Spatial ACR listening test results

Document for:	Agreement

Agenda Item:	1.5 – IVAS_Codec_Ph2

# 1. Introduction

IVAS Permanent Document IVAS-8b - Test Plan for Characterization Phase [1], discuss specifics of IVAS Codec characterization testing. Spatial multi-bandwidth ACR listening test was proposed to be included in the test plan in document [2]. In this document the source provides listening test results, which used the specified test material categories and subset of conditions as included in the [1].

# 2. Overview

In the current work in progress IVAS Permanent Document IVAS-8b - Test Plan for Characterization Phase [1], one ACR test has been added, currently in brackets. The source proposed specific test material and conditions that can be used to characterize IVAS codec with ACR test methodology [2]. Further information was requested about the suitability of the ACR test methodology with spatial multi-bandwidth test material with naïve listeners. The source conducted such a listening test with reduced number of listeners and conditions. The results are provided in Chapter 4. The source also noted that Instructions for conducting spatial ACR listening testing is missing from the Test Plan document. The proposal for instructions is in Chapter 3.

# 3. Instructions for ACR testing

The current test plan document [1] is missing instructions for spatial multi-bandwidth ACR listening testing. The following text is provided to be included as Annex A.4 in [1]. Text is strongly based on DCR instructions and guidance given in [3].

INSTRUCTIONS TO NAÏVE LISTENERS FOR P.800 ACR TEST In this experiment you will be evaluating systems that might be used for future immersive telecommunication services using spatial audio. Spatial audio means that you can locate various sound sources around yourself. For example, a first talker may appear to talk from the left-hand side and a second talker from the right-hand side, a talker can be moving, etc. In each trial, you will hear a test sample. The test sample is possibly impaired after it has passed through a telecommunication system. Your task is to evaluate the overall quality of the test sample, comprising both degradations in the sound quality (e.g., harshness / roughness, loss of bandwidth, unnatural speech, clicks or other distortions), and/or lack of quality in the spatial representation (e.g., unnatural sound source movement, spatial instability, lack of spatial width or other spatial problems). You should listen carefully the sample within a trial. When sample playback has finished, select the category that best describes your overall impression about the overall quality of the sample you just heard: 5 - Excellent 4 - Good 3 - Fair 2 - Poor 1 - Bad Note that the level of quality present in different test samples is expected to span the complete range of the rating scale during the experiment. Please do not discuss your opinions with other listeners participating in the experiment. If you have any questions, please ask the test administrator.

P.800 Voting screen (after playback of sample pair)

Please rate the OVERALL QUALITY of the sample: 5 - Excellent 4 - Good 3 - Fair 2 - Poor 1 - Bad

# 4. Spatial multi-bandwidth ACR listening test results

Nokia has performed a shortened version of the proposed ACR listening test. The main difference to the proposed test plan is the reduced number of conditions, additionally each test signal category had only 4 samples instead of 6. There was a total of 12 naïve listeners in the listening test. Each listener was familiarized with the full quality test material, by listening trough the samples in Direct FB quality prior to the actual listening test. This familiarization stage took approximately 3 minutes (24*8 seconds). After familiarization 16 practice votes were cast before the actual listening test started. In the actual listening test each listener casted 216 votes. No listeners were removed from the results. Results can be seen in Figure 1 in bar format.

Figure 1 Spatial ACR listening test ACR MOS with 12 naïve listeners in bar format with confidence intervals.

As can be seen from the listening test results, signal bandwidth significantly affects the MOS scores. Clean spatial NB with 4 kHz signal bandwidth gets about MOS 2.3 and FB signal bandwidth gets MOS 4.2. IVAS with SWB/FB gets better results than IVAS-WB at similar bitrates. Frame erasures conditions with 5% FER (Frame Error Rate) get somewhat reduced scores as expected. At the highest tested bitrates IVAS-FB 96- 160 kbps is statistically equivalent to the Direct FB. Some statistical result fluctuation can be seen e.g. in 12, 14 and 16 kHz Direct signal results. With more listeners it is expected that the results span even more nicely.

Figure 2 shows the same results in curve format with logarithmic bitrate scale on the X-axis and ACR MOS score on the Y-axis. It is easy to see, that the subjective quality significantly increases with increasing bitrate and bandwidth.

Figure 2 Spatial ACR listening test ACR MOS with 12 naïve listeners in curve format

# 5. Summary

We have presented spatial ACR listening test results and propose for agreement of the ACR listening test instructions in clause 3 to be included as “A.4 P.800 test instructions for ACR testing” as well as removal of brackets around chapter “Annex H: P.800 ACR experiment” in [1].

# References

[1]	S4-251138 “IVAS Permanent Document IVAS-8b - Test Plan for Characterization Phase”, Version v.0.7.0

[2]	S4-251043 “ACR characterization of IVAS codec”, Fukuoka, Japan, 19-23 May 2025

[3]	ITU-T P.800 – Use cases, P Suppl. 29 (01/2023)