Source:	Fraunhofer IIS, Nokia, Orange, Ericsson LM, VoiceAge Corporation

Title:	Proposed Corrections to the IVAS Characterization Processing and Test plans (IVAS-7b, IVAS-8b)

Document for:	Discussion and Agreement

Agenda Item:	1.5

# Introduction

While reviewing and implementing the configurations and scripts for the IVAS Characterization Phase Processing Plan IVAS-7b [1] and Test Plan IVAS-8b [2], several inconsistencies, typos and other issues have been observed. This Tdoc lists these in order to be corrected.

# Observed Issues

- 2.1	Issue in IVAS-8b, P800-8

Test P800-8 is a multi-channel test which evaluates different CICP configuration. For categories 5 and 6 the test doesn’t clearly specify, which CICP configuration is tested. Since the samples within one category are concatenated before coding, mixing of different CICPs doesn’t easily work. It is thus proposed to harmonize the configuration within one category and change Table F.8.5 as follows:

Table F.8.5: Mixed content and Music categories

| Category | Type |
| --- | --- |
| cat 5 – 5.1 | music and mixed content |
| cat 6 – 7.1+4 | music and mixed content |

- 2.2	Issue in IVAS-8b, P800-11

Test P800-11 is an object-based audio test which evaluates different audio object configuration. For categories 5 and 6 the test doesn’t clearly specify, which audio configuration is tested. Since the samples within one category are concatenated before coding, mixing of different audio object configurations doesn’t easily work. It is thus proposed to harmonize the configuration within one category and change Table F.11.5 as follows. It is further proposed to allow for effects to be mixed with speech for category 6.

Table F.11.5: Categories for Speech with effects, and Speech with music or Music

| Category | Type |
| --- | --- |
| cat 5 | Speech + effects (3 objects) |
| cat 6 | Speech + music/effects or music only (4 objects) |

- 2.3	Issue in IVAS-8b, P800-12/13/14

Tests P800-12/13/14 are evaluating MASA. The current testplan specifies the usage of FOA input material. However, by restricting all three tests to FOA input only, this also restricts the tests to the evaluation of the MASA 1-dir configuration. In addition, the usage of FOA input material is unnecessary for cat. 5 and 6 (pre-produced content), since no background noise mixing is involved. It is thus proposed, to apply the following changes:

Table F.12.1: Conditions for Experiment P800-12

| Main Codec Conditions |  |
| --- | --- |
| Candidate | CuT IVAS FX, CuT IVAS FL |
| Bitrates | 13.2, 16.4, 24.4, 32, 48, 64, 80, 96, 128 kbps |
| DTX | DTX off |
| Input level | -16, -26, -36 LKFS, as defined in Table 5 |
| Input frequency mask | 20KBP |
| Background | Idle noise for cat 1,2, 15 dB for cat 3,4 |
| Error Conditions | 0% |
| References |  |
| Direct | -26 LKFS |
| P.50 MNRU ESDRU | Q = 17, 22, 27, 32 dB α = 0.2, 0.4, 0.6, 0.8 |
| Input frequency mask | 20KBP |
| Common Conditions |  |
| Test item generation: pre-processing incl. spatialization | Cat. 1-4: Model-based generation according to convolution of raw mono clean speech sentences with FOA/HOA2 Spatial Room Impulse Responses corresponding to the talker positions relative to a capture point and spatial FOA/HOA2 background. MASA format generation from FOA/HOA2 according to MASA analysis [14]; provision of FOA/HOA2 material according to IVAS-7b, clause 4.9. Cat. 5-6: Produced MASA 1TC material. |
| Binaural renderer | IVAS MASA internal binaural rendering |
| Audio sampling Frequency/bandwidth | 48 kHz/maximum available audio bandwidth up to FB |
| Kind of samples | Sentence pairs uttered by different talkers and genders (3 male and 3 female), mixed content and generic audio |
| Number of categories | 6 Different environments and talker interactions |
| Number of samples | 6 + 1 (preliminaries) samples per category |
| Listening Level | 73 dB SPL |
| Listeners | Naïve listeners |
| Randomizations | 6 panels of 5 listeners |
| Rating Scale | Following clause 4.2.1.1 |
| Listening System | Headphones, in accordance with clause 4.4 |
| Listening Environment | No room noise |

Table F.12.4: Clean and noisy speech categories and scene definitions

| Category | Environment(1 | Background(2 | SNR [dB] | Overtalk [s](3 | Bandwidth | Talker positions | Talker selection by panel |
| --- | --- | --- | --- | --- | --- | --- | --- |
| cat 1 (HOA2) | env_1_MASA | env_1_cleanbg_MASA | 45 | 1 | Max | (4 | P1: f1m1 P2: m2f2 P3: f3m3 P4: m1f1 P5: f2m2 P6: m3f3 |
| cat 2 (FOA) | env_2_MASA | env_2_cleanbg_MASA | 45 | -1 | Max | (4 | P1: m3f3 P2: f1m1 P3: m2f2 P4: f3m3 P5: m1f1 P6: f2m2 |
| cat 3 (FOA) | env_3_MASA | env_3_noisebg_1_MASA | 15 | -1 | Max | (4 | P1: f2m2 P2: m3f3 P3: f1m1 P4: m2f2 P5: f3m3 P6: m1f1 |
| cat 4 (FOA) | env_4_MASA | env_4_noisebg_1_MASA | 15 | -1 | Max | (4 | P1: m1f1 P2: f2m2 P3: m3f3 P4: f1m1 P5: m2f2 P6: f3m3 |

Table  F.12.5: Mixed content and Generic audio categories

| Category | Type |
| --- | --- |
| cat 5 | mixed content, MASA 1 dir. |
| cat 6 | generic audio, MASA 2 dir. |

Table F.13.1: Conditions for Experiment P800-13

| Main Codec Conditions |  |
| --- | --- |
| Candidate | CuT IVAS FX, CuT IVAS FL |
| Bitrates | 16.4, 24.4, 32, 48, 64, 96, 128, 192, 384 kbps |
| DTX | DTX off |
| Input level | -16, -26, -36 LKFS, as defined in Table 5 |
| Input frequency mask | 20KBP |
| Background | Idle noise for cat 1,2, 15 dB for cat 3,4 |
| Error Conditions | 0% |
| References |  |
| Direct | -26 LKFS |
| P.50 MNRU ESDRU | Q = 17, 22, 27, 32 dB α = 0.2, 0.4, 0.6, 0.8 |
| Input frequency mask | 20KBP |
| Common Conditions |  |
| Test item generation: pre-processing incl. spatialization | Cat. 1- 4: Model-based generation according to convolution of raw mono clean speech sentences with FOA/HOA2 Spatial Room Impulse Responses corresponding to the talker positions relative to a capture point and spatial FOA/HOA2 background. MASA format generation from FOA/HOA2 according to MASA analysis [14]; provision of FOA/HOA2 material according to IVAS-7b, clause 4.9. Cat. 5-6: Produced MASA 1TC material. |
| Binaural renderer | IVAS MASA internal binaural rendering |
| Audio sampling Frequency/bandwidth | 48 kHz/maximum available audio bandwidth up to FB |
| Kind of samples | Sentence pairs uttered by different talkers and genders (3 male and 3 female), mixed content, generic audio |
| Number of categories | 6 Different environments and talker interactions |
| Number of samples | 6 + 1 (preliminaries) samples per category |
| Listening Level | 73 dB SPL |
| Listeners | Naïve listeners |
| Randomizations | 6 panels of 5 listeners |
| Rating Scale | Following clause 4.2.1.1 |
| Listening System | Headphones, in accordance with clause 4.4 |
| Listening Environment | No room noise |

Table F.13.4: Clean and noisy speech categories and scene definitions

| Category | Environment(1 | Background(2 | SNR [dB] | Overtalk [s](3 | Bandwidth | Talker positions | Talker selection by panel |
| --- | --- | --- | --- | --- | --- | --- | --- |
| cat 1 (HOA2) | env_1_MASA | env_1_cleanbg_MASA | 45 | 1 | Max | (4 | P1: f1m1 P2: m2f2 P3: f3m3 P4: m1f1 P5: f2m2 P6: m3f3 |
| cat 2 (FOA) | env_2_MASA | env_2_cleanbg_MASA | 45 | -1 | Max | (4 | P1: m3f3 P2: f1m1 P3: m2f2 P4: f3m3 P5: m1f1 P6: f2m2 |
| cat 3 (FOA) | env_3_MASA | env_3_noisebg_1_MASA | 15 | -1 | Max | (4 | P1: f2m2 P2: m3f3 P3: f1m1 P4: m2f2 P5: f3m3 P6: m1f1 |
| cat 4 (FOA) | env_4_MASA | env_4_noisebg_1_MASA | 15 | -1 | Max | (4 | P1: m1f1 P2: f2m2 P3: m3f3 P4: f1m1 P5: m2f2 P6: f3m3 |

Table F.13.5: Mixed content and Generic audio categories

| Category | Type |
| --- | --- |
| cat 5 | mixed content, MASA 1 dir. |
| cat 6 | generic audio, MASA 2 dir. |

Table F.14.1: Conditions for Experiment P800-14

| Main Codec Conditions |  |
| --- | --- |
| Candidate | CuT IVAS FX, CuT IVAS FL |
| Bitrates | 13.2, 16.4, 24.4, 32, 48, 64, 80, 96, 128 kbps |
| DTX | DTX on |
| Input level | -16, -26, -36 LKFS, as defined in Table 5 |
| Input frequency mask | 20KBP |
| Background | Idle noise for cat 1,2, 15 dB for cat 3,4 |
| Error Conditions | 4, 8% |
| References |  |
| Direct | -26 LKFS |
| P.50 MNRU ESDRU | Q = 18, 25, 32 dB α = 0.4, 0.6, 0.8 |
| Input frequency mask | 20KBP |
| Common Conditions |  |
| Test item generation: pre-processing incl. spatialization | Model-based generation according to convolution of raw mono clean speech sentences with FOA/HOA2 Spatial Room Impulse Responses corresponding to the talker positions relative to a capture point and spatial FOA/HOA2 background. MASA format generation from FOA/HOA2 according to MASA analysis [14]; provision of FOA/HOA2 material according to IVAS-7b, clause 4.9. Cat. 5-6: Produced MASA 1TC material. |
| Binaural renderer | IVAS MASA internal binaural rendering |
| Audio sampling Frequency/bandwidth | 48 kHz/maximum available audio bandwidth up to FB |
| Kind of samples | Sentence pairs uttered by different talkers and genders (3 male and 3 female), mixed content, generic audio |
| Number of categories | 6 Different environments and talker interactions |
| Number of samples | 6 + 1 (preliminaries) samples per category |
| Listening Level | 73 dB SPL |
| Listeners | Naïve listeners |
| Randomizations | 6 panels of 5 listeners |
| Rating Scale | Following clause 4.2.1.1 |
| Listening System | Headphones, in accordance with clause 4.4 |
| Listening Environment | No room noise |

Table F.14.4: Clean and noisy speech categories and scene definitions

| Category | Environment(1 | Background(2 | SNR [dB] | Overtalk [s](3 | Bandwidth | Talker positions | Talker selection by panel |
| --- | --- | --- | --- | --- | --- | --- | --- |
| cat 1 (HOA2) | env_1_MASA | env_1_cleanbg_MASA | 45 | 1 | Max | (4 | P1: f1m1 P2: m2f2 P3: f3m3 P4: m1f1 P5: f2m2 P6: m3f3 |
| cat 2 (FOA) | env_2_MASA | env_2_cleanbg_MASA | 45 | -1 | Max | (4 | P1: m3f3 P2: f1m1 P3: m2f2 P4: f3m3 P5: m1f1 P6: f2m2 |
| cat 3 (FOA) | env_3_MASA | env_3_noisebg_1_MASA | 15 | -1 | Max | (4 | P1: f2m2 P2: m3f3 P3: f1m1 P4: m2f2 P5: f3m3 P6: m1f1 |
| cat 4 (FOA) | env_4_MASA | env_4_noisebg_1_MASA | 15 | -1 | Max | (4 | P1: m1f1 P2: f2m2 P3: m3f3 P4: f1m1 P5: m2f2 P6: f3m3 |

Table F.14.5: Mixed content and Generic audio categories

| Category | Type |
| --- | --- |
| cat 5 | mixed content, MASA 1 dir. |
| cat 6 | generic audio, MASA 2 dir. |

- 2.4	Issue in IVAS-8b, P800-18/19/20

Tests P800-18/19/20 evaluate OMASA at different configurations. The test plan doesn’t specify the exact MASA configuration (number of directions). Also, the term “defined scenes” is misleading, since no mixing of OMASA metadata can easily be implemented. It is thus proposed to change this to “preproduced scenes”. To correct this, the following modifications are proposed:

Table F.18.1: Conditions for Experiment P800-18

| Main Codec Conditions |  |
| --- | --- |
| Candidate | CuT IVAS FX, CuT IVAS FL |
| Bitrates | 13.2, 16.4, 24.4, 32, 48, 64, 80, 128, 256 kbps |
| DTX | DTX off |
| Input level | -16, -26, -36 LKFS, as defined in Table 5 |
| Input frequency mask | 20KBP |
| Background | 15 dB for cat 1,2,3,4 |
| Error Conditions | 0% |
| References |  |
| Direct | -26 LKFS |
| P.50 MNRU ESDRU | Q = 17, 22, 27, 32 dB α = 0.2, 0.4, 0.6, 0.8 |
| Input frequency mask | 20KBP |
| Common Conditions |  |
| Test item generation: pre-processing incl. spatialization | Cat. 1-2: Preproduced scenes according to Table F.18.4, 1 ISM + MASA 2TC background, MASA 2 dir. Cat. 3-4: Preproduced scenes according to Table F.18.4, 2 ISMs + MASA 2TC background, MASA 2 dir. Cat. 5-6: Pre-produced content, MASA 2 dir. |
| Binaural renderer | OMASA to binaural internal rendering |
| Audio sampling Frequency/bandwidth | 48 kHz/maximum available audio bandwidth up to FB |
| Kind of samples | Sentence pairs uttered by different talkers and genders (3 male and 3 female) |
| Number of categories | 6 Different environments with background and talker interactions |
| Number of samples | 6 + 1 (preliminaries) samples per category |
| Listening Level | 73 dB SPL |
| Listeners | Naïve listeners |
| Randomizations | 6 panels of 5 listeners |
| Rating Scale | Following clause 4.2.1.1 |
| Listening System | Headphones, in accordance with clause 4.4 |
| Listening Environment | No room noise |

Table F.19.1: Conditions for Experiment P800-19

| Main Codec Conditions |  |
| --- | --- |
| Candidate | CuT IVAS FX, CuT IVAS FL |
| Bitrates | 16.4, 24.4, 48, 64, 80, 96, 128, 256, 512 kbps |
| DTX | DTX off |
| Input level | -16, -26, -36 LKFS, as defined in Table 5 |
| Input frequency mask | 20KBP |
| Background | About 15 dB |
| Error Conditions | 0% |
| References |  |
| Direct | -26 LKFS |
| P.50 MNRU ESDRU | Q = 17, 22, 27, 32 dB α = 0.2, 0.4, 0.6, 0.8 |
| Input frequency mask | 20KBP |
| Common Conditions |  |
| Test item generation: pre-processing incl. spatialization | Cat. 1-3: Pre-produced content, 3 ISMs + MASA 2TC background, MASA 2 dir. Cat. 4-6: Pre-produced content, 4 ISMs + MASA 2TC background, MASA 2 dir. |
| Binaural renderer | OMASA to binaural internal rendering |
| Audio sampling Frequency/bandwidth | 48 kHz/maximum available audio bandwidth up to FB |
| Kind of samples | ISMs with MASA background |
| Number of categories | 6 |
| Number of samples | 6 + 1 (preliminaries) samples per category |
| Listening Level | 73 dB SPL |
| Listeners | Naïve listeners |
| Randomizations | 6 panels of 5 listeners |
| Rating Scale | Following clause 4.2.1.1 |
| Listening System | Headphones, in accordance with clause 4.4 |
| Listening Environment | No room noise |

Table F.20.1: Overview of test conditions

| Main Codec Conditions |  |
| --- | --- |
| Candidate | CuT IVAS FX, CuT IVAS FL |
| Bitrates | 13.2, 16.4, 24.4, 32, 48, 64, 80, 96, 128, 160, 192, 256, 384, 512 kbps |
| DTX | DTX off |
| Input level | -16, -26, -36 LKFS, as defined in Table 5 |
| Input frequency mask | 20KBP |
| Background | About 10 dB |
| Error Conditions | 0%, 5% |
| References |  |
| Direct | -26 LKFS |
| P.50 MNRU ESDRU | Q = 22, 26, 30, 34 dB α = 0.2, 0.4, 0.6, 0.8 |
| Input frequency mask | 20KBP |
| Common Conditions |  |
| Test item generation: pre-processing incl. spatialization | Pre-produced content of defined scenes, ISMs + MASA 2TC, MASA 2 dir. |
| Binaural renderer | OMASA to binaural internal rendering |
| Audio sampling Frequency/bandwidth | 48 kHz/maximum available audio bandwidth up to FB |
| Kind of samples | Sentence pairs uttered by different talkers and genders (3 male and 3 female) with scene background |
| Number of categories | 6 |
| Number of samples | 6 + 1 (preliminaries) samples per category |
| Listening Level | 73 dB SPL |
| Listeners | Naïve listeners |
| Randomizations | 6 panels of 5 listeners |
| Rating Scale | Following clause 4.2.1.1 |
| Listening System | Headphones, in accordance with clause 4.4 |
| Listening Environment | No room noise |

- 2.5	Issue in IVAS-8b, Usage of Renderer

The language in the experiment definition is not very precise with respect to the intended renderer, which is used for coded and reference conditions. It is proposed to clarify this for all affected experiments by replacing the term “internal rendering” as follows:

P800 experiments P800-3 to P800-20, P800-22 to P800-23 (Tables F.X.1):

| Binaural renderer | XXX to to binaural rendering using IVAS_rend for reference/MNRU/ESDRU conditions and using IVAS_dec for all IVAS-coded conditions. |
| --- | --- |

BS1534 experiments BS1534-1 to BS1534-2 (Tables G.X.1):

| Codec references |  |
| --- | --- |
| Codec references | … Diotic playback for mono conditions. |

BS1534 experiments BS1534-3 to BS1534-20 (Tables G.X.1):

| Main Codec Conditions |  |
| --- | --- |
| Codec under Test (CuT) | IVAS FX operated with XXX audio input at … Rendering via IVAS_dec |
| Codec references |  |
| Codec references | … Diotic playback for mono conditions. |
| Other references |  |
| Reference | Direct signal, Nominal input level, rendering via IVAS_rend |
| Hidden Reference | Direct signal, Nominal input level, rendering via IVAS_rend |
| LP3k5 anchor | 3.5 kHz lowpass filtered signal, nominal level, rendering via IVAS_rend |

- 2.6	Issue in IVAS-8b, Experiment BS1534-19 (LP3k5 Anchor)

Experiment BS1534-19 evaluates the performance of different approaches to coding and downmixing of stereo material. Codec conditions and codec references are compared against a stereo reference. However, also the lower anchor was assumed to be a 3.5 kHz lowpass-filtered stereo signal. Given that all other conditions are just monaural, this makes the grading of this lowpass-anchor very difficult for the subjects. It is thus proposed to exchange this for a static downmix signal, as follows:

Table G.19.1: Conditions (BS1534-19 Generic Audio)

| Main Codec Conditions |  |
| --- | --- |
| Codec under Test (CuT) | EVS encode-decode after stereo (dynamic) downmix 13.2 kbps, 24.4 kbps DTX off at 0% FER |
| Codec references |  |
| Codec references | Dual EVS encode-decode before stereo (static) downmix 7,2 kbps*2, 13.2 kbps*2, DTX off at 0% FER |
| Other references |  |
| Reference | EVS encode-decode after stereo (static) downmix 13.2 kbps, 24.4 kbps DTX off at 0% FER |
| Hidden Reference | Direct stereo signal, |
| LP3k5 anchor | 3.5 kHz lowpass filtered stereo (static) downmix signal, |
| Common Conditions |  |
| Test item generation | According to material collection procedure for IVAS selection BS.1534 tests. |
| Audio sampling frequency/bandwidth | 48 kHz/FB |
| Input frequency mask | 20KBP |
| Nominal output loudness | -26 LKFS |
| Listening Level | Adjusted by listener |
| Listeners | Experienced Listeners |
| Randomizations | Individual per listeners |
| Rating Scale | Continuous BS.1534 scale from 0-100 |
| Listening System | High-quality headphone for diotic presentation |
| Listening Environment | No room noise |

- 2.7	Issue in IVAS-7b, Preamble definition

The length of the preamble as used for the processing of the P800 conditions isn’t defined. It is proposed to use a preamble length of 10s, which is consistent with the processing used during IVAS selecting testing and in EVS. It is proposed to change clause 3.9 as following:

3.9 Preamble definition

The purpose of the preamble is to ensure that the CuT and reference systems are in proper warm-up state when processing the file portion comprising the samples to be evaluated. A preamble shall be uniformly distributed low-level noise obtained as 16-bit values with amplitudes between +4 and -4. Uncorrelated noise shall be used across audio tracks. The preamble for concatenation according to clause 4.1.2 will be 10 seconds long.

- 2.8	Issue in IVAS-7b, removal of obsolete text on reference renderer

The text on Reference Renderer is now obsolete and will not be used for the IVAS Characterization testing. It is therefore proposed to delete the complete clause 5.1.9 (Rendering with Reference Renderer) from IVAS-7b.

- 2.9	Issue in IVAS-7b, Generation of P.800 speech samples

IVAS-7b, clause 4.2.1 mandates the addition of uncorrelated noise between the audio tracks for content which was created according to the described model. This is however not strictly necessary, since according to the experiment definition in IVAS-8b, most formats (with the exception of ISM) are anyway mixed with background noise. It is thus proposed to relax this condition as follows:

4.2.1	P.800 speech samples

(…)

For P.800 experiments except for categories of pre-produced content, low-level noise comprising artificial random sequence with amplitude [-4, +4], uncorrelated between the audio tracks, may be added to the samples. The purpose is to avoid unrealistic digital silence and to simulate a very low-level recording noise, which can be expected in real recordings.

- 2.10	Issue in IVAS-7b, Inconsistent scaling for OSBA and OMASA

A scaling factor of 0.5 was earlier added to processing of OSBA reference conditions. However, it was still not decided how to handle OMASA conditions. In order to maintain the same scaling for the components of the combined formats (ISM, SBA/MASA), it is proposed to remove the scaling factor from OSBA again and get a consistent handling as for OMASA. It is proposed to update IVAS-7b as follows:

### 4.11.1	Reference conditions

# […]

### 4.11.3	Reference codec conditions

For IVAS codec reference conditions with two separate instances (MASA+ISM) see clause 4.9 and 4.10.

[…]

### 4.12.1	Reference conditions

Figure 23: Processing for OSBA reference conditions

- 2.11	Issue in IVAS-7b, reference to processing scripts still in brackets

The processing scripts are now complete and although further bug fixes are not excluded, the brackets in IVAS-7b should be removed. It is proposed to update IVAS-7b as follows:

# 1	Introduction

This document describes the processing for the characterization tests of the 3GPP IVAS codec. It should be read in conjunction with the associated IVAS characterization phase test plan, IVAS-8b [1]. The corresponding processing scripts, which shall be used for the processing, are available at 3GPP Forge [11]. The latest available version of the scripts shall be used. Although the descriptions and the processing scripts should match, in case of any discrepancy, the processing scripts prevail over the descriptions in this document. Unless otherwise specified, processing steps are performed using built-in Python functions or as available in common Python libraries.

- 2.12	Issue in IVAS-7b, naming convention missing for listening lab raw material

As discussed over the Audio SWG reflector, a naming convention for raw audio material provided by the labs was missing. It is proposed to update IVAS-7b as follows:

#### 3.10.2.x	Raw input files provided by listening laboratories

lnneeeayszz.wav

where:

- l stands for the listening lab designator (a through m according to Table 2)

- nn stands for optional language designator, e.g. en for English

- eee stands for the experiment designator, e.g. p01 (see Table 1)

- a stands for audio, where:

  - “m” is used for male talker

  - “f” is used for female talker

  - “a” is used for stereo mixed and music content

- y stands for talker/category designation, where:

  - y = 1, 2, or 3 for talkers

  - y = 1 to 6 based on category for experiment

- s stands for sample and zz is the sample number

- 2.13	Issue in IVAS-7b, inconsistent version of the EVS reference codec

As basis for the IVAS fixed-point conversion, it was agreed to use TS 26.452 (the alternative fixed-point code). However, in the processing plan for the characterization testing, 26.442 is referenced. It is proposed to use the same EVS codec version for testing as the basis for the conversion. It is proposed to update IVAS-7b as follows:

      - A.2.2.1	EVS

| Source | TS 26.452: Codec for Enhanced Voice Services (EVS) |
| --- | --- |
| URL | https://www.3gpp.org/ftp/Specs/archive/26_series/26.452/26452-i00.zip |
| Version / Release | 18.0.0 |
| Description | EVS encoder and decoder software |
| Comments |  |
| Executables | EVS_cod.exe, EVS_dec.exe |
| Status | Available |

- 2.14	Issue in IVAS-7b, MATLAB scripts for room acoustics tests

There is an editor’s note in the processing plan concerning the availability of scripts for room acoustics experiments. The scripts have been made available in a merge request, which means the note can be removed. It is proposed to update IVAS-7b as follows:

#### 5.1.12.1	BRIR synthesis

Binaural room impulse responses (BRIR) are synthesized using the Binaural Spatial Decomposition Method (SDM) by combining recorded spatial room impulse responses and a (default) HRIR dataset. To generate the BRIR dataset with Binaural SDM, update the BinauralSDM_for_IVAS.m MATLAB script [11] with appropriate file paths for the recorded room impulse response files, room name, HRIR input, and BRIR output.

Execute the BinauralSDM_for_IVAS.m script to generate the BRIR dataset in SOFA format. Finally, convert the resulting BRIR dataset to the binary format as specified in [16], clause 5.10.

- 2.15	Issue in IVAS-7b, ACR low-pass conditions processing in brackets

The processing of ACR low-pass conditions, not supported by STL filter tool, is still within brackets. It is proposed to update IVAS-7b as follows:

#### 5.2.3.2	ACR low-pass conditions

P.800 ACR experiment requires several low-pass conditions.

Low-pass filtering is using ITU-T STL filter tool for frequencies 10, 12, and 14kHz (LP10, LP12, and LP14). For missing low-pass filters (4, 6, 8, and 16 kHz) following Python based processing is used.

# Design a Chebychev Type II  filter, band_pass-band_stop = 500 Hz

N, Wn = sig.cheb2ord(fc / (fs / 2), (fc + 500) / (fs / 2), 3, 60)

b, a = sig.cheby2(N, 60, Wn, "low")

y = sig.filtfilt(b, a, x, axis=0)  # zero-phase filter, batch processing

# Conclusion

It is proposed to correct the issues listed in clause 2 of this Tdoc for the next revision of IVAS IVAS-7b and IVAS-8b.

# References

- 3GPP S4-151487, “IVAS-7b: Processing plan for characterization phase, v1.0.0”

- 3GPP S4-251484, “IVAS Permanent Document IVAS-8b: Test Plan for Characterization Phase, v1.0.0”