3GPP TSG-SA4 Meeting #-SA4-e (AH) Audio SWG post 133-e	S4aA250095

# Online, , 8th Sep 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.253 | CR | 0022 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Audio focus level and forward indication for IVAS RTP PI data |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | IVAS_Codec |  |  |  |  |  | Date: |  |  | 2025-09-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Indicating the level of requested audio focus would improve the feature and would enable the receiver to have better control of the requested audio focus. A forward indication in the applied audio focus is useful for a receiver to adjust the requested audio focus accordingly and to monitor what kind of audio focus the sender has applied to the transmitted audio. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Audio focus level field is added to the audio focus PI data. The audio focus PI data is renamed from AUDIO_FOCUS_DIRECTION to AUDIO_FOCUS_REQUEST. A forward direction variant is added to the audio focus PI data type (AUDIO_FOCUS_INDICATION). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not possible to negotiate initial noise suppression mode for an IVAS session. The receiver is unaware of what level of suppression is applied at the start of the session. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | CR26253-0016 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

CHANGE 1

### A.3.5.5	Supported PI data types

Supported PI types are listed in tables A.3.5.5-1 and A.3.5.5-2 and described in the following subsections. Table A.3.5.5-1 lists PI types for forward direction signalling. Table A.3.5.5-1A lists the PI types for reverse direction signalling. Table A.3.5.5-2 lists additional PI types.

Table A.3.5.5-1: Supported forward direction PI types in an IVAS session.

| Type bits | Forward direction PI type | Description | SDP indication | Size (bytes) | Described in clause |
| --- | --- | --- | --- | --- | --- |
| 00000 | SCENE_ORIENTATION | Describes the orientation of a spatial audio scene in unit quaternions. | fsco | 8 | A.3.5.6.1.2 |
| 00001 | DEVICE_ORIENTATION_COMPENSATED | Describes the orientation of a device in unit quaternions. The orientation is compensated in the transmitted audio. | fdoc | 8 | A.3.5.6.1.3 |
| 00010 | DEVICE_ORIENTATION_UNCOMPENSATED | Describes the orientation of a device in unit quaternions. The orientation is not compensated in the transmitted audio. | fdou | 8 | A.3.5.6.1.3 |
| 00011 | ACOUSTIC_ENVIRONMENT | Selects and optionally describes the acoustic environment. | face | 1,5 or 8 | A.3.5.6.2 |
| 00100 | AUDIO_DESCRIPTION | Describes the content in the transmitted audio | faud | 1 to 5 | A.3.5.6.3 |
| 00101 | ISM_NUM | Indicates the number of ISM(s). | finm | 1 | A.3.5.6.4.2 |
| 00110 | ISM_ID | Indicates ID of each transported ISM. | fiid | Number of ISMs x 1 | A.3.5.6.4.3 |
| 00111 | ISM_GAIN | Describes gain factor for each ISM. | figa | Number of ISMs x 1 | A.3.5.6.4.4 |
| 01000 | ISM_ORIENTATION | Describes an orientation for each ISM. | fiso | Number of ISMs x 8 | A.3.5.6.4.5 |
| 01001 | ISM_POSITION | Describes a position of each ISM. | fipo | Number of ISMs x 6 | A.3.5.6.4.6 |
| 01010 | ISM_DISTANCE_ATTENUATION | Describes distance attenuation for all ISMs. | fida | 3 or Number of ISMs x 3 | A.3.5.6.4.7 |
| 01011 | ISM_DIRECTIVITY | Describes directivity for each ISM. | fidr | 2 or Number of ISMs x 2 | A.3.5.6.4.8 |
| 01100 | DIEGETIC_TYPE | Indicates if the audio is diegetic or non-diegetic. | fdit | 1 | A.3.5.6.5 |
| 01101 | AUDIO_FOCUS_INDICATION | Describes a direction of focus for the transmitted audio in Quaternions and/or a level for the focus. | fafi | 1, 8 or 9 | A.3.5.7.5.2 |

Table A.3.5.5-1A: Supported reverse direction PI types in an IVAS session.

| Type bits | Reverse direction PI type | Description | SDP indication | Size (bytes) | Described in clause |
| --- | --- | --- | --- | --- | --- |
| 10000 | PLAYBACK_DEVICE_ORIENTATION | Feedback. Describes the orientation of the playback device in Quaternions. | rpdo | 8 | A.3.5.7.1 |
| 10001 | HEAD_ORIENTATION | Feedback. Describes the head orientation of the listener in Quaternions. | rhor | 8 | A.3.5.7.2 |
| 10010 | LISTENER_POSITION | Feedback. Describes the position of the listener in 3D space. | rlip | 6 | A.3.5.7.3 |
| 10011 | DYNAMIC_AUDIO_SUPPRESSION | Describes receiver’s preference with respect to audio suppression | rdas | 2 | A.3.5.7.4 |
| 10100 | AUDIO_FOCUS_REQUEST | Describes a direction of interest for the listener in Quaternions and/or receiver’s preference with respect to audio focus level. | rafr | 1, 8 or 9 | A.3.5.7.5.1 |
| 10101 | PI_LATENCY | Round-trip latency for PI frames | rlat | 4 | A.3.5.7.6 |
| 10110 | R_ISM_ID | Indicates ID of each editing request ISM. | riid | Number of ISMs x 1 | A.3.5.7.7.2 |
| 10111 | R_ISM_GAIN | An editing request for gain factor for each ISM. | riga | Number of ISMs x 1 | A.3.5.7.7.3 |
| 11000 | R_ISM_ORIENTATION | An editing request for orientation for each ISM. | riso | Number of ISMs x 8 | A.3.5.7.7.4 |
| 11001 | R_ISM_POSITION | An editing request for position for each ISM. | ripo | Number of ISMs x 6 | A.3.5.7.7.5 |
| 11010 | R_ISM_DIRECTION | An editing request for direction for each ISM. | rido | Number of ISMs x 2 | A.3.5.7.7.6 |

Table A.3.5.5-2: Additional PI types in an IVAS session.

| Type bits | PI type | Description | SDP indication | Size (bytes) |
| --- | --- | --- | --- | --- |
| 01110, 01111, 11011-11110 | Reserved | - | - | - |
| 11111 | NO_PI_DATA | Indicates an empty PI data frame. | nopi | 0 |

CHANGE 2

#### A.3.5.7.5	Audio focus

##### A.3.5.7.5.1	Audio focus request

AUDIO_FOCUS_REQUEST PI frame indicates a direction of interest from the receiver (i.e., the listener) to the sender side. The focus direction is transmitted similarly as the orientation PI data, see clause A.3.5.6.1.1. The focus direction is relative to the frontal direction (w=0, x=1, y=0, z=0). Additionally, the PI frame indicates a desired amount of audio focus level with the Focus Level (FLVL) indicator of 4 bits, see Table A.3.5.7.5.1-1. The audio focus PI data frame structure with direction and level is presented in Figure A.3.5.7.5.1-1 where the total size of the PI frame is 9 bytes.

The FLVL takes values from 0 to 13 wherein the expected amount of audio focus is proportional (in approximate logarithmic domain) to the indicator value with 0 indication no audio focus and 13 indicating the maximum audio focus level supported by the media sender for the session. FLVL bits value of 1110 indicates a request for default audio focus level performed by the media sender. FLVL bits value of 1111 indicates no preference in the requested audio focus level.

AUDIO_FOCUS_REQUEST PI frame may include only the directional components (W, X, Y, Z) for the audio focus in which case the size of the PI frame is 8 bytes. In case only an audio focus direction request is received, the media sender should apply the latest applied audio focus level. If no audio focus has been applied previously during the session, the media sender should apply the default audio focus level.

AUDIO_FOCUS_REQUEST PI frame may include only the FLVL indication for the audio focus in which case the size of the PI frame is 1 byte (4 bits for FLVL value and 4 reserved bits). In case only an audio focus level request is received, the media sender should apply the latest applied audio focus direction. If no audio focus has been applied during the session, the media sender should ignore the audio focus request requesting only audio focus level.

The latest received AUDIO_FOCUS_REQUEST PI data is valid until a new AUDIO_FOCUS_REQUEST PI data is received.

0 1 2 3 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+ ¦ W ¦ X ¦ +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+ ¦ Y ¦ Z ¦ +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+ ¦ FLVL ¦ RES ¦ +-+-+-+-+-+-+-+-+

Figure A.3.5.7.5.1-1: Audio focus PI data frame with direction and level.

Table A.3.5.7.5.1-1: Audio Focus Level Indicator

| FLVL bits | Value | Description |
| --- | --- | --- |
| 0000 | 0 | No audio focus |
| 0001 | 1 | … |
| … | … | … |
| 1101 | 13 | Maximum audio focus supported by the media sender for the session |
| 1110 | - | Default audio focus |
| 1111 | - | No preference/No indication |

##### A.3.5.7.5.2	Audio focus forward indication

AUDIO_FOCUS_INDICATION PI frame describes the audio focus direction and level applied to the transmitted audio. The PI data follows the structure presented in Figure A.3.5.7.5.1-1. The FLVL field indicates the amount of applied audio focus according to Table A.3.5.7.5.1-1 and the description in clause A.3.5.7.5.1. In the forward indication audio focus PI type, the FLVL bit combination of 1111 is reserved for “No indication” in which case the applied audio focus level is undefined.

The media sender may create and transmit an audio focus forward indication PI data in response to a received audio focus request PI data when the requested audio focus is accepted and applied in the processing of the transmitted audio. The media sender may also send audio focus forward indication PI data to indicate the current audio focus status without receiving a corresponding audio focus request PI data.

The AUDIO_FOCUS_INDICATION PI type has the same size variants as the request counterpart with direction and level (9 bytes), direction only (8 bytes) and level only (1 byte).

If a direction only variant is received, the media receiver should assume that the applied focus level is the same as in a previously received audio focus indication during the session. If no audio focus level indication has been received during the session, the media receiver should assume that the applied audio focus level is the default level.

If a level only variant is received, the media receiver should assume that the audio focus direction is the same as in a previously received audio focus indication during the session. If no audio focus direction indication has been received during the session, the media receiver should ignore the level indication.

The latest received AUDIO_FOCUS_INDICATION is valid until a new AUDIO_FOCUS_INDICATION is received.

END OF CHANGES