3GPP Audio Telco Post TSG SA WG4 #133-e	S4aA250208

Erlangen, Germany, Sept 23-25, 2025

# Title:	On the additional TBS values for ULBC RAN simulation

Source:	Qualcomm Innovation Center Inc., vivo

Agenda item:	4.3

# Document for:	Agreement

# 1. Introduction

Some TBS values are agreed in the PD [1]. There were proposals for adding additional TBS values, and some of the proposals are included under the ‘open issues’ of [1], while others are not. To be consistent, the latest proposal from each company should be included.

# References

[1] S4-251585, Permanent Document FS_ULBC, 3GPP TSG SA WG4 #133-e, July 18-25, 2025.

# 2. Proposed changes to the PD [1]

# The change includes:

- update Dolby’s old proposal S4aA250076 by its updated proposal S4-251390

- add Qualcomm’s proposal: S4-251548

- add vivo’s proposal: S4aA250215

* * * First Change * * *

#### 5.2.2.4	Assumptions and Open Issues for NB-IoT GEO Simulation

This clause documents the assumptions and open issues that require further investigation.

| Issues | Description | Status |
| --- | --- | --- |
| 1. UE Power Class | Whether to use the specified 23 dBm power class for NTN NB-IoT or the broader range of power (e.g., 26, 29, 31, and 33 dBm) supported by commercial UEs remains undecided, as no consensus has been reached. | Pending: 37 dBm (Requires RAN’s input and confirmation) |
| 2. Latitude-Dependent Loss | Scintillation loss should be set to 2.2 dB or 0 dB, depending on latitude. As specified in TR 38.821, the impact becomes negligible and shall be consider to zero for beyond the ±20° latitude. | Solved: The simulation accounts for latitude-dependent loss using the X term. |
| 3. Elevation Angles | A proposal has been made to keeping both 2.3° and 12.5° elevation angles in the simulation to consider the worst-case scenario for maintaining acceptable quality. | Solved: The simulation accounts for elevation angles using the X term. |
| 4. UL/DL Guard Time | An assumption of a 1 millisecond (ms) guard time for UL/DL switching is used in link budget analyses. The feasibility of this duration may need to be confirmed by RAN. | Pending: Needs to be confirmed by RAN. |
| 5. Determine candidate TBS values | Whether to adopt the other potential TBS values need further verification. - Xiaomi's proposal: S4aA250035 - Fraunhofer's: S4aA250031 - Skylo’s proposal: S4-251540 - Dolby's proposal: S4-251390 - Huawei’s proposal: S4-251305 - Qualcomm’s proposal: S4-251548 - vivo’s proposal: S4aA250215 | Unsolved |
| 6.Approaches to select TBS | Three approaches were provided in S4aA250072, and requires further discussion. One of these approaches is described in detail in clause 5.2.2.4.1 | Unsolved |
| 7. Overall Simulation Methodology Description | There is a recognized need to have a high-level description of how the simulation would be run, including parameters to optimize and result parameters. | Unsolved: To be addressed after all simulation work is completed. |
| 8. Simulation Channel Model | NTN-TDL-C or NTN-TDL-C5 | Solved , NTN-TDL-C is used |
| 9. Protocol Overhead | Clarify packet header overhead for different combinations of user plane, control plane, and IP vs. non-IP.needs to be confirmed by RAN2 and SA2. | Pending: Needs to be confirmed by RAN. |
| 10. Repetition numbers | It was proposed to specify and report the number of repetitions in the simulation. | Solved |
| 11.RX G/T for downlink | A 3dB better value observed in the field compared to current assumptions by RAN. | Unsolved |