Source:	Orange

Title:	On complexity measurement for ULBC

## Document for:	Discussion

## Agenda Item:	4.4

- Introduction

The “Study on Ultra Low Bitrate Speech Codec” (FS_ULBC) has been approved at SA#107 [1], and the WID has been further revised in [2].

In the present Tdoc we address the following study objective from the WID description [2]:

3.	Identify the relevant design constraints for such a codec, in coordination with other WGs, including

-	Bit rates

-	Sample rate and audio bandwidth

-	Frame length

-	Complexity and memory demands

-	Algorithmic delay

-	Packet loss concealment (PLC)

-	Potential use of noise suppression as part of the codec

-	Discontinuous transmission including voice activity detection and comfort noise

-	Speech quality

-	Robustness to non-speech input

The present Tdoc was triggered by [3] and aims at providing crosschecking results from [3]. We discuss the proposal of counting FLOPs (or MACs) based on existing libraries.

- Report of measurements

We used the following setup:

- DAC codec: https://github.com/descriptinc/descript-audio-codec (latest commit c7cfc5d2647e26471dc394f95846a0830e7bec34)

  - Weights for model at 16 kHz: https://github.com/descriptinc/descript-audio-codec/releases/download/0.0.5/weights_16khz.pth

  - Weights for model at 24 kHz: https://github.com/descriptinc/descript-audio-codec/releases/download/0.0.4/weights_24khz.pth

  - Weights for model at 44.1kHz (only one tested): https://github.com/descriptinc/descript-audio-codec/releases/download/0.0.1/weights.pth

- ptflops 0.7.5: https://github.com/sovrasov/flops-counter.pytorch/

- thop 2.0.17: https://github.com/ultralytics/thop

In the following we report only figures for the 44.1k model.

The frame length (hop size) of DAC is 512 samples for the 44.1 model. In [3], complexity is reported for an input frame length of 20, 40, 80, 160, 320 ms. In the following we force an integral number of DAC frames by rounding down the input frame length to the nearest multiple of 512.

In Table 1 and Figures 1 and 2 we report measured Flops results when using default complexity profiling. Quantization is assumed to operate at the maximum bitrate (9 codebook stages).

Table 1: Flops for DAC (44.1k model) with default complexity profiling.

| Input shape | Full Model | Encoder | Quantizer | Decoder |
| --- | --- | --- | --- | --- |
| (1, 512) | 1.160 GFlops | 0.356 GFlops | 0.147 MFlops | 0.804 GFlops |
| (1, 1536) | 3.480 GFlops | 1.067 GFlops | 0.442 MFlops | 2.413 GFlops |
| (1, 3072) | 6.960 GFlops | 2.134 GFlops | 0.885 MFlops | 4.825 GFlops |
| (1, 6656) | 15.081 GFlops | 4.624 GFlops | 1.917 MFlops | 10.445 GFlops |
| (1, 13824) | 31.321 GFlops | 9.604 GFlops | 3.981 MFlops | 21.714 GFlops |

Figure 1: Flops for DAC encoder and decoder (44.1k model) with default complexity profiling.

Figure 2: Flops for full DAC model (44.1k model) with default complexity profiling.

In Table 2 and Figures 3 and 4 we report measured Flops results when adding custom hook functions to count parts that are “missing” in the default estimation (i.e., Snake activation, tanh layer, vector quantization outside projection/expansion operations). A complexity weight of 1 is counted for each floating-point operation (e.g., sin, exp, division). Quantization is assumed to operate at the maximum bitrate (9 codebook stages).

Table 2: Flops for DAC (44.1k model) with custom profiling hooks.

| Input | Full Model | Encoder | Quantizer | Decoder |
| --- | --- | --- | --- | --- |
| (1, 512) | 1.171 GFlops | 0.360 GFlops | 0.636 MFlops | 0.811 GFlops |
| (1, 1536) | 3.513 GFlops | 1.080 GFlops | 1.245 MFlops | 2.432 GFlops |
| (1, 3072) | 7.025 GFlops | 2.160 GFlops | 2.158 MFlops | 4.863 GFlops |
| (1, 6656) | 15.220 GFlops | 4.679 GFlops | 4.289 MFlops | 10.537 GFlops |
| (1, 13824) | 31.611 GFlops | 9.718 GFlops | 8.551 MFlops | 21.885 GFlops |

Figure 3: Flops for DAC encoder and decoder (44.1k model) with custom profiling hooks.

Figure 4: Flops for full DAC model (44.1k model) with custom profiling hooks.

Based on this measurement report one may raise questions on complexity weights to be used in Flops counting, if this complexity measure is considered appropriate for ULBC. Note that some libraries may count MACs as two FLOPs, others count flops but report them as MACs. In previous speech coding exercises in 3GPP SA4 we typically counted a MAC operation as one operation. The appropriate weight to be applied for specific operations (e.g. Snake activation, tanh) may be discussed.

When comparing with [3], it appears that the measured Flops reported in the present Tdoc do not match with [3]. This issue has been discussed offline and clarified to be due to different library versions for ptflops and thop. This issue should also be carefully considered if such tools are considered for ULBC complexity measurement.

## References

- 3GPP SP-250378, SID on Ultra Low Bitrate Speech Codec, Source: China Mobile Com. Corporation, vivo, Fraunhofer IIS, Qualcomm Incorporated, Spreadtrum, Dolby Laboratories Inc., Xiaomi, Huawei

- 3GPP SP-250635, Revised Study on Ultra Low Bitrate Speech Codec, Source : SA4

- 3GPP Tdoc S4-251333, [FS_ULBC] Considerations on measuring ULBC complexity, Source: Vivo