Source:	Ericsson LM, Dolby Laboratories Inc.

Title:	On assumptions for link budget analysis

Document for:	Discussion and Agreement

Agenda Item:	4.3

# 1	Introduction

As part of FS_ULBC, there is an objective to study channel characteristics and service-related dependencies, determining e.g. bitrates and loss/delay/jitter profiles. In relation to the link budget analysis, the current assumptions [1] are however too limited with respect to the scheduling schemes and require capabilities of the UEs which cannot be assumed to always be available.

# 2	Semipersistent Scheduling Schemes

The Rel-20 Work Item (WI) entitled “Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 4” [RP-251867], explicitly mentions as part of its objectives “Support of semi-persistent scheduling for NB-IoT-NTN for DL and UL data transmission for voice packets [RAN2, RAN1].” Thus, a close communication with RAN1 and RAN2 is required regarding the support of “semi-persistent scheduling for NB-IoT-NTN”.

In relation with it, the liaison statement (LS) sent to RAN1 and other working groups (WGs) in [R1-2505140], includes as part of the “excerpt_S4-251550_FS_ULBC_P,” two figures illustrating “DL and UL data transmission for voice packets” using “dynamic scheduling” (see Figure 5.2.2.3-1  in section 5.2.2.3) and “semi-persistent scheduling (SPS)” (see Figure 5.2.2.3-2 in section 5.2.2.3). On this matter, RAN1 provided the following responses on those figures as part of the LS-reply in [R1-2506541].

- RAN1/2 have not yet started the work on designing SPS. Therefore, RAN1 currently cannot confirm whether the example frame structure for SPS (related to Figure 5.2.2.3-2 and associated text) will be supported.

- Although the example Figure 5.2.2.3-1 is supportable by RAN1 specifications in most scenarios, it may not be supportable in the case where the cell is very large (e.g. >3000km), when the UE does not support TA report and the network does not support UE-specific K-offset. The example Figure 5.2.2.3-1 itself also requires the UE to be configured with two HARQ processes and with HARQ feedback disabled.

One important point to highlight is that some Companies in RAN1 tried to understand how the gap between DL (i.e., NPDSCH) and UL (NPUSCH Format 1) can be as small as 1 ms. From the discussions it was concluded that the figures depicted by SA4 rely on a “Timing Advance (TA) report” (which is an optional UE capability) such that the network-controlled scheduling offset known as “Koffset” can be based on a “UE_specific_Koffset”. In other words, the SA4 schemes assume that TA reports will be transmitted by the UE to the network such that the network can adjust the Koffset based on the reported TA (SA4 figures also assume a perfect/ideal correction such that the gap between DL and UL can be as small as 1 ms).

However, in our view, the support of a scheme for “data transmission for voice packets” should not be conditioned to a “TA report UE capability” which is an optional UE capability and more importantly is not desirable making mandatory for the support of “voice-over-GEO for NB-IoT NTN”. Thus, the schemes for supporting the transmission of voice packets should also account for the case where the “TA report UE capability” is not available, in which case the network-controlled scheduling offset is based on a “Cell_specific_Koffset”.

In this case, the gap between DL and UL should consider a margin for the network to handle the different TAs of different UEs in the NTN cell, such a margin can be given by the “Maximum differential delay” (i.e., essentially the difference between the max TA value in the cell and the min TA value in the cell). Table 1 illustrates side-by-side the schemes depicted in the SA4 LS based on a “UE_specific_Koffset” approach (i.e., TA report UE capability dependent), and a scheme based on “Cell_specific_Koffset” approach.

Table 1: Side-by-side comparison between the SA4 schemes based on a “UE_specific_Koffset” approach as depicted in R1-2505140, and a scheme based on “Cell_specific_Koffset” approach.

| Figure 5.2.2.3-1 as in R1-2505140, “An example frame structure for 80ms bundling period and dynamic scheduling”. | SPS scheme based on “Cell_specific_Koffset” approach, which does not depend on the “TA report UE capability”. Notes: The gap between DL and UL can consist of: - A “Processing time + DL-to-UL switching”: It can be discussed whether the time for decoding the DL transport block needs to be considered or not, but at least the time that a “half-duplex device” requires for switching from DL-to-UL shall be considered which is 1 ms. - The “Max differential delay” shall in our view be considered for the network to handle the different delays of different UEs in the NTN cell (~ 10.3 ms for GEO satellites in NB-IoT NTN). |
| --- | --- |
| Figure 5.2.2.3-2 as in R1-2505140, “An example frame structure for 80ms bundling period and SPS”. |  |

For the ongoing and future work in SA4, it is important to consider not only the “UE_specific_Koffset” approach as in the SA4 LS sent to RAN1 in R1-2505140 (schemes in left-hand side of Table 1 above), but also a “Cell_specific_Koffset” since e.g., the support SPS periodicities and achievable data rates will dependent on the assumed scheme.

The Rel-20 WID in RP-251867 states as one of its objectives: “Support of semi-persistent scheduling for NB-IoT-NTN for DL and UL data transmission for voice packets [RAN2, RAN1].” Thus, a close communication with RAN1 and RAN2 is required regarding the support of “semi-persistent scheduling for NB-IoT-NTN”.

The LS that SA4 sent to RAN1 and other WGs included one excerpt illustrating a “dynamic scheduling” scheme and an “SPS” scheme where the gap between DL and UL can be as small as 1 ms. RAN1 observed that the gap is so small because both schemes rely on a “TA report UE capability” and an ideal network-controlled scheduling offset adjustment (Koffset) based on a “UE_specific_Koffset” approach.

In relation to the previous observation, in our view, the support of a scheme for “data transmission for voice packets” should not be conditioned to a “TA report UE capability,” which is an optional UE capability and is not desirable making mandatory for the support of “voice-over-GEO for NB-IoT NTN”.

Thus, the schemes should also account for the case where the “TA report UE capability” is not available, in which case the network-controlled scheduling offset is based on a “Cell_specific_Koffset” approach. In this case, the gap between DL and UL should consider a margin for the network to handle the different TAs of different UEs in the NTN cell (i.e., “Max differential delay,” see right-side of Table 1).

The assumption on whether the scheme(s) will be based on a “UE_specific_Koffset” or “Cell_specific_Koffset”, will in some cases result e.g., in different supportable SPS periodicities and different achievable data rates.

SA4 to consider for the ongoing and future work on the “simulation assumptions for ULBC,” not only dynamic and SPS schemes that depend on a TA report UE capability “UE_specific_Koffset approach,” but also schemes that are not conditioned/dependent on it i.e., “Cell_specific_Koffset approach”.

# 3	Conclusion

Based on the discussion in the previous section we made the following observations:

Observation 1	The Rel-20 WID in RP-251867 states as one of its objectives: “Support of semi-persistent scheduling for NB-IoT-NTN for DL and UL data transmission for voice packets [RAN2, RAN1].” Thus, a close communication with RAN1 and RAN2 is required regarding the support of “semi-persistent scheduling for NB-IoT-NTN”.

Observation 2	The LS that SA4 sent to RAN1 and other WGs included one excerpt illustrating a “dynamic scheduling” scheme and an “SPS” scheme where the gap between DL and UL can be as small as 1 ms. RAN1 observed that the gap is so small because both schemes rely on a “TA report UE capability” and an ideal network-controlled scheduling offset adjustment (Koffset) based on a “UE_specific_Koffset” approach.

Observation 3	In relation to the previous observation, in our view, the support of a scheme for “data transmission for voice packets” should not be conditioned to a “TA report UE capability,” which is an optional UE capability and is not desirable making it mandatory for the support of “voice-over-GEO for NB-IoT NTN”.

Observation 4	Thus, the schemes should also account for the case where the “TA report UE capability” is not available, in which case the network-controlled scheduling offset is based on a “Cell_specific_Koffset” approach. In this case, the gap between DL and UL should consider a margin for the network to handle the different TAs of different UEs in the NTN cell (i.e., “Max differential delay,” see right-side of Table 1).

Observation 5	The assumption on whether the scheme(s) will be based on a “UE_specific_Koffset” or “Cell_specific_Koffset”, will in some cases result e.g., in different supportable SPS periodicities and different achievable data rates.

Based on the discussion in the previous sections we propose the following:

Proposal 1	SA4 to consider for the ongoing and future work on the “simulation assumptions for ULBC,” not only dynamic and SPS schemes that depend on a TA report UE capability “UE_specific_Koffset approach,” but also schemes that are not conditioned/dependent on it i.e., “Cell_specific_Koffset approach”.

# 4	References

S4-251585: Pdoc FS_ULBC, v.0.2.0