SA4-Audio SWG AH	S4aA250236

Erlangen, 23 – 25 September 2025

Source:	Fraunhofer IIS

Title:	Pseudo-CR on Design Constraints of ULBC: Noise suppression

Spec:	3GPP TR 26.940

Agenda item:	4.4

Document for:	Decision

1. Introduction

TR 26.940 V.2.0 (S4-251152) currently only lists a template for the design constraints. This document proposes design constraints which can be considered as independent of other tasks, e.g. channel simulation. Especially, the “Potential use of noise suppression as part of the codec” has been identifies to be independent. A first proposal has been contributed in S4-251395. This document aims to address the question and comments raised at the last meeting.

2. Reason for Change

Providing design constraints for ULBC regarding “Potential use of noise suppression as part of the codec”.

3. Motivation for enabling noise suppression:

TR 26.940 contains a list of ultra-low bitrate codecs, conventional ones as well as AI-based coding systems, see Table 7.1.1-1. Conventional and AI-based codecs, both may include a noise suppression functionality. Especially for the ultra-low bitrate operation, this seems to be an essential tool allowing good audio quality and intelligibility.

For clarification, the noise suppression functionality operates at the encoder side to enhance the input signal before the encoding process. Such functionalities also been added for instance in the EVRC codec or G.718, even though the latter one only uses the clean signal for parameter estimation.

In general, the noise suppression functionality in ULBC is considered as an additional safety net in addition to the existing acoustic front-end processing available in the UE. In case, the UE noise suppression capability is not sufficient, ULBC can further enhance the input signal and ensure a certain input characteristic. This can also ease deployment as legacy acoustic front-end algorithms can still be used together with ULBC.

Another reason might be the tuning of the legacy front-end processing. For instance, due to the music capability of EVS, the front-end may bypass any background music and forward the signal to the codec. While this is expected for the EVS codec, it may not be envisioned for the ULBC codec, esp. when operating at the lower rates.

It is expected that any DTX operation is not affected as the codec, which is responsible for the DTX operation, has access to the noisy and the clean signal. This means the decision can be based on the noisy signal like for any other codec.

Regarding the status of noise suppression, it is the understanding of the source that if there is a significant quality benefit due to noise suppression, it should be enabled by default, at least for certain bitrates. Of course, it is up to the proponents to decide the default value by themselves. All tests should be carried out with the default option.

Even though the noise suppression algorithm is enabled by default, it’s performance can be adaptive to the signal characteristic to avoid any additional complexity or artefacts on clean speech.

In clause 9 (Test methodologies) of TR 26.940, it is also stated that “speech codecs operating at ultra-low bit rates may imply speech enhancement algorithms, such as noise suppression, gain normalization etc”. The topic on testing procedures is further discussed in S4aA250238.

Therefore, it is proposed that the design constraints should state that noise suppression should be allowed at least up to a certain bitrate. As TR 26.940 already mentions other speech enhancements, an editor’s note is proposed to capture this.

It is proposed to agree the following changes to 3GPP TR 26.940.

* * * First Change * * * *

Table 6.2-1 List of ULBC design constraint parameter

| Parameter | Design Constraint | Note |
| --- | --- | --- |
| Bit rates |  |  |
| Sample rate and audio bandwidth |  |  |
| Frame length |  |  |
| Complexity and memory demands |  |  |
| Algorithmic delay |  | The algorithmic delay is defined as the frame size buffering delay plus any other delays inherent in the codec algorithm (e.g., look-ahead, sample-rate conversion, and decoder post-processing) |
| Packet loss concealment (PLC) |  |  |
| Potential use of noise suppression as part of the codec | Noise suppression functionality at encoder side is allowed withing ULBC Editor’s note: Further speech enhancements are TBD |  |
| Discontinuous transmission including voice activity detection and comfort noise |  |  |
| Robustness to non-speech input |  | Editor’s note: May need to be in performance requirement |

* * * End of Changes * * * *