SA4 (AH) Audio SWG post 133-e					                  S4aA250249

F2F Erlangen, 23 – 25 Sept 2025                                                     revision of S4aA250241

Source:	Dolby Laboratories Inc., Nokia, Xiaomi Technology, Orange, Novamint

Title:	[FS_ULBC] On ULBC complexity design constraints

Agenda item:	4

Document for:	DISCUSSION and AGREEMENT

# Background

As per the time plan of FS_ULBC [1], one of the objectives for SA4 #133-e meeting is to make progress with identifying the relevant design constraints for such a codec, in coordination with other WGs, including

  - Bit rates

  - Sample rate and audio bandwidth

  - Frame length

  - Complexity and memory demands

  - Algorithmic delay

  - Packet loss concealment (PLC)

  - Potential integration of noise suppression as part of the codec

  - Discontinuous transmission including voice activity detection and comfort noise generation

As identified in [2], an AI based solution may have the potential to provide substantially better speech quality compared to conventional solutions at the envisioned bitrates for ULBC. Conventionally, 3GPP speech codecs like AMR-WB [4], EVS [5] and IVAS [6] have used Weighted Million Operations Per Second (WMOPS) [3] as a metric for computational complexity estimation. While this metric has proved to be useful for the existing 3GPP speech codecs, the source is of the opinion that alternative computational complexity metrics which may be more suitable for AI based solutions should be studied as part of FS_ULBC because it appears natural that they would be implemented on widely deployed novel processor types that are especially designed for AI solutions. Furthermore, considering the advances in hardware capability in the last decade to perform critical AI basic operations like matrix multiplication and multiply and accumulate (MAC), the source is of the opinion that comparing the number of operations in an AI based ULBC codec with an existing non-AI based reference codec (e.g., an existing 3GPP speech codec) may not be an optimal way to define ULBC complexity design constraints. Instead, the likely power consumption limits for ULBC use cases on relevant devices should be studied and the ULBC complexity design constraints should be defined based on that. In this document, the source provides information on the target devices that are envisioned for ULBC and their respective processing capability and proposes complexity estimation metrics and potential complexity limits for AI-based codecs.

# Discussion

## Complexity metric for AI based codecs

Matrix multiplication and MAC operations are the key elements of AI based solutions. The NPU (Neural Processing Unit) [17] or TPU (Tensor Processor Unit) [19], which are specialized processors to accelerate AI based solutions, generally defines its computational capability in Tera Operations Per Sec (TOPS) [13], [14], [15], [16], wherein TOPS = 2 × MAC unit count × Frequency / 1 trillion. Here, MAC unit count is the theoretical number of MAC operations that can be executed in parallel in one processor cycle (this number has been reported as > 10000 in many of the processors listed in table A.1), a factor of 2 is present because each MAC has two operations (Multiply and addition) and Frequency is the processor clock speed. Note that these TOPS values are typically defined with certain precision (for example INT4, INT8, INT16, FP16 or FP32).

When it comes to power efficiency metric, TOPS per Watt (TOPS/W) is a key metric for AI processors that measures how many TOPS a chip can perform for each watt of power consumed, for e.g., the TOPS/W on smartphones with NPU ranges from 2 to 15 TOPS/W. So, the number of MAC operations in an AI based solution can give a theoretical estimate of the power consumption. Another metric that has been used to benchmark computational complexity of AI based solution is real-time factor or RTF, corresponding to the ratio between the frame length of the input audio and the time needed for encoding/decoding. For instance, RTF analysis for some AI based codecs has been done in [7]. Existing standard AI codecs like JPEG AI [8] have used both MAC (kMAC per pixel) and real-time factor as metrics for computational complexity assessment. While the real-time factor on various processors/platforms is a very reliable metric for complexity assessment, it may be time and resource consuming to compute that data and comparable numbers require use of same hardware. Therefore, metrics like MACs per second (MACS) or Million MACS may be more practical methods for computational complexity assessment of ULBC. There are existing tools like ptflops [11] that work with pytorch and aten backend. These tools recognize commonly used layers in an AI based solution and give number of MAC operations and model size for complexity estimation.

Traditionally, 3GPP speech codecs have relied on WMOPS metric [3] to estimate computational complexity. This method assigns weights to various operations (e.g., weight of 1 to arithmetic operations like add(), sub() and L_mac(), weight of 3 to IF conditions and weight of 18-to-16-bit division div_s()). The overall sum of weighted operations can be counted over the frame execution and the worst/average/min statistics can be generated for various functions and for the overall codec. The instrumentation and counting of WMOPS can be automated with floating point code using the WMC tool as mentioned in [3]. For complexity estimate with ULBC C code, WMOPS counting tools may also be modified to give MACS estimate while reporting all other operations (memory load/store, if/else etc) separately. This may be helpful with estimating the part of the code that can be executed with parallel MAC units. Furthermore, it can be beneficial to utilize or derive metrics that allow complexity comparison of existing 3GPP codecs and ULBC, e.g., to determine relative effects of codec use on battery life.

## Complexity target estimation for AI based codecs

    - Target devices for ULBC

As mentioned in ULBC SID [9], “the codec is expected to be deployable on the processing capabilities as can be found in today’s smartphones” and the complexity and memory demands should be studied accordingly. NPUs or Neural engines have become extremely popular in the last decade as they are specifically designed to accelerate artificial intelligence and machine learning workloads and are arguably 5-20x more power efficient than CPUs for AI tasks and hence, they are an important component in modern processors. Almost every smartphone today that is expected to carry out AI tasks has an NPU chip inside it. To give an idea of the presence of NPU in today’s smartphones and smartwatches, [28] provides a list of examples of processors and devices with NPU component.

It should be noted that while ULBC is expected to be deployable on recent smartphones (most of which have NPU components), the codec may still need to run on non-NPU platforms in certain configurations (for e.g., devices which may not have an NPU but have deployed ULBC for NB-NTN or non-NTN use cases). The design objectives for non-NPU accelerated configurations is for further study.

    - Computational complexity and power consumption limits for ULBC

The manufacturers of the various processors (as mentioned in [28]) have reported the theoretical max TOPS that can be achieved with the respective NPU, and it ranges from approximately 8 to 59 TOPS [27] [18] at varying numerical precision (typically INT8 precision at higher TOPS and INT16 or FP16 at lower TOPs). Furthermore, TOPS/W (as mentioned in clause 2.1) is a popular metric that has been used to benchmark power efficiency when running AI tasks on these processors and ranges from 2 to 15 TOPS/W [15] [25] [26]  (on smartwatches, the TOPS/W ranges from 1 to 3). The source inferenced an existing AI based codec (DAC [10]) with a complexity estimate tool ptflops [11] that estimates number of MAC operations. The average number of MAC operations were estimated to be around 150 Giga MAC per Second (OR around 0.3 TOPS). The actual power consumption estimates when running this on smartphone NPUs is TBD.

It should however be noted that the TOPS/W on the same device may vary depending on the AI algorithm architecture and model size. Higher model size would mean more memory operations which will impact the actual TOPS/W and power efficiency [21]. Some of the existing research provides TOPS/W values along with the model size information, for example [15] shows around 13 TOPS/W when inferencing 8-bit quantized model [20] with inferencing cost of 5 Giga MACs and model size of 25 million parameters. Lyra V2 [22], based on soundstream [23] [24], was reported to be approximately 35 times faster than real time when running on pixel 6 pro (although it is unclear which parts of the Lyra V2 codec (if any) were running on the TPU during this experiment ).

# Observations and conclusions

      - A large number of commercially available smartphone models have a dedicated chip (NPU or TPU or neural engine) for AI tasks and they are much more power efficient in executing AI tasks as compared to CPUs.

      - The capability of NPUs or neural engines is often reported in TOPS which are determined based on parallel MAC units and clock frequency. The max TOPS capability on these devices ranges from approximately 8 to 59 TOPS [27] [18].

      - The power efficiency performance of NPUs is often reported in TOPS/W and ranges from approximately 2 to 15 TOPS/W [15] [25] [26].

      - Given the advancement in hardware (NPUs as compared to traditional DSPs and CPUs), the ULBC complexity design constraints should not be defined in reference to existing 3GPP speech codecs (for example, complexity target of ULBC should not be defined as X times EVS OPS or Y times AMRWB OPS). Instead, the ULBC complexity design constraints should be defined based on the desired power consumption or computational performance limits for running an AI based solution on the NPUs or neural engines that are present on today’s typical NPU enabled smartphones. A theoretical power consumption estimate may be obtained based on number of MMACS (Million MACS), model size of the codec, precision of parameters and the TOPS/W estimate on relevant devices.

      - Number of Million MACS executed in an AI based solution along with model size (number of parameters and precision) can provide an indication of the overall complexity. This metric has been adopted in existing standards like JPEG AI [8].

      - Real-time factor or RTF, corresponding to the ratio between the frame length of the input audio and the time needed for encoding/decoding, has been used to benchmark computational complexity of AI based solutions [7] [8].  Using RTF as a metric to define ULBC design constraints will require identification of accessible test benches/platforms for performing the evaluations which may need further discussion. Furthermore, it should be ensured that RTF evaluations are done in a way such that they are not biased towards certain platform specific optimizations.

      - The conventionally used WMOPS metric may not be directly suitable for computational complexity estimation of an AI-based solution on NPU capable devices. However, it can be beneficial to specify complexity metrics that allow for practical complexity comparison between existing 3GPP speech codecs and ULBC, e.g., based on a mapping between WMOPS and TOPS or RTF based metrics.

# Proposal

  - Proposal 1

The source proposes to document clause 3 in the clause 6.4 of 3GPP TR 26.940 [12].

  - Proposal 2

The source proposes to document clause 2 and 3 in the relevant PDOC.

References

[1] Tdoc S4-250751, “Time plan for FS_ULBC study item”

[2] Tdoc S4-250923, “Analysis of existing technologies for ULBC”

[3] ITU-T Software Tool Library 2024 User’s Manual

[4] Adaptive Multi-Rate - Wideband (AMR-WB) speech codec: 3GPP TS 26.171

[5] Codec for Enhanced Voice services (EVS): 3GPP TS 26.445

[6] Codec for Immersive Voice and Audio Services (IVAS): 3GPP TS 26.253

[7] T. Muller et al., “Speech quality evaluation of neural audio codecs”, Interspeech 2024, https://www.isca-archive.org/interspeech_2024/muller24c_interspeech.pdfers

[8] JPEG AI: https://ds.jpeg.org/documents/jpegai/wg1n101029-105-COM-JPEG_AI_Overview_Slides.zip

[9] SP-250378 “New SID on Ultra Low Bitrate Speech Codec”

[10] DAC: Paper, Code, Demo

[11] ptflops: ptflops·PyPI

[12] 3GPP TR 26.940

[13] A Guide to AI TOPS and NPU Performance Metrics - Edge AI and Vision Alliance

[14] A guide to AI TOPS and NPU performance metrics | Qualcomm

[15] J. -W. Jang et al., "Sparsity-Aware and Re-configurable NPU Architecture for Samsung Flagship Mobile SoC," 2021 ACM/IEEE 48th Annual International Symposium on Computer Architecture (ISCA), Valencia, Spain, 2021, pp. 15-28, doi: 10.1109/ISCA52012.2021.00011. keywords: {Ports (computers);Deep learning;Privacy;Memory management;Silicon;Mobile handsets;Energy efficiency;neural processing unit;neural network;accelerator;sparsity;mixed-precision;re-configurable},

[16] Van Belle, Jean Louis. (2024). Why neural processing units (NPUs) are the next Big Thing in AI. 10.13140/RG.2.2.35619.87845.

[17] Kyuho J. Lee, Chapter Seven - Architecture of neural processing unit for deep neural networks, Editor(s): Shiho Kim, Ganesh Chandra Deka, Advances in Computers, Elsevier, Volume 122, 2021, Pages 217-245, ISSN 0065-2458, ISBN 9780128231234, https://doi.org/10.1016/bs.adcom.2020.11.001.

[18] Huawei Da Vinci PowerPoint Presentation

[19] Tensor Processing Units (TPUs) | Google Cloud

[20] C Szegedy, V. Vanhoucke, S. Ioffe, J. Shlens, and Z. Wojna, “Rethinking the inception architecture for computer vision,” in 2016 IEEE Conference on Computer Vision and Pattern Recognition (CVPR), 2016, pp. 2818–2826.

[21] V. Sze, Y. -H. Chen, T. -J. Yang and J. S. Emer, "How to Evaluate Deep Neural Network Processors: TOPS/W (Alone) Considered Harmful," in IEEE Solid-State Circuits Magazine, vol. 12, no. 3, pp. 28-41, Summer 2020, doi: 10.1109/MSSC.2020.3002140.

keywords: {Neural networks;Deep learning;Program processors;Scalability;Integrated circiut modeling;Performance evaluation},

[22] Lyra V2 open-source audio codec gets faster, higher quality and compatible with more platforms - CNX Software

[23] SoundStream: An End-to-End Neural Audio Codec

[24] https://doi.org/10.48550/arXiv.2107.03312

[25] C. -H. Lin et al., "7.1 A 3.4-to-13.3TOPS/W 3.6TOPS Dual-Core Deep-Learning Accelerator for Versatile AI Applications in 7nm 5G Smartphone SoC," 2020 IEEE International Solid-State Circuits Conference - (ISSCC), San Francisco, CA, USA, 2020, pp. 134-136, doi: 10.1109/ISSCC19947.2020.9063111. keywords: {Random access memory;Engines;Convolution;Optimization;Machine learning;Quantization (signal);Memory management},

[26] J. Song,, “An 11.5TOPS/W 1024-MAC Butterfly Structure Dual-Core Sparsity-Aware Neural Processing Unit in 8nm Flagship Mobile SoC,” ISSCC, pp. 130–131, Feb. 2019.

[27] Exynos 2500 | Mobile Processor | Samsung Semiconductor Global

[28] Tdoc S4-25xxxx, “On ULBC devices processing capabilities”