Source:	Orange

Title:	Subjective test results on neural audio codecs

## Document for:	Agreement

## Agenda Item:	4.5

- Introduction

The “Study on Ultra Low Bitrate Speech Codec” (FS_ULBC) has been approved at SA#107 [1], and the WID has been further revised in [2].

In the present Tdoc we address the following study objectives from the WID description [2]:

4.	Provide some evidence that the design criteria can be met, for example existing reference codecs.

However, the present Tdoc may also be considered for the following objective:

7. 	Identify relevant reference codecs for comparison and evaluation purposes.

Orange has conducted several subjective tests that may be relevant in the context of ULBC. Results have been published in conferences – see [3, 4, 5].

- Proposal

We propose to document these test results in TR 26.940, as suggested in the present Annex.

## References

- 3GPP SP-250378, SID on Ultra Low Bitrate Speech Codec, Source: China Mobile Com. Corporation, vivo, Fraunhofer IIS, Qualcomm Incorporated, Spreadtrum, Dolby Laboratories Inc., Xiaomi, Huawei

- 3GPP SP-250635, Revised Study on Ultra Low Bitrate Speech Codec, Source : SA4

- T. Muller et al., “Speech quality evaluation of neural audio codecs”, Proc. Interspeech 2024

- T. Muller et al., “Evaluation of Objective Quality Models on Neural Audio Codecs,” Proc. IWAENC, 2024

- T. Muller et al., “Comparative Quality Study of Neural Audio Codecs - focus on music and mixed content,” Proc. GRETSI, 2025 (in French)

Annex: pCR to TR 26.940

Change 1

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[22261]	3GPP TS 22.261: "Service requirements for the 5G system".

[22887]	3GPP TR 22.887: "Feasibility Study on satellite access - Phase 4".

[26071]	3GPP TS 26.071: "Mandatory speech CODEC speech processing functions; AMR speech Codec; General description".

[7-1] 	T. Muller et al., “Speech quality evaluation of neural audio codecs”, Proc. Interspeech 2024,

[7-x1] 	T. Muller et al., “Evaluation of Objective Quality Models on Neural Audio Codecs,” Proc. IWAENC, 2024

[7-x2]	T. Muller et al., “Comparative Quality Study of Neural Audio Codecs - focus on music and mixed content,” Proc. GRETSI, 2025 (in French)

[…]

Change 2

# 7	Existing technologies and feasibility evidence

Editor’s Note:

- Provide some evidence that the design criteria can be met, for example existing reference codecs.

## [7.x	Test results on clean speech and music/mixed content

### 7.x.1	Overview

The present clause collects test results comparing neural audio codecs and traditional codecs, summarizing results documented in [7-1], [7-x1], and [7-x2].

### 7.x.2	DCR test results on clean speech

A P.800 DCR experiment on clean speech is documented in [7-1]. The following codecs were included in the test:

- Conventional codecs:

- Opus at 12, 16, 24 kbps

- EVS-WB at 7.2 and 8 kbps

- EVS-SWB at 9.6, 13.2, 24.4 kbps

-	AI based codecs:

- LPCNet at1.6 kbps

- Lyra V2 at 3.2, 6, 9.2 kbps

- EnCodec at 1.5, 3, 6, 12, 24 kbps

- AudioCraft at 1.5, 3, 6 kbps

- AudioDec (libritts sym) at 6.4 kbps

- DAC at 1.7, 2.6, 5.2, 7.8 kbps

- AudioDec (vctk sym) at 12.8 kbps

At the time this DCR test was conducted, codecs like Mimi, DAC-ibm were not available

In addition to these codec conditions, the Direct condition was included together with calibration conditions, corresponding to P.50 MNRU with Q = 36, 23, and 10 dB. Resampling was used to match the input sampling frequency of codecs.

For this evaluation, only clean speech was used to compare codecs. The DCR test was conducted in French.  A total of 30 listeners were recruited for the subjective test and split in 5 panels of 6 listeners. The audio samples presented during the test consisted of 8sec double sentences from a phonetically balanced dataset, with 3 male and 3 female talkers. The input samples were pre-processed by a 20-20,000 Hz bandpass filter and normalized to -26 dB LKFS.

Figure 7.x.2-1 shows the Degradation Mean Opinion Scores (DMOS) and 95% confidence intervals.

Figure 7.x.2-1 DMOS scores and 95% confidence intervals

The following observations can be extracted from the results in Figure 7.x.2-1.

- DAC has the best DMOS score among the tested codecs operating around 1.5 kbps, and increasing bitrate improves quality to a score close to Direct at less than 8 kbps

- EnCodec does not achieve the quality of ”Direct” even at 24 kbps and is below EVS and Opus at this bitrate.

- Lyra V2 (at 6, 9.2 kbps) is on par with EVS-WB at similar bitrates (at 7.2 and 8 kbps).

### 7.x.3	ACR test results on clean speech

A P.800 ACR experiment on clean speech is documented in [7-x1]. The setup (test conditions, processed material, lab conditions) is the same as in clause 7.x.2, except that the test methodology is P.800 ACR. The motivation to replicate the experiment with an ACR test methodology to enable a more direct comparison with objective metrics predicting MOS scores such as POLQA or PESQ.

Figure 7.x.3-1 shows the Mean Opinion Scores (MOS) and 95% confidence intervals.

Figure 7.x.3-1 MOS scores and 95% confidence intervals

The same observations as in clause 7.x.2 can be extracted from the results in Figure 7.x.3-1.

### 7.x.4	DCR test results on music and mixed content

A P.800 DCR experiment on music and mixed content is documented in [7-x2]. The following codecs were included in the test:

- Conventional codecs:

- xHE-AAC at 8, 12, 16, 24 kbps

- Opus audio at 16 and 24 kbps

- Opus voip at 12, 16, 24 kbps

- EVS-SWB at 9.6, 13.2, 24.4 kbps

-	AI based codecs:

- EnCodec at 12 and 24 kbps

- DAC at 4.3, 6, 7.8 kbps

- HILCodec at 4.5, 6 and 9 kbps

- SNAC at 2.6 kbps

## - FlowDec (75m) at 4.5, 6 and 7.5 kbps

A total of 30 listeners were recruited for the subjective test and split in 5 panels of 6 listeners.

Note that the following additional neural codecs were also pretested, but not included in the test because their level of quality was found to be too low for music and mixed content: LPCNet. Lyra V2, AudioDec, FreqCodec, HifiCodec, Spectral Codecs, Vocos, DisCodec, Mimi and AudioCraft.

The test included 30 conditions : the Direct condition, MNRUs and 24 coded conditions. The test database consisted of 6 categories (instrumental and vocal classical music, instrumental and vocal modern music, captured mixed content, artificially generated mixed content in the form of clean speech with music background). The input samples were pre-processed by a 20-20,000 Hz bandpass filter and normalized to -26 dB LKFS.

## Figure 7.x.4-1 shows the Degradation Mean Opinion Scores (DMOS) and 95% confidence intervals.

Figure 7.x.4-1 DMOS scores and 95% confidence intervals

The following observations can be extracted from the results in Figure 7.x.4-1.

- Tested codecs achieving the best quality are EVS and xHE-AAC at a bitrate around 24 kbps

## - The advantage of neural audio codecs is clearly visible at low bit rates. Still, no tested neural codec achieves a quality level close to Direct, and FlowDec at 7.5 kbps achieves a score of 4,08 DMOS.

## ]