3GPP TSG SA WG4 Audio SWG AH	S4aA250217 ->. S4aA250262

Erlangen, Germany, 23-25 September 2025

Source:	Fraunhofer IIS

Title:	On noise suppression for ULBC

Spec:	3GPP TR 26.940

Agenda item:	4.5

Document for:	Discussion

1. Introduction

This document aims to provide clarification and further evidence on the impact that adding or considering a noise suppression integration in ULBC may have. The possibility of allowing a noise suppression within the ULBC coding algorithm has already been discussed in S4-251395. This proposal is supplemented in this document by new evidence showing that the overall system based on neural coders can benefit from such an integration.

It is therefore proposed to add the following clauses to TR 26.940 under “Existing technologies and feasibility evidence”.

* * * First Change * * * *

## 7.x	Impact of noise suppression on AI based codecs

### 7.x.1	Existing systems using noise suppression

#### 7.x.1.1	General

In the following and in the first part, a brief review of existing coding systems that already integrate a noise reduction or suppression into a classical coding scheme is given. In the second part, some considerations are made about the robustness of neural coding in noisy environments and the merit of noise suppression in this context.

#### 7.x.1.2 	Classical speech coding

Several studies have investigated the impact of noise suppression or reduction on low bit rate classical parametric speech coders, such as MELPe [1] and AMR[2]. It reveals that noise reduction as preprocessing generally improves the quality of the extracted coding parameters and can improve the final decoded speech, especially in noisy conditions and at low SNRs. This preprocessing helps to correct errors in parameter estimation (like fundamental frequency and LPC coefficients) and reduce distortion, leading to higher intelligibility and perceptual quality.

Noise reduction modules were also designed as part of the 3GPP2 standards EVRC [2] and VMR-WB [3], with the aim of improving performance in noisy environments. In these cases, the speech coder transmits the denoised version of the noisy speech directly.

#### 7.x.1.3	 Neural speech coding

Neural speech codecs are known to be sensitive to noisy environments [4]. Their robustness can be influenced by factors such as training data diversity, their very low bit rates, their capacity and complexity, and quantization strategies. However, data-driven approaches implicitly make it difficult to anticipate failure modes, especially in unseen acoustic scenarios. Using noise suppression or reduction as a preprocessing step or integrated into the neural speech coder can minimize these potential issues as will be seen in the following. For low bitrates and low complexity, this also allows the neural coder to focus on modeling only the useful signal.

### 7.x.2	 Impact of noise suppression on AI based coded speech

#### 7.x.2.1	General

To determine the value of noise suppression for low-bit-rate speech coding, two listening tests with an absolute opinion scale were conducted in various noisy scenarios. The objective of using an absolute opinion scale is to identify listener preferences and understand how adding or not adding noise suppression to the processing chain affects user experience.

As noisy environments may vary greatly, two different assumptions and associated tests are made:

- In a first test, the assumptions done in 3GPP EVS characterization (S4-141372.zip) are adopted. It uses relatively high mixing ratios targeting SNRs between +15 dB and +20 dB in WB mode. In this first test, the same noisy types as in 3GPP EVS testing are considered, i.e. car, street and office noises.

- In a second test, more adversarial environments are considered, using also more diverse types of background noises. The SNRs have been lowered to between -5 and +15 dB, and background noise has been augmented with construction, transportation, and restaurant noises.

#### 7.x.2.2 	Test design and mixing procedure

For avoiding too exhaustive test effort, different noises are tested in the same test. The tests were performed on English speech, following the ITU-T P.808 recommendation [6]. The noises were taken from Annex B of the ITUT-P.501 [7].

In the first test, car, office and street noises are mixed randomly to 8 pairs of sentences each, at a SNR of 15, 20 and 20dB, respectively. It leads to a total of 24 pairs of sentences.

In the second test, street, construction, metro, car, office and restaurant were mixed to 4 pairs of sentences each at -5, -2.5,  0, 2.5, 10, and 15 dB respectively, leading to a total of 24 pairs of sentences.

The mixing between the noises and the speech item was performed after loudness normalization of the speech and noise signals using the BS1770demo tool from ITU-T STL [8]. RMS long-term option was used for the background noise level measurement by disabling the gating.

It should be noted that ACR is not the methodology used in the past in 3GPP to evaluate noise suppression or noisy speech. P.835 is typically used for evaluating systems with noise suppression. As all conditions contain the same noise suppressor, P.835 does not provide any additional information but additional testing overhead. In this experiment, the DCR methodology, which is typically used for noisy speech tests, was not chosen as subjects may consider the noise suppression as degradation when comparing with the uncoded noisy speech as given reference.

#### 7.x.2.3	Conditions under test

Different classical and neural audio coders are under evaluation. They are tested with and without combining them with a noise suppression module used as pre-processor. An advanced and state-of-the-art noise suppression technique known as DeepFilterNet2 was used. It is based on a DNN-based discriminative model and is available as open-source software [9]. It has the advantage of operating at 48kHz and can be applied for the different audio bandwidth supported by the codecs under test.

For the classical speech coders, we selected MELPe, AMR, AMR-WB and EVS, covering a large range of bitrates and quality obtainable by legacy systems.

For neural audio codecs, models specifically trained on speech and targeting relatively low bitrates relevant to ULBC activities were favored. Thus, SNAC[10], MIMI[11], and DAC model with IBM weights (DAC_IBM) [12], which all target bitrates below 2 kbps and are trained on relatively clean speech data, are incorporated into the test. In addition, LyraV2 at 3.2 kbps has been added [13]. It was most likely trained on a more diverse type of data, including noisy speech, following the training strategy of the original Soundstream model from which it is derived. Furthermore, the performance of a neural codec like LyraV2 can be appreciated at a lower level of complexity in comparison to other neural codecs selected.

In addition, and only for the first test, the original DAC model and weights [14] at a sampling rate of 24 kHz are tested at bitrates of 1.5, 3, and 6 kbps. This will allow us to appreciate the influence of the training strategy in relation to DAC IBM.

#### 7.x.2.4	High SNR test results

The results of test 1 and for 20 listeners are shown in Figure 1. The suffix of the conditions corresponds to the operated bitrate in bps. The suffix “_nr” in the codec names corresponds to the conditions where the codecs process and code the denoised speech after processing the noisy speech items by the noise suppressor DeepFilterNet2. For better illustration and visualization, the difference between the MOS obtained with noise suppression and without noise suppression for each codec under test is plotted in Figure 2.

The main observations are:

- Listeners prefer the uncoded clean and denoised speech over the uncoded noisy version.

- With these high SNRs and using a state-of-the-art noise suppression method, the denoised speech is as good as clean speech. This shows that current noise suppression methods introduce no or very few artifacts under these high SNR conditions.

- Noise suppression as a preprocessor is beneficial for all codecs, except for MELPe in this test, for which, at these high SNRs and global low quality, the advantage of speech enhancement could simply be greatly minimized. It should also be noted that MELPe already features a noise reduction module, which further diminishes the value of a more aggressive noise suppression at least for high SNRs.

- For classical speech codecs, the benefit of noise suppression appears to increase with bit rate or, at least, with the quality of the coded speech. One possible interpretation is that at low quality, limited bandwidth or high quantization noise could mitigate the benefit of the noise suppressor.

- For neural codecs, the benefit of noise suppression is greater, with more than 0.5 MOS point of improvement for different codecs (e.g., SNAC, DAC_ibm, DAC at 3 kbps).

- The comparison between DAC_ibm and DAC shows that the same model (same architecture and same complexity) behaves very differently depending on the training data and the target bitrate(s) during training. It is also shown that the plain DAC with original weights at 24kHz is not competitive at 1.5 kbps and probably below.

- LyraV2 operates with significantly less complexity than other neural codecs (about 70 times less complex). At 3.2 kbps, it performs worse than other neural codecs under test except for DAC at 3kbps, which is on par.

Figure 1 – P.808 ACR test results for high SNRs

Figure 2 – Delta MOS between coded denoised speech and coded noisy speech for high SNRs

#### 7.x.2.5	Low SNR test results

In this test, DAC codec was removed from the conditions under test, since it was already shown at high SNRs to be suboptimal for the applications considered. The results for 21 listeners are shown in Figure 3. The difference between the MOS obtained with noise suppression and without noise suppression for each codec under test is plotted in Figure 4.

The main observations are:

- Listeners prefer largely uncoded denoised speech over uncoded noisy speech, and this by about 1 MOS of difference.

- All classical speech coders benefit from coding the denoised speech. Still, the improvement is less than 1 MOS.

- Neural speech coders benefit even more from the noise suppression stage, with an improvement which can be over 1 MOS.

- Neural speech codecs with vastly lower bit rates can compete with conventional codecs, even under adverse conditions, when combining with a state-of-the-art noise suppression method.

- Generative-AI based neural codecs as illustrated by the DAC IBM score in the test on the coding of the denoised speech can even improve the absolute quality of the input signal to code.

Figure 3 – P.808 ACR test results for low SNRs

Figure 4 – Delta MOS between coded denoised speech and coded noisy speech for low SNRs

### 7.x.3	Conclusions

This clause presents two different listening tests demonstrating that performance of speech coders in noisy conditions can be significantly enhanced by ensuring a dedicated high SNR, e.g. by combining noise suppression and speech coding. Current neural speech coders seem sensitive to noisy environments and may benefit more from noise suppression than traditional speech coders. Ensuring a dedicated high SNR enables improved performance at very low bitrates under both high and low signal-to-noise ratio (SNR) condition.

Generally, using noise suppression methods might have an impact on overall delay and complexity, which is for further study. However, complexity and delay aspects  do not affect the results of the subjective evaluation presented and the observations done thereafter.

NOTE: While speech is considered the primary information, removing all parts of the background audio scene may in some cases not be desirable, e.g. when there is no active speech, as the background may contain relevant information, e.g. in the case of emergency calls.

Editor’s note: It is TBD whether the design constraints and performance requirements should reflect the note above.

* * * End of Change * * * *

References

[1]  Martin, R., Malah, D., Cox, R.V. et al. A Noise Reduction Preprocessor for Mobile Voice Communication. EURASIP J. Adv. Signal Process. 2004, 147306 (2004). https://doi.org/10.1155/S1110865704312138

[2] D. Virette, P. Scalart and C. Lamblin, "Analysis of background noise reduction techniques for robust speech coding," 2002 11th European Signal Processing Conference, Toulouse, France, 2002, pp. 1-4

[3] 3GPP2 C.S00I4-0 "Enhanced Variable Rate Codec (EVRC) Service Option for Wideband Spread Spectrum Communication Systems", 3GPP2 Technical Specification, Dec. 1999.

[4] M. Jelinek and R. Salami, "Noise reduction method for wideband speech coding," 2004 12th European Signal Processing Conference, Vienna, Austria, 2004, pp. 1959-1962.

[5] Tseng, W.-C., Harwath, D. (2025) Probing the Robustness Properties of Neural Speech Codecs. Proc. Interspeech 2025, 5013-5017, doi: 10.21437/Interspeech.2025-355

[6] Recommendation ITU-T P.808 (06/2021) “Subjective evaluation of speech quality with a crowdsourcing approach”, International Telecommunication Union

[7] Recommendation ITU-T P.501 (04/2025) “Test signals for use in telephony and other speech-based applications”, International Telecommunication Union

[8] Recommendation ITU-T G.191 (05/2024) “Software tools for speech and audio coding standardization”, International Telecommunication Union

[9] https://github.com/yuguochencuc/DeepFilterNet2

[10] https://github.com/hubertsiuzdak/snac

[11] https://huggingface.co/kyutai/mimi

[12] https://huggingface.co/ibm-research/DAC.speech.v1.0

[13] https://github.com/google/lyra/releases/tag/v1.3.2

[14] https://github.com/descriptinc/descript-audio-codec